3GPP TSG-RAN WG4 Meeting #117	R4-2521580

Dallas, USA, 17th – 21st November, 2025

Agenda item:		6.13.2.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#116bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## Work-split for CRs drafting

The following work split was agreed during RAN4#116bis regarding the CRs drafting:

| Content for both 38.141-1 and 38.141-2 | Current/ New clauses | Responsible company |
| --- | --- | --- |
| References Defintions, symbols and abbreviations Operating bands and channel arrangement | 2, 3, 5 | CATT |
| General conducted and radiated test conditions and declarations | 4 | Ericsson |
| Conducted and Radiated Transmitter characteristics - General - Base station output power - Output power dynamics - Transmit ON/ OFF power | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Samsung |
| Conducted and Radiated Transmitter characteristics - Transmitted signal quality - Unwanted emissions - Transmitted intermodulation | Old clauses: 6.5 to 6.7 New potential clauses to align how it was done with Core specs: 9.5 to 9.7 | Nokia |
| Conducted and Radiated Receiver characteristics - General - Reference sensitivity level - Dynamic range - In-band selectivity and blocking | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Huawei |
| Conducted and Radiated Receiver characteristics - Out of band blocking - Receiver spurious emissions - Receiver intermodulation - In-channel selectivity | Old clauses: 6.5 to 6.8 New potential clauses to align how it was done with Core specs: 9.5 to 9.8 | ZTE |
| Big CR 38.141-1 and 38.141-2 |  | Moderator (Ericsson) |

The list above is missing the in-channel adjacent subband leakage, which needs to be introduced to the conformance specifications aligned with the core specification. Below one can find what has been currently agreed for the core specification for conducted in-channel adjacent subband leakage:

### 12.2.9	In-channel adjacent subband leakage for SBFD

#### 12.2.9.1	General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall apply during the transmitter ON period.

#### 12.2.9.2	Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

#### 12.2.9.3	Minimum requirement for SBFD BS type 1-H

The ICASL requirements are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the in-channel adjacent subband leakage requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent subband leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The in-channel adjacent subband leakage power at each TAB connector shall be less than or equal to the limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

In-channel adjacent subband leakage needs to be included in transmitter test specifications.

There is still ongoing discussion regarding the necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104. We must note immediately that there is no additional relaxation in the legacy ACLR absolute basic limit for band n104. There is a relaxation only in the relative ACLR for legacy. As RAN4 has agreed to only use the absolute limit for in-channel adjacent subband leakage, there should not be any additional relaxation for band n104.

RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# Conclusion

In this contribution, our views on the potentially new BS RF requirements for SBFD operation are presented. The following observations were listed:

- In-channel adjacent subband leakage needs to be included in transmitter test specifications.

- RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# References

- R4-2515063 “Way Forward for SBFD BS conformance”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.