3GPP TSG-RAN WG4 Meeting #117	R4-2521582

Dallas, USA, 17th – 21st November, 2025

Agenda item:		4.19.2

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

Way Forward was agreed in RAN4#116bis [1] regarding the maintenance of evolution of NR duplex operation. The core part for NR duplex evolution was finished in RAN4#116 with final agreements made. There were still some items that needs a return to and this contribution shares Nokia’s views on those aspects.

# Discussion

## In-channel adjacent subband leakage

The following was agreed in RAN4#116bis:

<Way forward>: FFS on whether to introduce 3dB relaxation for the in-channel adjacent subband leakage for band n104.

The agreement in RAN4#116 was to reuse legacy ACLR absolute basic limit as described above. However, there is a bullet stating that the necessity of additional 3dB relaxation for band n104 shall be studied in maintenance. We must note immediately that there is no additional relaxation in the legacy ACLR absolute basic limit for band n104. There is a relaxation only in the relative ACLR for legacy. As RAN4 has agreed to only use the absolute limit for in-channel adjacent subband leakage, there should not be any additional relaxation for band n104.

RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# Conclusion

In this contribution, our views on the SBFD-capable BS RF requirements are presented. The following proposals were listed:

- RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# References

- R4-2515170 “Way Forward for [116bis][304] NR_duplex_evo_Maintenance”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.