3GPP TSG-RAN WG4 Meeting #117	R4-2521583

Dallas, USA, 17th – 21st November, 2025

Agenda item:		7.2.3

Source:	Nokia

Title:	On enhancement of co-location requirements

Document for:		Approval

# Introduction

A WF was agreed in RAN#116bis [1] regarding NR base station RF requirement evolution for FR1 and testing phase 2. There are two topics introduced in this work item and this contribution further elaborates Nokia’s views on topic 1: Enhancement of co-location requirements.

# Discussion

## General

The following was agreed in RAN4#116bis:

Agreements: - Use work-plan in R4-2513998 (ZTE) as starting point. As guidance for coming meetings adopt the work-plan in section 3 of this document. - For SBFD BS to SBFD BS adjacent channel coexistence work technical background create new TR. - As starting point have one agenda per WI objective. If needed later ask for sub-agendas.

The agreed work plan mentions the following for the RAN4#117:

Enhancement of co-location requirements - Continue the discussion on co-location scenario assumptions and the impacts on the OTA co-location requirements. - Continue the coupling loss analysis for missing cases from Rel-19 (e.g. band combination). - Continue the discussion on OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics

The missing cases for coupling loss analysis are well documented in R4-2513942:

Table 2.1-1: Intra-band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical separation | SA-to-SA | M: 54 (R4-2504358) | M: 53 (R4-2504358) | M: 56 (R4-2504358) | FFS |
|  | A-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-A | FFS | FFS | FFS | FFS |
| Horizontal separation | SA-to-SA | M: 31 (R4-2504358) | M: 34 (R4-2504358) | M: 46 (R4-2504358) | FFS |
|  | A-to-SA | FFS | M: 35 (R4-2507633) S: 55±15 (R4-2506307) | M: 51 (R4-2507445) M: 69 (R4-2507511) | M: 55 (R4-2507633) S: 70±15 (R4-2506307) |
|  | A-to-A | FFS | FFS | FFS | S: 77±15 (R4-2506307) |

Table 2.1-2: Inter-band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz to 3.5 GHz | 3.5 GHz to 4.9 GHz | 4.9 GHz to 7 GHz | 3.5 GHz to 7 GHz |
| Vertical separation | SA-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-A | FFS | FFS | FFS | FFS |
| Horizontal separation | SA-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-SA | FFS | FFS | FFS | S: 57±10 (R4-2506307) |
|  | A-to-A | FFS | FFS | FFS | S: 54±10 (R4-2506307) |

One thing that is necessary to discuss is the coupling loss type. For A-to-A and A-to-SA, it is needed to define the size of the antenna array as it will have an impact to results. The size of the array should be included in the table above to have comparable results.

We conducted some initial measurements for inter-band coupling from 3.5 GHz to 7 GHz and the results are found in the table below:

| Co-location scenario | Coupling loss type | Coupling loss (dB) |
| --- | --- | --- |
|  |  | 3.5 GHz to 7 GHz |
| Horizontal separation | SA-to-SA | M: 65.1 |
|  | A-to-SA | M: 64.8 |
|  | A-to-A | M: 71 |

Nokia studied the wideband horn antenna intensively in the Rel-19 WI and the results are found in clause 6.4.4.4 in TR 37.941 [2]. We would like to continue to study alternative test antennas and are currently identifying the most promising ones.

There is also a mention if the existing OTA metrics could be used to define the requirements. This would be a very good solution, however, there is a problem when measuring low power levels. The issue is the dynamic range of the measurement equipment. There might not be a straight-forward solution for this.

Using existing OTA metrics to define the requirements is challenging due to measuring low power levels.

- The co-location requirements and related measurement method need to be considered requirement by requirement. It might be that there is a need to have different measurement method for different co-location requirement.

There might be a need to have different measurement method for different co-location requirement.

## Enhancement of co-location requirements

The following was agreed in RAN4#116bis:

Agreements: - At this point give equal priority to both option 1 and option 2. - As starting point consider side-by-side co-location scenario with 10 cm separation. Further investigate typical scenarios for bands in upper region of FR1. - At this point focus on licensed bands when coupling loss is evaluated.

There was a discussion in RAN4#116bis regarding the co-location scenario and the separation of the co-located BSs. It was questioned if the current side-by-side is representing what is typically deployed in the field. While there are many examples of vertical placement of BSs today, there are also many examples of side-by-side deployments. And as the coupling loss between the BSs has been shown to be higher for vertical deployment, it is reasonable to still consider the side-by-side deployment as the worse case of these two. The discussion was also extended to question the 10cm separation of the two co-located BSs and if it represents what the installation guidelines are stating. While the installation guidelines may say a higher separation should be maintained, we have seen that the reality is not reflecting to this and the BSs are deployed where there is space in the masts. Keeping the current separation of 10cm is seen to reflect well the actual deployments in the field and guarantee the co-location requirements.

Consider to keep the 10cm separation and side-by-side deployment to guarantee co-location requirements are met in the field.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- Using existing OTA metrics to define the requirements is challenging due to measuring low power levels.

- There might be a need to have different measurement method for different co-location requirement.

- Consider to keep the 10cm separation and side-by-side deployment to guarantee co-location requirements are met in the field.

# References

- R4-2515091 “Way-Forward for NR_BS_RF_Ph2”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.

- TR 37.941 “Technical Specification Group Radio Access Network; Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements, 3GPP