3GPP TSG RAN4 Meeting #117							R4-2521594

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	6GR Considerations on Spectrum aspects

Agenda item:	8.5

Document for:	Approval

Introduction

In the current SID for Study on 6G Radio [1] the following topics are listed under 6GR, with the points in bold being relevant for this discussion.

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. --- --- --- --- --- --- --- --- From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. --- --- --- --- --- --- --- --- - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations --- --- --- --- --- --- --- --- - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

Furthermore, at RAN4#116 a plan for the 6G study in RAN4 was presented by the RAN4 chair as captured in [2]. One of the topics was “Spectrum” with a scope listed as:

- Band/band combination definition and simplification

- Definition of frequency ranges

- Spectrum related regulatory survey

In this contribution we discuss our considerations on the item listed above.

Discussion

Definition of frequency ranges

During NR the concept of frequency ranges were introduced to the specifications to address/quantify the significantly different properties and thereby related requirements in the newly adopted so-called cm/mm-wave spectrum. As a result, RAN4 introduced frequency range 1 (FR1) for the “legacy” spectrum and frequency range 2 (FR2) for the new spectrum. As a result, NR separates the frequency ranges as shown from TS 38.101-1 in Table 2.1-1.

Table 2.1-1: Definition of frequency ranges in NR

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

It can be noted in Table 2.1-1 that FR2 was separated into two sub-ranges as FR2-1 covers spectrum used for licensed NR deployment while FR2-2 currently covers only unlicensed deployment of NR.

FR2 is separated into two sub-ranges FR2-1 and FR2-2, where FR2-2 currently is only for unlicensed deployment of NR.

Given 6GR is not intended to cover unlicensed deployments, at least in the first release, it is suggested to exclude the FR2-2 range as of now from the 6G discussions. This in alignment with the SID description stating that it is not in the scope of the study.

RAN4 shall excluded the FR2-2 range from the discussions in alignment with the SID.

Alignment of frequency ranges between TN and NTN

With the introduction of NTN to NR there was a need for expanding the usable spectrum ranges even further. However, to keep the impact to terrestrial NR at a minimum, it was decided to introduce frequency ranges specific for NTN as shown from TS 38.101-5 in Table 2.1-2.

Table 2.1-2: Definition of NTN frequency ranges in NR

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

From Table 2.1-2 it is noted that FR1-NTN and FR2-NTN is not aligned to FR1 and FR2 respectively. With the latest updates to TS 38.101-5 FR1-NTN and FR2-NTN are even overlapped in the central range. This is illustrated in Figure 2.1.1-1.

Figure 2.1.1-1 Current Frequency Ranges defined for NR

NR currently has multiple frequency range definitions dependent on not only frequency range but also deployment type. The frequency ranges defined for TN deployments are not aligned to those defined for NTN deployments.

Since the intent when defining the 6G SI is that NTN operation shall be a part of the generic requirements we believe it should be considered whether the frequency ranges for TN and NTN use still is to be separated.

At RAN4#116bis a WF [4] was agreed to first consider TN deployment when discussing the frequency ranges:

- WF - RAN4 will initially focus the discussion on frequency ranges based on TN deployment.

Even, we believe there is benefit in also considering the alignment between TN and NTN definitions we acknowledge the significantly different deployment scenarios. However, we would still try and align if possible.

Framework for frequency definition

At RAN4#116bis a great amount of the discussion time was on the framework for the frequency range definitions. It resulted in a WF [4]:

- WF - RAN4 acknowledge that a definition of frequency ranges depends on multiple criteria’s which may have different importance dependent on individual company views. As a starting point the following can be discussed, not precluding companies to also consider others: - Regulatory framework - UE RF front-end architecture (e.g. whether requirements can be verified conducted or not) - Commonality of UE RF/RRM requirements - Numerology dependency - Unification of TN and NTN operation - BS types and requirements - Companies are encouraged to share information on the criteria they are considering for their proposal on frequency range definition(s)

In addition to this we would like to emphasize the RAN guidance and what is being discussed in RAN1 related to frequency ranges. This is illustrated in Table 2.1.2-1.

Table 2.1.2-1: Frequency Ranges discussed in other WGs based on RAN guidance

| FR | Range | SCS (kHz) | Max CBW (MHz) |
| --- | --- | --- | --- |
| FR1 | Sub 6GHz (410 MHz – 6.425 GHz) | 15 (FDD) | 100 |
|  |  | 30 (TDD) | 200 |
|  | Around 7GHz (6.425 – 8.4 GHz) | 30 | 400 |
| New FR(s) |  |  |  |
|  | Around 15GHz† (8.4 - 24.25 GHz) | 30* | 400 |
|  |  | 60* |  |
|  |  | 120* |  |
| FR2-1 | 24.25 – 52.6 GHz | 120 | 800 |

* Select one SCS per Range or sub-Range

** This is an implementation issue

† This range can be divided into several sub-Ranges, each with its own SCS.

Other WGs are already using sub-ranges of the current frequency ranges for discussion.

From Table 2.1.2-1 it can be deduced that there are some sub-ranges used for the de discussion within other groups. From this it can in our view be concluded that at least the band to be defined around 7GHz can be included into FR1. We base this on the similarity to the operation in existing bands not far below in frequency which already is defined as FR1 bands.

RAN4 shall conclude that a band around 7 GHz can be included into FR1

When it comes to the general frequency range framework for 6G multiple options were captured in the feature lead summary [6] as well as some drawn during ad-hoc sessions. One of the proposals presented in the ad-hoc session at RAN4#116bis is shown in Figure 2.1.2-1.

Figure 2.1.2-1 Proposal for framework of Alternative 1

The intention with the proposal in Figure 2.1.2-1 is that FR1 is extended upwards towards frequency point Y where also FR2 is extended downwards towards. FR2 is keeping FR2-1 for the extended range as the overall FR2 range is then possible to extend upwards if needed during the 6G timeframe. FR1 is then intended to cover multiple sub-ranges denoted FR1-1, FR1-2 and if needed even more to separate the FR1 range based on additional criteria’s as numerology, regulations etc.

A similar alternative to this framework is shown in Figure 2.1.2-1 where the difference is that FR2-1 is kept unchanged and a new FR2-0 is added below this to contain the extension of FR2 downwards towards frequency point Y.

Figure 2.1.2-1 Proposal for framework of Alternative 2

RAN4 shall consider the proposal of extending FR1 upwards and FR2 downwards to cover the frequency range between FR1 and FR2 as the basis for a 6G frequency range framework. This does not preclude the option of defining sub-ranges under FR1 and FR2.

In our view it would make sense to reconsider the use of frequency ranges and their relation to specified requirements more actively. This having them paired to system parameter attributes as sub-carrier spacing and maximum/minimum channel bandwidth etc.

RAN4 shall consider pairing frequency sub-ranges to specified assumptions and requirements in 6GR.

Furthermore, RAN4 needs to consider the potential new 6GR TN spectrum around 7GHz, 12GHz and 15GHz and its inclusion into frequency ranges or basis for new ranges. From our perspective it would makes sense to consider multiple ranges spanning from around 400MHz until around 52GHz. These new ranges can be influenced by the current NR ranges but not necessarily follow the same naming convention.

RAN4 shall discuss a split into multiple frequency sub-ranges spanning from around 400MHz to around 52GHz in 6GR.

Some have proposed defining a new frequency range to cover the range between FR1 and FR2. We do not believe this is a viable framework. Some are referring to this new range as FR3, this definition does not exist. To avoid any confusion on what it referred to it is suggested to at least within in 3GPP only to refer to agreed defined frequency ranges. This is also captured by the RAN4#116bis WF [4] as:

- WF - RAN4 encourage companies to be precise when referring to proposed frequency ranges by only using defined NR ranges as FR1, FR2-1 or FR2-2 or write the definition with frequency point(s) when referring to a proposal. - Note this is not related to the discussion of a band around 7 and 15 GHz

Definition of bands and band combinations

Operating band numbering

NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G use same band number if frequency arrangement is same. LTE uses Arabic numeral, WCDMA uses Roman numeral and NR has prefix on “n” in front of Roman numeral.

NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G had used same band number if frequency arrangement is same. LTE uses Arabic numeral, WCDMA uses Roman numeral and NR has prefix on “n” in front of Roman numeral.

At RAN4#116bis it was agreed in [4] to continue using the prefix concept for band naming in 6G also:

- Agreement: - RAN4 shall re-use the prefix concept from NR for 6G - Note: this agreement has no implication or limitation on the RAN1 and potential RAN4 discussion of UL and DL decoupling.

The question on what should be the prefix was left open at last RAN4 meeting but we would again propose to use “s”.

- For 6GR operating band naming re-use prefix concept from NR but replace “n” with “s”.

In Rel-21 both new 5G and 6G (and perhaps even 4G) operating bands will be defined. In those cases, the number is chosen from same number pool 65 – 256 on first come first served basis. This pool is currently shared also with 5G NTN bands which select band numbers from top of the range. This is summarized in Table 2.2.1-1 which also includes a suggestion for how to handle new 6G bands.

Table 2.2.1-1: Operating band numbering scheme

As shown in Table 2.2.1-1 a similar approach as used in NR for denoting bands can be used for 6G bands.

- Keep using numbering space 1-512 for new bands according to the division shown in Table 2.2.1-1.

If the range from 1-512 is deemed not sufficient in 6GR then it can be considered to add a ranged starting from 513 for use for new 6GR bands as indicated also in Table 2.2.1-1.

Carrier aggregation notation

Band combinations, i.e Carrier Aggregation (CA) or Dual Connectivity (DC), have its own notation. Reading a CA or DC declaration tells exactly the number of carriers and their frequency location through an agreed syntax. For CA syntax is shown on high level in Figure 2.2.1-1.

Figure 2.2.2-1 Inter and intra-band CA syntax of 5G

While the dash in the notation is known always to separate inter-band combinations the underscore is used in many relations and easily leads to confusion. This is because the underscore has the following purposes in the notation for 5G:

- Separates the band combination Type (CA/DC/V2X/SUP/ULSUP_TDM/SL) from the band declaration

- Separates RAT in EN/DC and NE/DC

- Separates downlink from uplink carriers in CA downlink configuration, where SUL is part of the uplink (Not in the Uplink CA configurations)

- Separates band combination Types to allow a second Type declaration in both EN/DC downlink (_SUP) and EN/DC uplink (_ULSUP_TDM)

- V2X and SL are not obligated to use an underscore or at least they are not doing so in current specifications.

The underscore seems not to have a clearly unified defined purpose in current band combinations.

In the band and band combination syntax there is also lots of types. These are CA/ DC/ V2X/ SUP/ ULSUP_TDM/ SL /SLCA.

CA is most frequently used syntax, indicating a combination of carriers, not limited to bands.

DC is introduced as a RAT indication but shares the use of underscore before band declaration. A combination consisting of bands with no letter in front of the band number and those with the letter “n” separated by underscore indicates exactly the same. So, it can be asked why use “DC”, when the bands already show the mix of RATs

The band prefix already can be used to identify DC combinations.

V2X or SL concerns specific bands, but are rather features or capability, than a Type. Most commonly linked to band n47 but has one rare case of n79-n79. These special combination indications are a result of limiting the adaptation of these for a specific device type.

From our perspective every band combination could be created with CA and underscores as the only syntax. This even with a distinguishing for SUL, V2X and SL as this remains a part of the declaration of the bands (e.g. n14, n38, n46, n47, n79, n96, n102).

There is no need to declare the type, but use only CA, when more carriers are combined.

Therefore, it can be questioned whether RAN4 shall adopt similar in 6G timeframe. We believe this is not needed.

- RAN4 shall not to use RAT or device type indications for band combinations in 6GR.

If possible, to eliminate the special band combination declarations it would allow simplification of band combination notation in general.

Removing special band combination declarations would allow simplification of band combination notation.

The syntax for band combination notation must cover all aspects as simple as possible. This means on CA is used as the Type, and no DC, SUL or other types are supported, since it is absorbed in the new syntax. The CA syntax must underscore only to separate the parts of the declaration as Type_DL_UL. An example is shown in Figure 2.2.1-2.

Figure 2.2.1-2 CA syntax in 6GR.

The band combination (CA) covers the downlink bands of s1, s46 and s78, with the uplink bands of s1 and s46. This compact notation is easy to read, it considers flexible channel BW, so no BCS needed, and it has both uplink and downlink in the same string. For Basket Notation we allow use of DL and UL as types.

- In case of DC -> CA_n1-s3_n1-s3 (“n” and “s” easily identifies RAT)

- In case of SUL ->CA_s1_s1-s81 (SUL is in the band enumeration)

- In case of Sidelink -> s47 (Sidelink is in the band enumeration)

- In case of V2X -> CA_s1-s47_s1-s47 (V2X is in the band enumeration)

In 6G CA syntax consists of a single type (“CA”), underscore separators for Type, DL and UL, one or more downlink bands separated by dash and one or more uplink bands separated by dash.

- In 6GR the band combination syntax shall consists of a single type (“CA”), underscore separators for Type, DL and UL, one or more downlink bands separated by dash and one or more uplink bands separated by dash.

Carrier aggregation and switching

In 5G NR the various switching schemes, like UL TxSwitching was introduced and broke with the notation of radio configuration through CA notation. A dedicated UE capability group allowed multiple switchedUL or DualUL declarations related to the carriers of the downlink operation. This means that the CA notation no longer serves for identifying the radio configurations state, and not only carrier deactivation, but also switching may change the active radio configuration at any point during operation.

In NR UL and DL switching schemes means that the CA notation is no longer enough to identify a radio state configuration.

As an example, it is not obvious reading the notation CA_n12A-n29A that a hidden feature of LBCAsw is applicable for making that combination work. The switching is covered in UE capabilities and may even not be read from the UE radio specification, but only in the system parameter specification.

In 6GR there’s a motivation to improve the CA framework, especially considering flexibility in of the configurations. In relation to switching this is also discussed as part of the spectrum aggregation framework under a different agenda. Allowing this additional flexibility for CA is desirable but it also means there is even more reason to carefully consider how CA configuration is identified. This to ensure all attributes of the individual configurations are clearly identifiable.

- RAN4 shall consider how CA attributes as DL/UL switching can be clearly identifiable for 6GR band combinations

Band combination specific requirements simplification

Definition of UE relaxations per band combination

During NR time frame RAN4 have been investigating how to improve the specifications. As proposed in [3], RAN4 was encouraged to investigate if some of the information/requirements now spread across multiple tables listing band combinations can be merged into a single table listing the band combination.

In the current NR specification multiple tables are now listing band combination specific requirements. To understand the meaning of the requirements for a single band combination one will have to consult numerous long tables in the specification.

A general thing for the band combination specific requirements is that most of these are UE relaxations for either Tx or Rx requirements. UE relaxations per band combination, e.g. ΔTIB,c and ΔRIB,c and even MSD in many cases, have the same value dependent on the type of issue, e.g. harmonics, creating the need for the relaxation. Even so, these relaxation values are currently listed per band combination in multiple tables for each type of issue.

Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonic mixing issues.

The work currently ongoing to move listing of supported band combinations to a database instead of listing them in the DOCX based specification is not addressing the band combination specific requirements but only the configurations and their supported channel bandwidth etc.

The to move listing of supported band combinations to a database instead of listing them in the DOCX specification is not addressing the band combination specific requirements.

To simplify the specification and the reading of it a suggestion is to list relaxation per UE issue/type in a single table and then list all the types of issues for the specific band combinations in list of supported band combinations. By grouping a list of issues per band combination would provide an overview of the individual band combinations combined need for UE relaxation. This was not done during NR times as the specification impact was deemed too large but for 6G the change principle can be used from the beginning.

- RAN4 shall consider for 6GR to providing a single list of supported band combinations together with their impairments requiring relaxation to provide an overview instead of spreading the information over multiple tables in the specification.

In a practical example from 5G NR for TS 38.101-1 only targeting Clause 7 is presented in [3].

Spectrum related regulatory survey

Use of spectrum for 6GR is based on the availability provided by international designations for IMT and regional regulations for the specific use.

Spectrum for 6GR is based on the availability provided by international designations for IMT and regional regulations.

The World Radiocommunication Conference 2023 (WRC-23) made positive decisions about additional bands for 5G/6G, opening the door to the regulatory discussions ongoing globally.

- WRC-23 took the decision that the U6 GHz can be used for 5G/6G in EMEA, a few Asian countries, and Brazil and Mexico. Additional countries have indicated they would back the band in the coming years and are expected to formalize this during the next radio conference, WRC-27. Currently, Europe is studying technical and regulatory conditions of whether all 700 MHz of the band to be made available for 5G/6G, or only a portion, or if a potential shared use by mobile and Wi-Fi would require limitations which would reduce the appeal for CSPs to deploy 5G/6G using this band. Regulation for the band is expected for 2027, making upper 6 GHz band potentially the pioneer band for the launch of 6G. Under the technology neutral principle, it can also be used, if needed, to support advanced cases of 5G and further be repurposed to 6G.

- The WRC-23 also agreed – for the WRC-27 cycle (AI 1.7) – to study 7-8.4 GHz for 6G, with the potential of identifying up to 1275 MHz in this range in 2027. Additional bands – 4.4-4.8 GHz and 14.8-15.35 GHz – are also being studied. For the 7-8.4 GHz range, the USA is a key country as it has limited spectrum available in mid-bands following FCC’s 2020 decision to allow unlicensed operations, including Wi-Fi in the full 6 GHz range.

In general, the use and availability of spectrum can be summarized via Figure 2.4-1. From Figure 2.4-1 it can be noted that the potential new 6GR TN spectrum around is focused around 7GHz, 12GHz and 15GHz.

Figure 2.4-1 Potential use of new and existing spectrum bands with 6G technology

Potential new 6GR TN spectrum around is focused around 7GHz, 12GHz and 15GHz

In relation to the potential new 6GR bands in specific, at the ITU-R level, stakeholders are engaging in defining the studies to evaluate the potential of the bands 4.4-4.8, 7.1-8.4 GHz and 14.8-15.35 GHz (under AI 1.7) to ensure that building economically viable and high-performing 6G networks using this spectrum is possible. The findings of the studies will be presented to the ITU-R which will make the final decision of whether to assign the spectrum to 6G at the World Radiocommunication Conference 2027 (WRC-27).

ITU-R is engaging in defining the studies to evaluate the potential of the bands 4.4-4.8, 7.1-8.4 GHz and 14.8-15.35 GHz

Outside of the ITU-R process and given the shortage of spectrum in the mid-bands, the USA is considering additional spectrum in the 3.1-3.45 GHz and in 12 GHz range. The successful use of these bands depends on defining the right operational conditions, i.e. an optimal sharing mechanism in the 3.1-3.45 GHz band to assure coexistence between military radars and mobile with least restrictive conditions for mobile deployment; efficient technologies to allow using the exclusively licensed 12 GHz range for wide area coverage.

FCC is assessing the potential of the bands 3.1-3.45 GHz 12.7-13.25 GHz.

Figure 2.4-2 provides a snapshot of mid-band frequencies that are considered at regional level for 6G based on harmonized conditions as per the WRC-23 decisions, new bands under study for WRC-27, and additional bands considered by the USA for its internal market.

Figure 2.4-2 Mid-band spectrum considered for 6G: 
Harmonized bands as per WRC-23 and new bands under study for WRC-27

From Figure 2.4-2 it can be noted that the upper 6GHz band (1) is already decided for unlicensed use in some regions while it is under discussion in the remaining. For the around 7GHz band (2) this is potentially available for 6GR in all regions. The same in principle applies for the around 15GHz band (3). However, we note that for (3) the incumbents in this bands make coexistence with 6G systems challenging. For (4) this has limited geographical footprint and also faces coexistence challenges. The USA specific bands around 3 GHz and 12 GHz (5) currently no other geographical adaptation.

In conclusion from the above the currently most attractive band, in our view, for 6GR studies within RAN4 is the around 7GHz band.

The currently most attractive band, in our view, for 6GR studies within RAN4 is the around 7GHz band.

Even ITU-R will not make the final decision of whether to assign the around 7GHz spectrum to 6G before WRC-27 (October, 2027 - Shanghai, China) we still believe RAN4 should focus on this band initially for 6GR development.

RAN4 shall initially focus on a new 6GR band around 7GHz.

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations have been made:

Observation 1: FR2 is separated into two sub-ranges FR2-1 and FR2-2, where FR2-2 currently is only for unlicensed deployment of NR.

Proposal 1: RAN4 shall excluded the FR2-2 range from the discussions in alignment with the SID.

Observation 2: NR currently has multiple frequency range definitions dependent on not only frequency range but also deployment type. The frequency ranges defined for TN deployments are not aligned to those defined for NTN deployments.

Observation 3: Other WGs are already using sub-ranges of the current frequency ranges for discussion.

Proposal 2: RAN4 shall conclude that a band around 7 GHz can be included into FR1

Proposal 3: RAN4 shall consider the proposal of extending FR1 upwards and FR2 downwards to cover the frequency range between FR1 and FR2 as the basis for a 6G frequency range framework. This does not preclude the option of defining sub-ranges under FR1 and FR2.

Proposal 4: RAN4 shall consider pairing frequency sub-ranges to specified assumptions and requirements in 6GR.

Proposal 5: RAN4 shall discuss a split into multiple frequency sub-ranges spanning from around 400MHz to around 52GHz in 6GR.

Observation 4: NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G had used same band number if frequency arrangement is same. LTE uses Arabic numeral, WCDMA uses Roman numeral and NR has prefix on “n” in front of Roman numeral.

Proposal 1: For 6GR operating band naming re-use prefix concept from NR but replace “n” with “s”.

Proposal 2: Keep using numbering space 1-512 for new bands according to the division shown in Table 2.2.1-1.

Observation 5: The underscore seems not to have a clearly unified defined purpose in current band combinations.

Observation 6: The band prefix already can be used to identify DC combinations.

Observation 7: There is no need to declare the type, but use only CA, when more carriers are combined.

Proposal 3: RAN4 shall not to use RAT or device type indications for band combinations in 6GR.

Observation 8: Removing special band combination declarations would allow simplification of band combination notation.

Proposal 4: In 6GR the band combination syntax shall consists of a single type (“CA”), underscore separators for Type, DL and UL, one or more downlink bands separated by dash and one or more uplink bands separated by dash.

Observation 9: In NR UL and DL switching schemes means that the CA notation is no longer enough to identify a radio state configuration.

Proposal 5: RAN4 shall consider how CA attributes as DL/UL switching can be clearly identifiable for 6GR band combinations

Observation 10: In the current NR specification multiple tables are now listing band combination specific requirements. To understand the meaning of the requirements for a single band combination one will have to consult numerous long tables in the specification.

Observation 11: Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonic mixing issues.

Observation 12: The to move listing of supported band combinations to a database instead of listing them in the DOCX specification is not addressing the band combination specific requirements.

Proposal 6: RAN4 shall consider for 6GR to providing a single list of supported band combinations together with their impairments requiring relaxation to provide an overview instead of spreading the information over multiple tables in the specification.

Observation 13: Spectrum for 6GR is based on the availability provided by international designations for IMT and regional regulations.

Observation 14: Potential new 6GR TN spectrum around is focused around 7GHz, 12GHz and 15GHz

Observation 15: ITU-R is engaging in defining the studies to evaluate the potential of the bands 4.4-4.8, 7.1-8.4 GHz and 14.8-15.35 GHz

Observation 16: FCC is assessing the potential of the bands 3.1-3.45 GHz 12.7-13.25 GHz.

Observation 17: The currently most attractive band, in our view, for 6GR studies within RAN4 is the around 7GHz band.

Proposal 7: RAN4 shall initially focus on a new 6GR band around 7GHz.

- References

RP-252912, Revised SID: Study on 6G Radio

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

R4-2413338, On UE RF specifications table improvement, Nokia

R4-2514626, WF on 6G Spectrum, Feature lead (Nokia)

R4-2514511, [116bis][104] Feature Lead Summary for 6G Spectrum, Feature lead (Nokia)