3GPP TSG-RAN WG4 Meeting # 117						   R4-2521597

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia, Ericsson

Title:	Progress update and next steps for the Band Combination Database

Agenda item:	12

Document for:	Discussion

Introduction

At RAN#102, it was highlighted that there is a need for a new tool/methodology for handling the specification of CA combinations in RAN4. It is widely recognised that tens of thousands of band combinations cannot be specified efficiently using tables in DOCX files. And the number of combinations continues to grow exponentially – a situation expected to continue in the 6G timeframe.

RAN#102 commissioned an activity to identify a new methodology with a small working party with a small number of expert delegates from RAN4 in conjunction with ETSI. A solution was identified together with ETSI, and work on enabling has been ongoing since then.

At the joint RAN#108 and SA#108 meeting in June 2025, a joint Study Item [1] was agreed which can be seen as an extension of the work initiated by the work on the Band Combination Database for simplifying the RAN4 specifications. The scope of the new SI spans beyond the work related to the Band Combination Database; even the developed database can be integrated into any new framework for 3GPP specifications.

Status updates on the work have previously been presented with the latest in [2]. This document presents the current state of progress of this activity.

Progress Update

Since the last status update at RAN4#116bis in [2], the work continues in three main areas:

- Establish working procedures aligned with MCC for used with the Band Combination Database.

- Updates of the Web-App.

- Generation of JSON data files containing the DC Band Combination.

Scope and objective for the Band Combination Database

The scope of the work was first agreed at RAN4#113 in [3], stating that:

<Agreement>:

-	The band combination information will be contained in JSON file format

- A separate JSON file per downlink configuration shall be created to contain information sorted based on downlink configuration to maintain the current organization of the listing of band combinations.

<Agreement>:

-	The JSON files shall be stored in a GIT repository managed by ETSI

<Agreement>:

-	The JSON file shall contain at least:

-	UL configurations supported per DL configuration

-	Supported Channel Bandwidths per DL band

-	Supported Bandwidth Combination Set(s)

At RAN4#113 the agreement was to excluded DC combinations and focus only on NR CA combinations but at RAN4#116bis the scope was extended to also include these combinations with the following agreement [7]:

< Agreement >:

- RAN4 agrees the database shall include the information now provided by:

  - TS 38.101-1

    - Table 5.5A.1-1, Table 5.5A.2-1, Table 5.5A.2-2, Table 5.5A.3.1-1, Table 5.5A.3.2-1, Table 5.5A.3.3-1, Table 5.5A.3.4-1, Table 5.5A.3.5-1, Table 5.5B-1, Table 5.5B-2, Table 5.5B-3, Table 5.5B-4, Table 5.5C-1, Table 5.5C-2, Table 5.5C-3, Table 5.5C-4, Table 5.5C-5.

  - TS 38.101-2

    - Table 5.5A.1-1, Table 5.5A.2-1, Table 5.5A.2-2, Table 5.5A.3-1.

  - TS 38.101-3

    - Table 5.5A.1.1-1, Table 5.5A.1.2-1, Table 5.5A.1.3-1, Table 5.5A.1.4-1, Table 5.5B.2-1, Table 5.5B.2a-1, Table 5.5B.3-1, Table 5.5B.3-2, Table 5.5B.4.1-1, Table 5.5B.4.2-1, Table 5.5B.4.3-1, Table 5.5B.4.4-1, Table 5.5B.4.5-1, Table 5.5B.4a.1-1, Table 5.5B.4a.2-1, Table 5.5B.4a.3-1, Table 5.5B.4a.4-1, Table 5.5B.5.1-1, Table 5.5B.5.2-1, Table 5.5B.5.3-1, Table 5.5B.5.4-1, Table 5.5B.5a.1-1, Table 5.5B.5a.2-1, Table 5.5B.5a.3-1, Table 5.5B.5a.4-1, Table 5.5B.6.2-1, Table 5.5B.6.3-1, Table 5.5B.6.4-1, Table 5.5B.6.5-1, Table 5.5B.6a.2-1, Table 5.5B.6a.3-1, Table 5.5B.6a.4-1, Table 5.5B.6a.5-1, Table 5.5B.7-1, Table 5.5B.7-2, Table 5.5B.7-3, Table 5.5B.7-4

Given the scope has been significantly broaden by including also all DC combinations from TS 38.101-3 (marked with underline in the agreement above) the work is still currently ongoing on including these to the database. This means that the content of the database as of now only includes CA, and not DC, configurations.

Work is ongoing for adding DC combinations to the database which now only includes CA combinations.

Timeline for the Band Combination Database

At RAN4#114, a WF [4] was agreed, stating that:

<Agreement>:

-	RAN4 will plan for a transition from MS Word table representation of band combinations to the ETSI MCC provided database either by the end of Rel-19 or during Rel-20

From the above agreement and the fact that RAN4 are soon to start Rel-20 work, it is needed for RAN4 to assess whether to target a transition to fully use the database as part of the transition from Rel-19 to Rel-20, or instead, the transition should be targeted during the Rel-20 timeframe.

At RAN4#116, a WF [6] was agreed, stating that:

<Agreement>:

-	RAN4 intends to transition to fully use the database and remove the current DOCX table-based representation of supported band combinations in Rel-20

Given the current state of development of the database, it seems likely that a transition can take place during the Rel-20 timeframe. The question is then whether there needs to be a work item in RAN4 to handle the CRs needed to make the needed specification changes, or RAN4 shall consider having these changes included as objectives in the Rel-20 band combination basket work items.

With a transition to the Band Combination Database during Rel-20, there is a need for either an independent work item or objectives for the Rel-20 band combination basket work items to facilitate the CRs needed to make the specification changes.

One suggestion is to use the Rel-20 basket work-items which is the continuation of the Rel-19 WIs on NR CA (NR_CADC_SUL_R19) and EN/NE-DC (DC_R19_xBLTE_yBNR). These new Rel-20 WIDs can include an objective of introducing the use of the Band Combination Database to the specifications.

RAN4 shall consider endorsing adding an objective of introducing the use of the Band Combination Database to the specifications at least in the Rel-20 NR CA and EN/NE-DC basket WIDs

One issue which may occur by utilizing the Rel-20 basket WIs is that they, since they are for Rel-20, only can modify the Rel-20 specifications meaning that the first version of the Rel-20 specifications will have the content intended removed and replaced by the database. The proposed solution to this is to work together with MCC on the potentially introduce the change. It may be possible to modify the unpublished Rel-20.0.0, but this is to be discussed further.

RAN4 shall work together with ETSI MCC on how the adaptation of the Band Combination Database can best be handled and reflected in the affected specifications.

Working with the Band Combination Database

Working with the Band Combination Database is new to all, and practice and learning are expected by all who want to continue involvement in this work.

As discussed in previous RAN4 meetings, there are multiple steps in the process of introducing new band combinations, and all of these can, to some extent, be part of the Band Combination Database. However, as using the database is new to RAN4, there needs to be a balance between how much is implemented and when RAN4 can start to use it.

RAN4 adaptation of the Band Combination Database

RAN4 have agreed, as captured in the WF at RAN4#115 in [3], to adopt the so-called stage 1 implementation of the Band Combination Database as described in the following. Since the term “stage 1” was misleading for some, who understood it in relation to the specification stages, it is proposed to adopt the term “Phase 1” instead.

Changing reference to the RAN4 agreed Procedures from “Stage 1” to “Phase 1”

RAN4 agreed Phase 1 Procedures

Phase 1 – Procedures:

- Request for new band combinations, tracking and WID listing is continued with no change. This means Word is still used for WIDs and SRs, and Excel is used for listing band combinations and their status. There may be changes to the needed information included in the Excel sheets, but otherwise, no major changes.

- When a company wants to submit a new or modify an existing band combination, they prepare a discussion paper (TP) which outlines the intended change and provides the needed technical justification/analysis. This is done in Word with a Tdoc just like the current procedure. The new step is that a JSON file shall also be provided and included in the Tdoc Zip file, which includes either the new or modified JSON container for the CA configuration or single band channel bandwidth support.

- During a RAN4 meeting, the submitted Tdocs can be handled similarly as now, using the basket procedure with flagging and block approval. Any issues identified during flagging or the meeting, can be corrected with a revision, just like now, where the new thing is that a revision to the JSON file also has to be provided.

- After the RAN4 meeting, first the Co-Rapporteurs and then the Main-Rapporteur will collect all the agreed JSON files and, together with a potential BigCR, if needed, submit this for post-meeting approval. When the batch of JSON files is approved, these are either directly by the Main-Rapporteur or via MCC committed to the ETSI Forge (GIT repository).

- The newly committed JSON file batch (i.e. commit number/identifier) will then be added to the “CR pack” for the RAN meeting by MCC.

- When RAN approves the JSON file batch, just like previously with CR packs, these become the new version of supported band combinations attached to a new version of the specifications.

- The WebApp will always display the latest approved batch of JSON files.

The procedures described above can now be adopted by RAN4 to efficiently start utilising the Band Combination Database. It is understood that multiple optimisations, like inclusions of automated checks, etc., are already mentioned/proposed by some companies, and these will be considered implemented at a later stage.

RAN4 have agreed on the “Phase 1 process” as a starting point for further adoption of the Band Combination Database.

At RAN4#116 the steps 4-6 were discussed related to how they will function on practice. This has led to further work with ETSI MCC and the working group on how to formalise these steps. They naturally have to be aligned with 3GPP working procedures and conform with ETSI Forge procedures as they are responsible for version control, etc.

Any process for updating and/or adding to the Band Combination Database needs to be aligned and agreed with 3GPP working procedures and ETSI as the host of the Forge repository.

The steps 4-6 after further refinement can for a single WI be visualized as in Fig.  1.

Fig.  1 Handling of JSON files during and after a RAN4 meeting

In Fig.  1 the JSON files submitted included in the Zip files (which can be more than one) for the RAN4 meeting is shown on the left side. They are processed during the RAN4 meeting as described in Step 3 of the Phase 1 Procedure. This result in some being agreed (marked in green) and some being rejected (marked in red). Only the agreed JSON files, corresponding to the agreed new or modified band combinations, is collected by the co-rapporteurs (marked in blue) or directly by the main rapporteur (marked in magenta). Until this point the JSON files are handled as individual files with no direct attachment to the Band Combination Database. This is done as there are requirements that RAN4 shall be able to function “offline” or in other words disconnected from the ETSI Forge hosting the Band Combination Database. Another benefit is that it is only the main rapporteur who is required to work within the ETSI Forge environment.

Handling of JSON files can be performed offline during the RAN4 meetings.

It is only the main rapporteur who is required to work within the ETSI Forge environment.

The main rapporteur will after the RAN4 meeting create a so-called merging branch with all the agreed JSON files and attach a reference to this branch created in the Band Combination Database to the BigCRs cover sheet as required by the Technical Specification Group working methods (TS 21.900) shown in excerpt below:

### 4.6.3		Contents of Change Requests

Although the CR form shall indicate the details of change, each CR shall have attached the clauses of the specification that are affected by the CR, using the latest version of the major version...

[…]

If the CR proposes changes to stage 3 specification files which are normatively documented in the 3GPP Forge repository according to clause 5C, then the proposed modifications shall be documented in the 3GPP Forge repository. In this case the cover page of the CR form shall contain a reference to a Forge Merge-Request clearly listing the proposed modifications in the "Other comments:" field, and excerpts of all affected stage 3 specification files clearly showing the proposed changes shall be appended to the cover page of the CR form.

…

- 5C	Normative availability and distribution of stage 3 specification files

In this option, the normative code parts of the stage 3 specification shall be normatively stored in the 3GPP Forge repository.

The TS document specifying the stage 3 definition shall indicate that the 3GPP Forge repository is normative for the corresponding stage 3 specification files and:

-	The TS document shall contain a link to a 3GPP Forge repository tag that implicitly includes the versioning information about the TS, e.g. https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA100;

-	The TS document shall contain the directory path where the files are stored, e.g. "yang-models";

-	The TS document shall contain the name of the files specified by this TS;

-	The TS document shall not contain a copy of the stage 3 specification files.

Before making available any new version of a TS specifying stage 3 files, the responsible MCC officer shall download the specification files from 3GPP Forge and store them in the zip file containing the new version of the TS document. This zip file shall be published in the usual places.

The Technical Specification Group working methods (TS 21.900) specifies how references shall be made from BigCR coversheet to the ETSI Forge Merge-Request to the Band Combination Database.

The merging branch with all the agreed JSON created by the main rapporteur can also be visualized during the RAN4 meeting, potentially using a local copy of the Band Combination Database if needed, as marked in grey in Fig.  1.

The updated Band Combination Database can be visualized during the RAN4 meeting.

After the RAN4 meeting the BigCR created for the WI will enter the MCC CR pack for the next RAN meeting where it needs to be approved before it can become the next release of the specification. When approved by RAN the merge request will be pushed to the Band Combination Database by the MCC officer.

The push of a merge request to the Band Combination Database will be handled by a MCC officer.

After the push of the merge request to the Band Combination Database it will now reflect the latest agreed band combinations. This new version of the Band Combination Database will be tagged with versioning information corresponding to the DOCX based specifications release to ensure alignment between these.

Tags will be used in the Band Combination Database to reflect which specification release is presented.

The version control build into the ETSI Forge and thereby the Band Combination Database allows users by selecting a different/old Tag within the Band Combination Database to see supported band combinations from that specific release of the specification.

Using the Band Combination Databases version control allows used to see supported band combinations from a specific release of the specification.

RAN4 shall further examine the agreed “Phase 1 process” with the perspective of the practicalities now presented after discussion with ETSI MCC.

The intention was that the ETSI Web App only shall reflect the latest release of the specification. There is however a possibility of also implementing a feature which would allow it to show a selected release of the specification. This will be considered further in the future as captured in the WF [7].

RAN4 shall consider whether there is a need to reflect the other releases than then latest within the EISI Web App.

Inclusion of JSON files within the TS zip file

When studying the 3GPP Technical Specification Group working methods (TS 21.900) it was found in Clause 5C (shown on previous page) that when publishing a new release of the specification (TS) MCC is currently required to include all the latest versions of the JSON files within the TS zip file.

MCC is currently required by TS 21.900 to include all the latest versions of the JSON files within the TS zip file.

Given the large number of JSON data files and also their relation to the JSON schema files (the difference between these is further elaborated in Section 3.3) it can be questioned whether this working practice is practical for the RAN4 specifications.

It can be questioned whether including all JSON files within the TS is practical for the RAN4 specifications.

If a reference is to be made to the repository with the Band combination Database within the TS, instead of including all the files within the zip file, the 3GPP agreed Working Methods first needs to be included within TS 21.900. Since this TS is under change control by SA it would be necessary to liaison with them to facilitate the changes. This type of liaison may also need RAN coordination, why RAN4 needs to first agree if including the JSON files within the TS zip can be accepted as a solution.

As discussed at RAN4#116bis there is no intention to change the working methods of TS 21.900 as of now from RAN4 side. This as long as the current working methods can function together with the Band Combination Database.

RAN4 has no intention of changing the working methods of TS 21.900 as of now and will therefore include the JSON files within the TS zip.

Additional, since the JSON files are not easily readable in large numbers, it is suggested to also include an auto generated table from the content of the band combination database within the zip TS file. The auto generated table(s) showing the supported band combinations can be added as an individual file within the zip file and does not need to be part of the Word file. The details on how this can practically be done needs further discussion with ETSI.

Since the content of the Band Combination Database is to be included to the TS zip file it will be available for all who currently uses the TS, even offline.

Visualizing the supported band combinations and including that to the TS zip file needs further discussion.

RAN4 shall further discuss how the supported band combinations can be best visualized also within the downloadable TS zip file. The visualization shall be based on the content of the Band Combination Database.

Access to the Band Combination Database and its content

The ETSI Forge

All 3GPP members can gain access to the Band Combination Database within a repository in 3GPP’s Git(lab) instance (also known as ETSI Forge) via:

https://forge.3gpp.org/rep/ran4/carrieraggregation

using their EOL account.

ETSI have its own 3GPP Git(lab) instance known as the ETSI Forge hosting the Band Combination Database

Currently all work related to the Band Combination Database is stored in the ETSI Forge repository for the Band Combination Database. Thereby all have access to the folder/repository containing JSON data files, channel bandwidths per NR band, as well as the corresponding schema files.

Fig.  2 Current repositories in the ETSI Forge

All 3GPP members have access to the Band Combination Database, where JSON data and schema files for bands and band combinations are stored.

The ETSI Forge inherently incorporates version control of the data stored, as it uses the underlying GIT framework. It is possible to align the versions of the JSON data and schema files to the versions of the specification, meaning that one similar to now can extract and view a specific version of supported band combinations. However, currently, the ETSI Forge version of the JSON data and schema files is not necessarily in sync with the official DOCX-based specifications. This is the transformation from the DOCX band combination tables are currently not automated, but run when needed during the trial phase of the implementation of the Band Combination Database.

The JSON files currently within the Band Combination Database is not necessarily in sync with the official DOCX-based specifications.

The intention is that when RAN4 sets a transition point from the DOCX-based specification to the Band Combination Database, then the alignment will be done beforehand to facilitate verification. But until this, there is no need for frequent updates.

The alignment between official releases of the specification and the versions of the Band Combination Database will be implemented at a later stage.

To facilitate the alignment of the version of the Band Combination Database with the specification release it is needed to structure the ETSI Forge repository for the Band Combination Database differently than what has been done so far. Therefore, RAN4 endorsed a new structure at RAN4#116bis as captured in [7] and presented in Fig.  3.

New structure for the ETSI Forge repository for the Band Combination Database is presented in Fig.  3

From Fig.  3 it can be noted that the Band Combination Database structure is now under a folder for “RAN4”/”Specifications”. Within this is a sub-folder for “38.101” with a sub-folder for “-1”, “-2” and “-3” specifications as well as a “common” folder. With this it is possible to have versions of the repository for each specification tagged in alignment to the specification release.

With the proposed structure for the Band Combination Database, it is possible to have versions tagged in alignment to the specification release.

Fig.  3 Endorsed new repository structure

RAN4 shall encourage ETSI to restructure the Forge repository according to the endorsed structure.

ETSI Web-App

The content of the Band Combination Database can be visualised via a web app.  This can be accessed via:

https://tnm.etsi.org/ca/

Via the Web-App, it is possible to download all the content of the repository for the Band Combination Database using the EOL account login information. This can be accessed via the link below.

All the content of the latest version (main branch) of the Band Combination Database can be downloaded using the EOL account login information.

Note that, as mentioned, the version of the available data is currently not necessarily in sync with the official DOCX-based specifications.

Currently, ETSI is working on a tool hosted on the Web-App to facilitate validation of JSON data files against the JSON schema files and also provide the possibility of generating and editing JSON data files directly in the tool.

ETSI is working on providing a simple tool for generating, editing and validating JSON data files.

The tool is intended as a simple assistant to companies, as companies for more advanced use, use as running batch generation of JSON data files, are expected to have their own tools. This is no different from now, where different companies have their work processes for providing TPs and draftCRs.

Companies may create their own tools for more advanced generation and/or modifications of JSON data files.

Currently ETSI have implemented a visualizer for the JSON data files which can be accessed via:

https://tnm.etsi.org/ca/visualize

ETSI have provided a JSON data file visualizer accessible via the Web App.

Introduction of JSON data- and schema files

The JSON data files represent a specific downlink CA or DC configuration with all its attributes. Meaning that one can envision each row of the current DOCX band combination tables as a representation of what is included in a JSON data file.

A JSON data file represents the information we currently have in each row of the band combination tables in the current DOCX specifications.

This means that instead of adding new rows to the band combination tables in the DOCX-based specifications, as we do now, we will instead add a new JSON data file to the database.

Similar to an agreed table format in the current DOCX-based specifications, the JSON data also needs to follow a specified structure to make sure it can be represented correctly. Therefore, RAN4 have agreed on a JSON schema file which can be envisioned as an empty DOCX-based table with columns, rows and cells ready to represent the content of the JSON data files. Hence, the JSON schema file defines the structure of the corresponding JSON data files.

The JSON schema files define the structure (i.e table-structure) of the content of the JSON data files.

In conclusion, there are two different JSON file formats that it is important to distinguish:

- JSON Data Files

- JSON Schema Files

- These two are further explained in the following.

JSON Data Files

There are currently two different JSON data files stored in the ETSI forge. One is for the individual bands and the other for the CA band combinations. These are stored in different repositories, as shown in Fig.  2. The content of the channelBandwidths JSON data files representing all supported channel bandwidths of the individual bands is to be aligned to what is listed in TS 38.101-1/2 Table 5.3.5-1. The other repository, bandCombinations, contains all the JSON files representing the band combination configurations now included to the Band Combination Database.

Both the channelBandwidths and bandCombinations JSON data files follow the structure given by the JSON schema files stored in the jsonSchemas repository. The JSON schema files are further explained in the next section 3.2.2.

An example of a JSON Data file for CA_n5A-n8A is shown in Fig. 2 with Table 5.5A.3.1-1d from TS 38.101-1 for reference below.

Fig.  4 JSON data file for CA_n5A-n8A

In Fig.  4, the following variables can be found in the JSON data file for CA_n5A-n8A:

- bcId (string, required):

  - Unique identifier for the band combination.

- bcsList (array, required):

  - List of Band Combination Sets (BCS) for this band combination. Each BCS describes a possible configuration.

- bcsId (integer, required):

  - Unique identifier for this BCS within the band combination.

  - Note that bcsId 4 is used to indicate BCS 4&5, as an integer is used for this variable.

- bandList (array, required):

  - List of bands involved in this BCS. Each entry describes one band and its carrier configuration.

- bandNumber (string, optional):

  - Band number for a single-carrier configuration.

- nonContiguousCarriers (array, optional):

  - One or more non-contiguous carriers. Each item is an array of integers, where each integer is a carrier bandwidth in MHz.

- notes (object, optional):

  - Additional requirements, constraints, or informative notes applicable to selected bands. Contains boolean flags for specific technical notes (see section 3.2.3).

Note that some of the possible components/attributes of the JSON data files are not used in the example in Fig.  4. The full list is part of the JSON schema files described in [8].

Since the development of the JSON data file now present in the repository and described in this contribution it has been agreed to add additional information within the JSON data files to enable tracing changes and origin of the individual band combinations captured by the JSON data file.

It is considered to add an information field within the JSON data file to allow tracing changes and origin of the individual band combinations.

The content of the information field is as of now envisioned to be the Tdoc number of the latest agreed JSON datafile and also the WI code for it. It is suggested that RAN4 discuss what information is relevant for enabling tracing changes and origin of the individual band combinations keeping in mind that the file is intended to be lean and the general version control is handled by the ETSI Forge environment and not the files themselves.

RAN4 shall discuss what information is relevant for enabling tracing changes and origin of the individual band combinations in addition to Tdoc number of the latest agreed JSON datafile and WI code, if any.

An alternative solution is that all information needed to allow tracing changes and origin of the individual band combinations is generated within the ETSI forge environment. i.e. GIT.

It is possible to have the ETSI forge environment provide all the information needed to allow tracing changes and origin of the individual band combinations.

Additionally, since RAN4 has now agreed to also include EN/NE-DC combinations to the database it is intended to include the pre-fix, i.e. “CA_” or “DC_” in front of both filenames and bcId handle to separate the two types. As agreed with the scope of this work, listed in the tables listed in Section 2.1, there is no intention of adding additional band combinations besides NR CA and EN/NE-DC combinations. Note that some SUL combinations are included within the scope of the NR CA and EN/NE-DC combinations and a solution to handle these will also be needed.

It is intended the pre-fix, i.e. “CA_” or “DC_” in front of both filenames and bcId handle to separate the two types of band combinations now within the scope of this work.

JSON Schema Files

JSON schema files are templates which define the structure of corresponding JSON data files. JSON editors can use the schema files to assist by auto-completion when writing or editing new data files and to highlight syntax errors when a data file does not comply with the schema.

JSON editors can use the schema files to assist by auto-completion when writing or editing new data.

Currently, there are two different JSON schema files, as shown in Fig.  5.

Fig.  5 Current JSON schema files in the ETSI Forge

The two different schema files give the structure for the channel bandwidths supported per band in the Band.json file and the individual NR CA band combinations in the BandCombination.json file. In relation to this work, when referring to the JSON data file, it is generally the band combinations files meant unless it is wanted to present a single band and its supported channel bandwidths. The reason there are two separate files is that the structure and information per band are different from that of the band combinations.

There are currently two different JSON schema files in the ETSI Forge. One for the Channel Bandwidths and one for the NR CA combinations.

When DC combinations are added to the Band Combination Database, there may be a need to define a specific JSON schema file for this, as the current BandCombination.json file is tailored to the NR CA combinations.

There will be a new JSON schema file for DC combinations, as the ones used now are specific to NR CA combinations or channel bandwidths.

A modification of a schema file corresponds to a change in today’s DOCX table layout (adding/removing columns; using different content in existing cells; …). If there is a need to modify a schema, RAN4 should review those changes with great care to ensure that existing data files are still compatible with the updated schema.

When schema files are modified, there will typically be a need to modify the Web-App, too. This is to ensure that all necessary data is also displayed in the Web-App as needed.

Finally, a modification of a JSON schema will typically also require updating tools that are used to validate consistency of the tables or to extract information, e.g. for use in product development.

It should be noted that modifications of the traditional DOCX table formats had the same consequences, but those were much more difficult to detect and account for than with the new JSON format. Nevertheless, it has also been an aim for RAN4 not to modify the DOCX tables unless completely justified. The same should apply to the JSON schema.

Frequent changes to the JSON schema should be avoided by RAN4.

Since some additions is needed to include more information to the JSON data files, as discussed in previous section, the JSON schema files also need to be updated to facilitate the container for this information. As the JSON schema file will influence all the tools and general handling performed by ETSI it is needed to stabilize the JSON schema file as soon as possible and not later that before Dec. 2025 if the intention is to mandate JSON files for all band combinations in Rel-20.

RAN4 shall strive to finalize JSON schema files before Dec. 2025 to ensure these are ready for the Rel-20 band combination work.

(Foot-)Notes – Additional attributes of the Band Combinations

In the DOCX version of TS 38.101, the note numbering and sometimes the wording for bands and band combinations were not aligned across tables and files. The same footnote numbers had different meanings in different tables and vice versa. Furthermore, definitions of those footnotes tended to deviate slightly in wording among tables, even though they were meant to be the same.

In the Band Combination Database, each band and each band combination is defined in a separate JSON data file. To ensure consistency among those files and to simplify creating and utilising those files, notes are now defined in the JSON schema files and associated with a short but descriptive identifier.

The JSON schema file includes the possible additional attributes of band combinations, as we are now capturing via notes in the DOCX-based specifications. This includes, e.g. HPUE UL support and/or DL channel bandwidth restrictions.

The JSON schema files include the possible additional attributes of a band combination as e.g. additional UL power class support and/or DL channel bandwidth restrictions.

A clear advantage with this approach is that there will inherently need to be alignment/agreement on wording for these notes, defining additional attributes of band combinations. Further, as an extension to ensure unambiguous representation of the JSON data, the JSON schema file templates that the JSON editors (i.e proponents of a new or modified band combination) can use the schema files to assist by auto-completion when writing or editing new data files and to highlight syntax errors when a data file does not comply with the schema

By having the “master” JSON schema file as a starting point for all new band combinations, now including all the attributes (including notes), the task for those proposing a new combination will then become only to specify the required attributes. With this approach, each JSON file becomes independently readable and as lean as possible.

The use of the JSON schema files as a basis for adding notes to the JSON files is further described in [8].

Examples of creation/modification of JSON data files

To facilitate the adaptation of the Band Combination Database by RAN4, further an introduction to it and some guidance on how to use it have been added in this section of this contribution.

There are two different recommended approaches to either introduce a new band combination or modify/amend an existing band combination within the Band Combination Database.

Note that the below is aligned to the Phase 1 procedure.

- Edit an existing JSON data file:

  - Navigate to the JSON data file located in the ETSI Forge bandCombinations repository (e.g. /38.101-1/) of the band combination which is to be modified/amended.

  - Download the JSON data file and save it as a local copy

    - It is suggested to append the Tdoc number of the contribution in front of the file name to associate the modified JSON data file.

  - Open the JSON data file in your preferred code editor (e.g. Notepad++ or VS Code).

  - Perform the needed additions or modifications to the JSON data file and save it

  - Add the modified JSON data file to the Zip folder together with other content, i.e. draftCR or TP.

  - Submit as a contribution for RAN4

- Add a JSON data file:

  - Copy an existing JSON data file as a template, or create a new file with a .json extension.

  - Save the new JSON data files as a local copy using the naming convention of the included bands

    - It is suggested to append the Tdoc number of the contribution in front of the file name to associate the new JSON data file.

  - Open the JSON data file in your preferred code editor (e.g. Notepad++ or VS Code).

    - If using a code editor as VS Code, one can use the autocompletion features provided by the JSON schema file to fill in required fields according to the schema.

  - Perform the needed additions or modifications to the JSON data file and save it

  - Add the modified JSON data file to the Zip folder together with other content, i.e. draftCR or TP.

  - Submit as a contribution for RAN4

When submitting the JSON files together with the Tdocs then need to be included to the zib files as explained in the following [7]:

  - JSON files are saved including the TDoc number prefixed the file name:

  - JSON files are saved within the zip file in a sub-folder named “JSON”:

  - If multiple JSON files these are all in the same JSON folder (Note this is not zipped):

JSON files need to be uploaded within the Tdoc zip files as shown in the 3-steps above.

The best way to learn how to work with the JSON files and the database is to start using it. This is also encouraged by RAN4 as captured in previous WFs [5].

As captured in WF at RAN4#115, RAN4 is already now encouraged to start working with the JSON files and database to get a more tangible understanding of what it is and can become.

Next steps and Future aspects

Next steps

Already now, multiple companies have started to work with the JSON data files and are getting themselves accustomed to them. For RAN4 to gain more experience it is important that companies already now get involved in the use of the Band Combination Data Base as the agreement is that all will have to work with this latest in Rel-20.

All companies will have to work with the Band Combination Database latest in Rel-20.

Work is ongoing to align the working procedures of 3GPP to how RAN4 intends to use the Band Combination Database. Dependent on outcome of the RAN4 discussion there may either be a need to change 3GPP Working Methods or slightly adjust the current RAN4 understanding that the JSON Data files only shall be available via the ETSI Forge.

RAN4 needs to conclude whether the JSON files shall be available via the TS zip or only via the ETSI Forge.

The Rel-19 band combination work have been extended to the end of 2025 whereafter the Rel-20 work will start. The expectation is that the Rel-20 specification is not available before after June 2026 meaning that there will be 6 months from December 2025 until June 2026 where it would be possible to perfect the adaptation of the Band Combination Database without having published official RAN4 Rel-20 specification.

There is 6 months from December 2025 until June 2026 where there will no published official Rel-20 RAN4 specification.

As this time period is used for RAN4 to adapt the Band Combination Database it is at the same time needed to keep the Band Combination Database updated with newly agreed or modified band combinations handled by the Rel-20 basket work items. Therefore, it is needed that JSON files are included as part of this work from the beginning of Rel-20.

RAN4 shall mandate inclusion of JSON files for all new or modified band combinations starting from Rel-20

With this clear point in time alignment of the versions of the Band Combination Database to the release of the specification can be efficiently facilitated.

Future aspects

Currently, the focus of the work is on NR CA/DC combination tables listed in TS 38.101-1/2/3, as in the channel bandwidth and CA configuration tables. This means that configuration-specific requirements such as delta values and potential MSD, are not within the scope of the Band Combination Database. This sis also reflected in the agreement below [5]:

<Agreement>:

-	RAN4 will focus on inclusion of the CA configuration tables to the database as a first step. Other aspects as delta values and MSD requirements can be considered at a later stage.

However, there have been multiple suggestions to also consider moving the long tables listing band combination-specific requirements to the Band Combination Database. It is therefore proposed that RAN4 discuss how this can be facilitated. Nevertheless, since RAN4 has started 6G work it can also be considered whether this work can be part of the next generation specification.

RAN4 shall discuss whether the inclusion of band combination-specific requirements in the Band Combination Database shall be part of the 6G SI.

Conclusion

In this contribution, we present the status of the Band Combination Database and what steps are planned next. The following observations have been made:

Observation 1: Work is ongoing for adding DC combinations to the database which now only includes CA combinations.

Observation 2: With a transition to the Band Combination Database during Rel-20, there is a need for either an independent work item or objectives for the Rel-20 band combination basket work items to facilitate the CRs needed to make the specification changes.

Proposal 1: RAN4 shall consider endorsing adding an objective of introducing the use of the Band Combination Database to the specifications at least in the Rel-20 NR CA and EN/NE-DC basket WIDs

Proposal 2: RAN4 shall work together with ETSI MCC on how the adaptation of the Band Combination Database can best be handled and reflected in the affected specifications.

Proposal 3: Changing reference to the RAN4 agreed Procedures from “Stage 1” to “Phase 1”

Observation 3: RAN4 have agreed on the “Phase 1 process” as a starting point for further adoption of the Band Combination Database.

Observation 4: Any process for updating and/or adding to the Band Combination Database needs to be aligned and agreed with 3GPP working procedures and ETSI as the host of the Forge repository.

Observation 5: Handling of JSON files can be performed offline during the RAN4 meetings.

Observation 6: It is only the main rapporteur who is required to work within the ETSI Forge environment.

Observation 7: The Technical Specification Group working methods (TS 21.900) specifies how references shall be made from BigCR coversheet to the ETSI Forge Merge-Request to the Band Combination Database.

Observation 8: The updated Band Combination Database can be visualized during the RAN4 meeting.

Observation 9: The push of a merge request to the Band Combination Database will be handled by a MCC officer.

Observation 10: Tags will be used in the Band Combination Database to reflect which specification release is presented.

Observation 11: Using the Band Combination Databases version control allows used to see supported band combinations from a specific release of the specification.

Proposal 4: RAN4 shall further examine the agreed “Phase 1 process” with the perspective of the practicalities now presented after discussion with ETSI MCC.

Observation 12: RAN4 shall consider whether there is a need to reflect the other releases than then latest within the EISI Web App.

Observation 13: MCC is currently required by TS 21.900 to include all the latest versions of the JSON files within the TS zip file.

Observation 14: It can be questioned whether including all JSON files within the TS is practical for the RAN4 specifications.

Observation 15: RAN4 has no intention of changing the working methods of TS 21.900 as of now and will therefore include the JSON files within the TS zip.

Observation 16: Since the content of the Band Combination Database is to be included to the TS zip file it will be available for all who currently uses the TS, even offline.

Proposal 5: RAN4 shall further discuss how the supported band combinations can be best visualized also within the downloadable TS zip file. The visualization shall be based on the content of the Band Combination Database.

Observation 17: ETSI have its own 3GPP Git(lab) instance known as the ETSI Forge hosting the Band Combination Database

Observation 18: All 3GPP members have access to the Band Combination Database, where JSON data and schema files for bands and band combinations are stored.

Observation 19: The JSON files currently within the Band Combination Database is not necessarily in sync with the official DOCX-based specifications.

Observation 20: The alignment between official releases of the specification and the versions of the Band Combination Database will be implemented at a later stage.

Observation 21: New structure for the ETSI Forge repository for the Band Combination Database is presented in Fig.  3

Observation 22: With the proposed structure for the Band Combination Database, it is possible to have versions tagged in alignment to the specification release.

Proposal 6: RAN4 shall encourage ETSI to restructure the Forge repository according to the endorsed structure.

Observation 23: All the content of the latest version (main branch) of the Band Combination Database can be downloaded using the EOL account login information.

Observation 24: ETSI is working on providing a simple tool for generating, editing and validating JSON data files.

Observation 25: Companies may create their own tools for more advanced generation and/or modifications of JSON data files.

Observation 26: ETSI have provided a JSON data file visualizer accessible via the Web App.

Observation 27: A JSON data file represents the information we currently have in each row of the band combination tables in the current DOCX specifications.

Observation 28: The JSON schema files define the structure (i.e table-structure) of the content of the JSON data files.

These two are further explained in the following.

Observation 29: It is considered to add an information field within the JSON data file to allow tracing changes and origin of the individual band combinations.

Proposal 7: RAN4 shall discuss what information is relevant for enabling tracing changes and origin of the individual band combinations in addition to Tdoc number of the latest agreed JSON datafile and WI code, if any.

Observation 30: It is possible to have the ETSI forge environment provide all the information needed to allow tracing changes and origin of the individual band combinations.

Observation 31: It is intended the pre-fix, i.e. “CA_” or “DC_” in front of both filenames and bcId handle to separate the two types of band combinations now within the scope of this work.

Observation 32: JSON editors can use the schema files to assist by auto-completion when writing or editing new data.

Observation 33: There are currently two different JSON schema files in the ETSI Forge. One for the Channel Bandwidths and one for the NR CA combinations.

Observation 34: There will be a new JSON schema file for DC combinations, as the ones used now are specific to NR CA combinations or channel bandwidths.

Observation 35: Frequent changes to the JSON schema should be avoided by RAN4.

Proposal 8: RAN4 shall strive to finalize JSON schema files before Dec. 2025 to ensure these are ready for the Rel-20 band combination work.

Observation 36: The JSON schema files include the possible additional attributes of a band combination as e.g. additional UL power class support and/or DL channel bandwidth restrictions.

Observation 37: JSON files need to be uploaded within the Tdoc zip files as shown in the 3-steps above.

Observation 38: As captured in WF at RAN4#115, RAN4 is already now encouraged to start working with the JSON files and database to get a more tangible understanding of what it is and can become.

Observation 39: All companies will have to work with the Band Combination Database latest in Rel-20.

Observation 40: RAN4 needs to conclude whether the JSON files shall be available via the TS zip or only via the ETSI Forge.

Observation 41: There is 6 months from December 2025 until June 2026 where there will no published official Rel-20 RAN4 specification.

Proposal 9: RAN4 shall mandate inclusion of JSON files for all new or modified band combinations starting from Rel-20

Proposal 10: RAN4 shall discuss whether the inclusion of band combination-specific requirements in the Band Combination Database shall be part of the 6G SI.

- References

- SP-250802, Study on Modernization of Specification Format and Procedures for 6G, Nokia et. al.

- R4-2514216 Progress update and next steps for the Band Combination Database, Nokia, Ericsson

- R4-2420341, WF on next step for CA framework database, Nokia

- R4-2502976, WF on next step for CA framework database, Nokia

- R4-2508014, WF for the ETSI band combination database, Nokia, Ericsson

- R4-2511804, WF for the ETSI band combination database, Nokia

- R4-2514595, WF on Band Combination Database, Nokia

- R4-2507377, Progress update and next steps for the ETSI MCC band combination database, Nokia, Ericsson

- Annex A

- Explanatory comments to the rows in the JSON structure

The following shows an example with explanatory comments to the rows in the JSON structure.

{

"bcId": "n3A-n20A", // Unique identifier for the band combination

"bcsList": [

{

"bcsId": 0, // Unique identifier for this BCS

"ulConfigList": [

{

"bandNumber": "n3", // Uplink band number

"notes": {

"pc2_2tx": true // Uplink-specific note: Power Class 2, 2Tx

}

},

{

"bcId": "n3A-n20A" // Reference to another band combination for uplink

}

],

"bandList": [

{

"bandNumber": "n3", // Band number

"nonContiguousCarriers": [

[5, 10, 15, 20, 25, 30, 40] // List of supported non-contiguous carrier bandwidths (MHz)

]

},

{

"bandNumber": "n20",

"nonContiguousCarriers": [

[5, 10, 15, 20] // List of supported non-contiguous carrier bandwidths (MHz)

]

}

]

},

{

"bcsId": 4,

"ulConfigList": [

{

"bandNumber": "n3",

"notes": {

"pc2_2tx": true

}

},

{

"bcId": "n3A-n20A"

}

],

"bandList": [

{

"bandNumber": "n3",

"referencedComponents": [

{

"bandNumber": "n3" // Reference to a single-carrier band

}

]

},

{

"bandNumber": "n20",

"referencedComponents": [

{

"bandNumber": "n20"

}

]

}

]

}

]

}