3GPP TSG-RAN WG4 Meeting #117	R4-2521630

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on RRM testing for NGSO

Source: 	Huawei, HiSilicon

Agenda item:	6.6.3.2

Document for:	Discussion

Introduction

RRM testing with TE-emulated channel model with varying Doppler and delay shifts (so called new channel model) are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Applicability of new channel model for the existing test cases

- Applicability of the new channel model for pre-Rel-19 NTN UEs

In this paper, we will provide our views on RRM testing for NGSO.

Discussion

Applicability of new channel model for the existing test cases

#### Issue 2-2-1: Extend the applicability of the new channel model for the existing test cases WayForward - Further discuss the options. - Option 1: apply to only UL transmit timing test cases. - Option 2: apply to all NTN FR1 RRM tests. - Companies are encouraged to provide a more concrete analysis of the impact of RRM test cases.

We support option 1, i.e. to apply the new channel model only to UL transmit timing test cases.

- Procedure wise, the WID [1] is very clear that only UL transmit timing test cases are considered for the new channel model. We believe a change to the WID is needed in RAN Plenary before RAN4 extends the scope to cover more test cases. In RAN#109 there was a proposal to update the WID to extend the applicability of the new channel model, but there was no agreement.

- The main motivation to define the new channel mode is to verify that UE can perform correct time and frequency pre-compensation. UL transmit timing is the most sensitive requirement to time varying Doppler and delay shifts. If a UE can pass the UL transmit timing test, we can safely assume the UE can correctly use ephemeris information to estimate satellite location and velocity.

- It is noted that Demod session has down selected a very limited set of test cases where the new channel model will be applied. We should keep aligned principle across different sessions in RAN4. In particular, as long as there is one single test case among all test cases defined in RAN4 RF, RRM and demod and RAN5 for signaling is using static channel model, the test mode needs to be implemented.

- Applying the new channel model to measurement and mobility test cases require RAN4 to define initial ephemeris for a second satellite, which has not been done in the BDaT session. After the ephemeris for the second satellite is ready, RAN4 need analyze the configurations of SMTC and MG in each of the test cases involving 2 satellites, to make sure the SMTC and MG cover the SSB of the concerned satellite.

- In addition, many RRM test cases need to be run with multiple iterations. It is unclear so far how to make sure the initial condition can remain constant at beginning of each test iteration. For example, if UE is powered off after each test iteration and then powered on for the next, there is no way to guarantee that UE has completed the initialization (e.g. get into connected mode and receive necessary configurations) by the time when satellite gets to the location corresponding to the specified ephemeris (with 30 degree elevation angle).

Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only.

Applicability of the new channel model for pre-Rel-19 NTN UEs

#### Issue 2-3-1: Applicability of the new channel model for pre-Rel-19 NTN UEs WayForward - Further discuss the options - Option 1: not apply to pre-Rel-19 test cases - Option 2: apply to pre-Rel-19 test cases subject to DUT declaration

We support option 1, i.e. not to apply the new channel model for pre-R19 UE. We understand there could already be pre-R19 UEs that have been tested or are to be tested in near term, and changing applicable channel model for the test may impact the commercialization process and increase test cost.

Proposal 2: The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UE only.

Conclusions

In this paper we provided our views on RRM testing for NGSO.

Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only.

Proposal 2: The TE-emulated channel model with varying Doppler and delay shifts is applied to R19 UE only.

Reference

- R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, Moderator (MediaTek inc.)