3GPP TSG-RAN WG4 Meeting #117	R4-2521633

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on performance requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	5.10.2

Document for:	Discussion

Introduction

RRM performance requirements for Ku band NTN are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Band group for accuracy requirements

- UE mobility in UL timing test

- Test cases for Ku bands in FR1-NTN

- Test applicability for VSAT supporting both FR1-NTN and FR2-NTN

In this paper, we will provide our views on RRM performance requirements for Ku band NTN.

Discussion

Band group for accuracy requirements

#### Issue 1-1-1: Measurement accuracy with new band Agreement - The legacy measurement accuracy requirements for Ka band NTN (FR2-NTN) shall be reused for Ku band with FR1 numerology and FR2 numerology. - Introduce FR1 numerology in FR2-NTN accuracy requirements table. - The following measurement accuracy requirements shall be considered: - Intra-frequency RSRP accuracy requirements - Inter-frequency RSRP accuracy requirements - Intra-frequency RSRQ accuracy requirements - Inter-frequency RSRQ accuracy requirements - Intra-frequency SINR accuracy requirements - Inter-frequency SINR accuracy requirements - L1-RSRP accuracy requirements - FFS the band group to include Ku band

Before introduction of Ku bands, bands in FR1-NTN and FR2-NTN are in separate band groups. So far, there is no band group defined for FR2-NTN in clause 3.5, as the band group is not used in clause 10 or Annex B for FR2-NTN.

For Ku bands, although band n247 and n248 are in FR1-NTN, our understanding is that they should not be included in the band group for FR1-NTN. In 38.101-5, the refsens requirements for band n247 and n248 are defined as OTA requirements and same as those for band n508 and n509.

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | ≤ -122 |
|  |  | 3 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | ≤ -122 |
| n248, n247, n509, n508 | Mobile VSAT | 4, 5 | ≤ -126 |
| n248, n247, n509, n508 | Mobile VSAT | 6 | ≤ -122 |

Proposal 1: Do not include band n247 and n248 in the band group for FR1-NTN.

UE mobility in UL timing test

#### Issue 1-2-2: UE transmit timing test for NGSO VSAT Agreement - Introduce NGSO configuration for transmit timing test case of a mobile VSAT. - Further discuss the need to emulate the mobility of the mobile NGSO VSAT UE.

We do not support to emulate UE mobility in the UL timing test.

- We understand the UL timing requirements for mobile VSAT are stringent, particularly from GNSS point of view. The assumed GNSS error is reduced from 50m in FR1-NTN to 10m in FR2-NTN. However, GNSS is not modeled in the test, but UE location and velocity are set by AT command, so the critical part of the requirements (GNSS performance) is not really verified.

- The channel model does not support UE mobility. To support UE mobility, the channel model needs to be updated such that the UE location and velocity in every milli-second are considered to generate delay and Doppler shift. We understand it may too late to update the channel model at this late stage of R19.

- To support UE mobility, RAN4 also needs to define the UE motion trajectory during the test time, so UE has to do GNSS location interpolation following the UE motion trajectory defined by RAN4, otherwise the UE will get penalized. As the UE location and velocity is set by AT command in the test which is used in test mode, this is effectively defining requirements for test mode.

Proposal 2: Do not emulate UE mobility in the UL timing test.

Test cases for Ku bands in FR1-NTN

#### Issue 1-2-4: Test cases for Ku bands in FR1-NTN WayForward - Further discuss the following options - Option 1: reuse exiting test cases defined for SAN FR1 - Add a table clarifying test applicability for the corresponding test cases for VSAT UEs. - Add OTA related parameters - Option 2: reuse existing test cases defined for FR2-NTN, unless otherwise specified - Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

We do not have strong view, but slightly prefer option 2.

Although the spec impacts and efforts in drafting CR may be smaller with option 1, option 2 is technically more reasonable and more aligned with the core requirements. The core requirements for Ku band in FR1-NTN is mostly re-using those defined for FR2-NTN, so for the test, it is more straightforward to add FR1-NTN related test configuration in FR2-NTN test cases.

Proposal 3: Re-use existing test cases defined for FR2-NTN for Ku band in FR1-NTN, and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Test applicability for VSAT supporting both FR1-NTN and FR2-NTN

#### Issue 1-2-5: Test applicability and principle Agreement - The testing related to satellite access defined in A.3.36 with FR2 can be a reference. - If mobile VSAT UE supports both GSO and NGSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. - RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists. - Option 1: it is at least to pass two test configurations: one with FR1 numerology and one with FR2 numerology. - Other options are not precluded.

For VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, our view is that this VSAT should pass the test with one configuration except for the UL timing test.

The consideration is that most of the RRM requirements applicable for VSAT are agnostic to SCS, so it is enough for the VSAT to pass the test with one numerology, and this will also help to reduce the test efforts. UL timing requirement is an exception because it is dependent on SCS.

Proposal 4: VSAT supporting Ku bands in both FR1-NTN and FR2-NTN should pass the test with one configuration (either FR1 or FR2 numerology), except for the UL timing test.

Conclusions

In this paper we provided our views on RRM performance requirements for Ku band NTN.

Proposal 1: Do not include band n247 and n248 in the band group for FR1-NTN.

Proposal 2: Do not emulate UE mobility in the UL timing test.

Proposal 3: Re-use existing test cases defined for FR2-NTN for Ku band in FR1-NTN, and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Proposal 4: VSAT supporting Ku bands in both FR1-NTN and FR2-NTN should pass the test with one configuration (either FR1 or FR2 numerology), except for the UL timing test.

Reference

- R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, Moderator (MediaTek inc.)