3GPP TSG-RAN4 Meeting #117	R4-2521641

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on test configuration for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2514811, test configurations for LP-SS need to be introduced as RMC for LP-WUR test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test configurations for LP-SS as RMC for LP-WUR test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-SS RMC for LP-WUR test cases are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## A.3.X	LP-SS configurations

### A.3.X.1	LP-SS Configuration 1: M=1

Table A.3.X.1-1: LP-SS.1: LP-SS with M=1

| Field | Value |
| --- | --- |
| lpss-MvalueAndSeqConfig | nOne |
| lpss-BinarySeqLen | n8 |
| lpss-OverlaidSeqRoot | 1 |
| lpss-PeriodicityAndOffset | ms320, 4 |
| lpss-StartSymbol | startSymbol1 = 3 |
| lpss-EPRE-Ratio | dB0 |
| lpwus-LPSS-StartRB | NOTE 1 |
| lpwus-LPSS-BeamSubset | NOTE 2 |
| NOTE 1: The start RB for LP-SS is configured such that the LP-SS does not overlap with initial BWP. NOTE 2: Same as configured value as ssb-PositionsInBurst in SIB1. |  |

### A.3.X.2	LP-SS Configuration 2: M=4

Table A.3.X.2-1: LP-SS.2: LP-SS with M=4

| Field | Value |
| --- | --- |
| lpss-MvalueAndSeqConfig | nFour |
| lpss-BinarySeqLen | n32 |
| lpss-OverlaidSeqRoot | 1 |
| lpss-PeriodicityAndOffset | ms320, 4 |
| lpss-StartSymbol | startSymbol1 = 3 |
| lpss-EPRE-Ratio | dB0 |
| lpwus-LPSS-StartRB | NOTE 1 |
| lpwus-LPSS-BeamSubset | NOTE 2 |
| NOTE 1: The start RB for LP-SS is configured such that the LP-SS does not overlap with initial BWP. NOTE 2: Same as configured value as ssb-PositionsInBurst in SIB1. |  |

<End of Change 1>