3GPP TSG-RAN WG4 Meeting #117	R4-2521649

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on performance requirements for R19 A-IoT

Source: 	Huawei, HiSilicon

Agenda item:	6.14.3

Document for:	Discussion

Introduction

RRM performance requirements for R19 A-IoT are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Procedure and parameter for the test case

In this paper, we will provide our views on RRM test case for R19 A-IoT.

Discussion

In RAN4#116-bis, following was agreed for RRM test case for R19 A-IoT.

- Issue 1-1-1: Test case for TD2R_min Agreement: - Test the performance of TD2R_min. And TD2R_min are tested with other RRM requirements. - Issue 1-1-2: How to define RRM test cases Agreement: - Define a single dedicated RRM test case - Transmit timing requirements, RA procedure and TD2R_min are tested in the same test - Issue 1-1-3: Test configuration/procedure Agreement: - In high level, take the test procedures/FRCs for sensitivity from RF session as baseline - CBRA is in use in the test

RAN4 needs to discuss the detailed procedure and parameter for the test case. Figure 1 shows our view on the procedure of the test case, which is described below.

- Step 1: TE sends paging message.

- Step 2: Device transmits Msg1 at Toffset after the paging message.

- Step 3: TE sends Msg2/RAR at TD2R_min after receiving Msg1, which contains the same access random ID as transmitted by the device in Step 2.

- Step 4: Device transmits Msg3 at Toffset after the Msg2.

A correct device shall transmit Msg1 and Msg3 at the defined time point, and the D2R transmission shall meet the timing accuracy requirement.

Figure 1: Procedure of the RRM test case

Besides the procedure, there are also several parameters that needs to be discussed in RAN4.

- For R2D transmission, i.e. paging message and RAR, we suggest to re-use FRC A-FR1-B1-1 in clause B.2.1 of 38191 which is used for refsens test. It is noted that in this FRC, M=2 and number of PRB is 1.

Table B.2.1-1: Fixed Reference Channels for reference sensitivity level (OOK)

| Reference channel | Configuration | A-FR1-B1-1 | A-FR1-B1-2 | A-FR1-B1-3 | A-FR1-B1-4 |
| --- | --- | --- | --- | --- | --- |
| SIP | SCS | 15 | 15 | 15 | 15 |
|  | PRB | 1 | 2 | 3 | 4 |
|  | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
|  | chip number except for SIP,padding | 228 | 228 | 228 | 228 |
|  | Padding | 6 | 12 | 2 | 2 |

- For the number of access occasions, we suggest to set “Number of Access Occasions” to 1. This means there is no TDM (X=1) or FDM (single RSFS value) of access occasions, and there is no need for TE to send access trigger. The intention is to reduce the implementation complexity of TE, as well as the uncertainty caused by different device selecting different access occasions.

- For R2D transmission, i.e. Msg1 and Msg3, we suggest to re-use FRC A-FR1-A1-1 in clause B.1 of 38194 which is used for BS test. Of course, since RRM test is for UE, we do not need to define “modulation” and “SFO” in the FRC.

| Reference channel | A-FR1-A1-1 | A-FR1-A1-2 | A-FR1-A1-3 | A-FR1-A1-4 |
| --- | --- | --- | --- | --- |
| DSB (kHz) | 15 | 15 | 2880 | 2880 |
| Payload size (bits) | 96 | 96 | 96 | 96 |
| CRC (bits) | 16 | 16 | 16 | 16 |
| Preamble length | 31 | 31 | 31 | 31 |
| Midamble length | 31 | 31 | 31 | 31 |
| Midamble configuration I | 48 | 48 | 48 | 48 |
| FEC | 1/3 | 1/3 | 1/3 | 1/3 |
| Line code | Manchester | Manchester | Manchester | Manchester |
| Modulation | BPSK | OOK | BPSK | OOK |
| Waveform (CW) | unmodulated single tone | Unmodulated single tone | unmodulated single tone | unmodulated single tone |
| Sampling frequency（SFO） | between 0.01 and 0.1 | Between 0.01 and 0.1 | between 0.01 and 0.1 | Between 0.01 and 0.1 |
| Total symbols | 398 | 398 | 398 | 398 |

  - It is noted that in this FRC, the transmission BW is 15kHz, so the bit duration configured by Bit Duration in the R2D transmission should be ‘2’.

| Bit Duration | 3 bits | {2, 1, 1/2, 1/4, 1/8, 1/16, 1/32, 1/96} ×τ, where τ=2×106/15000 | The duration in microseconds of each D2R bit. | TbitD2R |
| --- | --- | --- | --- | --- |

  - RAN4 also needs to define the small frequency shift factor RSFS, which is configured by Frequency Resource IndicationBroadcast in the R2D transmission. This parameter is not reflected in the FRC, and we suggest to use value ‘8’ which is the middle of the configurable values.

| Frequency Resource IndicationBroadcast | 8 bits | An 8-bit bitmap. The values of small frequency shift factor are {1, 2, 4, 8, 16, 32, 64, 128}. In the bitmap, the first/leftmost bit of the bitmap corresponds to the first value of small frequency shift factor, the second bit corresponds to the second value of small frequency shift factor, and so on. For each bit, value 0 indicates that the corresponding value is not allowed, while value 1 indicates that the corresponding value can be used. | This field indicates: - the set of NSFS potential small frequency shift factors when present in A-IoT Paging message for CBRA. Each small frequency shift factor corresponding to X access occasion(s). NSFS is the number of frequency domain resource of access occasions triggered by A-IoT Paging message or one Access Trigger message, i.e., the number of bits set to value 1. Or - one or multiple values of small frequency shift factor when present in Random ID Response message. A device determines its small frequency shift factor value for the following D2R transmission based on its order of Echoed Random ID field in the Random ID Response message, i.e., the i-th device selects the i-th small frequency shift factor. | RSFS associated to the selected access occasion or configured resource for D2R transmission |
| --- | --- | --- | --- | --- |

It is noted that once RAN4 decides on the bit duration and the frequency shift factor RSFS, the chip length for D2R transmission can be derived, and the value for the Toffset and TD2R_min can be calculated following RAN1 spec.

Proposal 1: RAN4 to adopt the following procedures for the RRM test case.

- Step 1: TE sends paging message.

- Step 2: Device transmits Msg1 at Toffset after the paging message.

- Step 3: TE sends Msg2/RAR at TD2R_min after receiving Msg1, which contains the same access random ID as transmitted by the device in Step 2.

- Step 4: Device transmits Msg3 at Toffset after the Msg2.

A correct device shall transmit Msg1 and Msg3 at the defined time point, and the D2R transmission shall meet the timing accuracy requirement.

Proposal 2: RAN4 to adopt the following parameters for the RRM test case.

- For R2D transmission, re-use FRC A-FR1-B1-1 in clause B.2.1 of 38191,

- For R2D transmission, re-use FRC A-FR1-A1-1 in clause B.1 of 38194 with following update

  - Remove “modulation” and “SFO” for RRM test,

  - In addition, define the small frequency shift factor as ‘8’,

- Set “Number of Access Occasions” in Paging message to 1.

Conclusions

In this paper we provided our views on RRM test case for R19 A-IoT.

Proposal 1: RAN4 to adopt the following procedures for the RRM test case.

- Step 1: TE sends paging message.

- Step 2: Device transmits Msg1 at Toffset after the paging message.

- Step 3: TE sends Msg2/RAR at TD2R_min after receiving Msg1, which contains the same access random ID as transmitted by the device in Step 2.

- Step 4: Device transmits Msg3 at Toffset after the Msg2.

A correct device shall transmit Msg1 and Msg3 at the defined time point, and the D2R transmission shall meet the timing accuracy requirement.

Proposal 2: RAN4 to adopt the following parameters for the RRM test case.

- For R2D transmission, re-use FRC A-FR1-B1-1 in clause B.2.1 of 38191,

- For R2D transmission, re-use FRC A-FR1-A1-1 in clause B.1 of 38194 with following update

  - Remove “modulation” and “SFO” for RRM test,

  - In addition, define the small frequency shift factor as ‘8’,

- Set “Number of Access Occasions” in Paging message to 1.

Reference

- R4-2515031, Way Forward for [116bis][315] A-IoT_device, Moderator (CMCC)