3GPP TSG-RAN4 Meeting #117	R4-2521650

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM test case for A-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2515031, RAN4 should define a one RRM test case for A-IoT to verify RRM requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define RRM test case for A-IoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for A-IoT are not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9A	RRM Test Case

9A.1	Introduction

The purpose of this clause is to define test cases and the relevant configurations for verifying various types of RRM requirements for Ambient IoT device.

9A.2	Test case

9A.1.1	Test purpose and environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements, that the D2R transmit timing are within specified limits, and that R2D is monitored TD2R_min after the end of a D2R transmission. This test will verify the requirements in clause 9.1, 9.2 and 9.3 in an AWGN model. The step 1) to 4) in measurement procedure defined in clause 8.4.3 applies to this test.

9A.1.2	Test procedure and parameters

The test consists 3 successive time periods, T1, T2 and T3.

At the beginning of T1, test equipment transmits an R2D message carrying A-IoT Paging message. The test equiments determines whether there is a D2R transmission carrying Access Random ID (Msg1) from the device TR→D  after the end of the transmitted A-IoT Paging message.

T2 starts at the beginning of Msg1 transmission, and ends at TD2R_min after the end of Msg1 transmission.

At the beginning of T3, test equipment transmits an R2D message carrying Random ID Response message (Msg2). The test equiments determines whether there is a D2R transmission carrying D2R Upper Layer Data Transfer message (Msg3) from the device TR→D  after the end of the transmitted Msg2.

The general parameter for the test is in Table 9A.1.2-1. The configuration for R2D tranmsisison is in Table 9A.1.2-2, and the configuration for D2R tranmsisison is in Table 9A.1.2-3.

Table 9A.1.2-1: General test parameters for Ambient IoT RRM test case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | s | TBD |  |
| T2 | s | TBD |  |
| T3 | s | TBD |  |
| Number of access occasions | s | 1 | Configured by Number of Access Occasions as defined in clause 6.2.1.1 of TS 38.391 [6]. |

Table 9A.1.2-2: R2D related test parameters for Ambient IoT RRM test case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRB number |  | 1 |  |
| Number of chips per OFDM symbol |  | 2 | Corresponds to Mchipsymb in TS 38.291 [3] |
| Transport block (TB) size | bits | 96 |  |
| CRC size | bits | 16 |  |

Table 9A.1.2-3: R2D related test parameters for Ambient IoT RRM test case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Dual Side BW | kHz | 15 |  |
| Bit duration |  | 2 | Corresponds to Bit Duration in clause 6.2.1.1 of TS 38.391 [6] |
| Payload size | bits | 96 |  |
| CRC | bits | 16 | Corresponds to Mchipsymb in TS 38.291 [3] |
| Sequence length of preamble/midamble | bits | 31 | Corresponds to Lamble in TS 38.291 [3] |
| Midamble interval |  | 48 | Corresponds to Ibit in TS 38.291 [3] |
| FEC rate |  | 1/3 |  |
| Frequency Resource Indication for braodcast |  | 0001 0000 | Configured by Frequency Resource IndicationBroadcast as defined in clause 6.2.1.1 of TS 38.391 [6] Corresponds to RSFS = 8 |

9A.1.3	Test requirements

During T1, the device shall tranmsit a D2R message carrying Access Random ID (Msg1) TR→D  after the end of the transmitted A-IoT Paging message. The transmit timing shall be within ±Te from the reference point, where Te and the reference point are defined in clause 9.2.1.

During T3, the device shall tranmsit a D2R message carrying D2R Upper Layer Data Transfer message (Msg3) TR→D  after the end of the transmitted Msg2. The transmit timing shall be within ±Te from the reference point, where Te and the reference point are defined in clause 9.2.1.

<End of Change 1>