3GPP TSG-RAN WG4 Meeting #117	R4-2521651

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on time pre-compensation for IoT NTN TDD

Source: 	Huawei, HiSilicon

Agenda item:	4.27.4

Document for:	Discussion

Introduction

In RAN1#122-bis, an LS [1] is sent to RAN4 related to segmented pre-compensation in IoT NTN TDD, with the following action.

The RAN4 CR may not be aligned with the latest RAN1 agreement. RAN1 would like to ask RAN4 if they have concerns on the updated agreement, and: - If RAN4 has concerns on the RAN1 agreement, RAN1 will modify its specifications to introduce segmented precompensation within the set of 8 consecutive uplink subframes. - If RAN4 has no concerns on the RAN1 agreement, we recommend RAN4 to update their specifications to clarify that segmented precompensation is not supported in IoT NTN TDD mode.

In this paper, we will provide our views on segmented timing pre-compensation in IoT NTN TDD.

Discussion

In [1] RAN1 included the following agreement.

Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation

We understand what RAN1 mentioned as “not aligned with the latest RAN1 agreement” is the following text in UL timing requirements in 36.133. It is noted that RF spec may have counterpart in 38.102, but this paper is focused on RRM part.

### 7.20B.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20B.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA for NPRACH is defined as 0. NTA+NTA,offset+NTA,adjcommon+NTA,adjUE (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22B was applied. NTA for other channels is not changed until next timing advance is received. Table 7.20B.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. Note 1: TS is the basic timing unit defined in TS 36.211. When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in clause 7.22B is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in Table 7.20B.2-1. When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds. 2) The minimum aggregate adjustment rate, apart from a change ofNTA,adjUE due to satellite position update and NTA,adjcommonbetween the previous transmission and the current transmission, shall be 7*TS per 1second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, shall be 58.33*TS per 200ms. when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

The highlighted text means UE is allowed to adjust the uplink transmission timing with segmentation during the set of 8 consecutive uplink subframes, which is not aligned with highlighted RAN1 agreements.

Technically, in IoT NTN TDD, UE is allowed to do pre-compensation adjustment before each of the set of 8 consecutive uplink subframes, so the set of 8 consecutive uplink subframes is a segment by nature. We do not see the need to have further segmentation within the 8 subframes. Our simulation in RAN1 showed that in the scenario of band 249, the segment length can be larger than 10ms. It is noted that segmentation comes with the pre-compensation gap which will impact the data performance. In IoT NTN TDD, the data resource is already scarce, so we prefer to not allow pre-compensation gap as agreed in RAN1.

Proposal: RAN4 to confirm that there is no concern on the updated RAN1 agreement from RRM perspective, and RAN4 to make the following update to clause 7.20B.

…… when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission, as defined above.

Conclusions

In this paper we provided our views on segmented timing pre-compensation in IoT NTN TDD.

Proposal: RAN4 to confirm that there is no concern on the updated RAN1 agreement from RRM perspective, and RAN4 to make the following update to clause 7.20B.

…… when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission, as defined above.

Reference

- R1-2508064, Reply LS on precompensation for NB-IoT NTN TDD mode, RAN1