3GPP TSG-RAN WG4 Meeting #117	R4-2521655

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on RRM impact of Rel-20 NTN enhancement

Source: 	Huawei, HiSilicon

Agenda item:	7.6.4.2

Document for:	Discussion

Introduction

RRM core requirements for R20 NTN enhancement are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Requirements impacted by HD-FDD

- Collision handling

In this paper, we will provide our views on RRM core requirements for R20 NTN enhancement.

Discussion

Requirements impacted by HD-FDD

Issue 2-1-3: Handover interruption time WayForward - Option 1: HD-FDD has no impact on NR handover. - Option 2: FFS impact due to reduced Rx time - Further discuss the following conditions for HD-FDD - For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - One SSB is available during T∆ - One SSB is available during TIU

We do not see mobility requirements, including HO, CHO and satellite switch, are impacted by HD-FDD. The same issue has been discussed in R19 for RedCap. In R19, RAN1 defined the collision handling rule such that SSB reception is always prioritized over UL. Following same principle, in RAN4 it was agreed that no spec change is needed for HD-FDD because the adding side condition on SSB availability implies that UE may in some cases prioritize UL over SSB, which is not the case, i.e. there is no case where SSB would be not available during mobility procedure. The same principle should apply for VSAT.

Proposal 1: RAN4 to confirm that HO requirements are not impacted by HD-FDD.

Issue 2-1-5: UE UL timing accuracy WayForward - Further discuss the following options - Option 1: HD-FDD has no impact on UE UL timing accuracy. - Option 2: FFS impact due to Tx/Rx pattern.

We do not see UL timing requirements are impacted by HD-FDD. In UL timing requirements, there is a side condition on SSB availability every 160ms. As discussed above, the SSB availability is not impacted by HD-FDD, following RAN1 collision handling rule for RedCap which RAN4 agreed to re-use for VSAT. It is also noted that there was no impact to UL timing requirements dur to HD-FDD in R19 RedCap requirements.

Proposal 2: RAN4 to confirm that UL timing requirements are not impacted by HD-FDD.

Issue 2-1-2/4/6/7: other RRM requirement impact due to HD-FDD WayForward - RAN4 to further discuss if any RRM requirement impact due to HD-FDD for the following aspects - Maximum interruption in paging reception - initial access/random access - inter and/or intra-measurement requirement - scheduling availability

Following R19 RedCap conclusions,

- Paging interruption requirements are impacted by HD-FDD, and we see the impact also applicable for VSAT.

- Random access requirements are impacted by HD-FDD, and we see the impact also applicable for VSAT.

- Inter and/or intra-measurement requirements are impacted by HD-FDD, where special scheduling restriction to UL during L3 measurement is defined, and we see the impact also applicable for VSAT.

- In addition, RLM and L1-RSRP requirements are also impacted by HD-FDD where CSI-RS based measurement is subject to RAN1 collision handling rule, and the L1 indication in RLM are impacted by HD-FDD, and we see the impact also applicable for VSAT.

Proposal 3: RAN4 to confirm that following requirements are impacted by HD-FDD.

- Paging interruption requirements

- Random access requirements

- Scheduling restriction requirements for inter and/or intra-measurement

- RLM and L1-RSRP requirements, including CSI-RS based measurement and L1 indication.

Collision handling

Issue 2-1-11/12: WayForward - RAN4 can further study the following aspects - RAN4 to discuss recommending to RAN1 that TA reporting should become a mandatory feature for Ku Bands supporting HD-FDD. - RAN4 to study potential enhancements in intra-satellite measurements to support mobility measurements in HD-FDD mode.

We do not see strong motivation to make TA reporting a mandatory feature for HD-FDD VSAT. On one hand, RAN1 has defined collision handling rule, so the scheduling can still work without UE TA reporting. Of course, UE supporting TA reporting may get less collision and may enjoy better throughput, and this is a normal case where a more capable UE can benefit from the additional capability. On the other hand, TA reporting causes additional signaling overhead. To get collision free, the TA reporting may need to be done every 10 or 20 seconds, and this overhead is non-negligible when number of UEs is large.

Proposal 4: RAN4 not to make TA reporting a mandatory feature for HD-FDD VSAT.

Conclusions

In this paper we provided our views on RRM core requirements for R20 NTN enhancement.

Proposal 1: RAN4 to confirm that HO requirements are not impacted by HD-FDD.

Proposal 2: RAN4 to confirm that UL timing requirements are not impacted by HD-FDD.

Proposal 3: RAN4 to confirm that following requirements are impacted by HD-FDD.

- Paging interruption requirements

- Random access requirements

- Scheduling restriction requirements for inter and/or intra-measurement

- RLM and L1-RSRP requirements, including CSI-RS based measurement and L1 indication.

Proposal 4: RAN4 not to make TA reporting a mandatory feature for HD-FDD VSAT.

Reference

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek