3GPP TSG-RAN WG4 Meeting #117                                          R4-2521659 Dallas, USA, November 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On BS conformance for new requirements for SBFD

Agenda Item:	6.13.2.1

Document for:	Approval

Introduction

During RAN4#116, more analysis on SBFD conformance testing are presented. Discussion progress can be found in WF [1]. In last meeting, good progress was made as captured in WF [2].

Issue 1-1-1: References and Definitions related to SBFD capable BS Agreement: - Discuss this when drafting CRs based on the work-split Issue 1-1-2: Manufacture declaration related to SBFD capable BSs Agreement: - Introduce new manufacture declarations related to SBFD capable BSs in TS 38.141-1 and TS 38.141-2 based on the work-split. - Remove multi-band operation from this release and align to Core specs. - For configuration-limited SBFD-capable BS, all RF channels that implement analogue filters shall be declared together with subband configuration declaration. - Add the declarations in existing table in clause 4.6 Issue 1-1-3: Testing applicability for SBFD operation Agreement: - If both DU/UD and DUD subband configurations are declared to be supported, both DU/UD and DUD subband configurations shall be tested. Issue 1-1-4: Test signal used to build Test Configurations when testing existing Tx requirements Agreement: - For configuration-limited SBFD-capable BS, Table 1 shall be used. FFS SCS in the table. - Table 1: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-limited. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; [30] kHz for FR1 [120] kHz for FR2-1. Subband configuration; All declared subband configurations. - For configuration-flexible SBFD-capable BS, use Table 2 as a starting point. FFS SCS in the table. - Table 2: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-flexible. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; [30] kHz for FR1 [120] kHz for FR2-1. Subband configuration; DUD; DU/UD. - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2.; - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL.. Issue 1-1-5: Test configuration Agreement: - Consider single carrier Issue 1-1-7: RF channel to be tested for SBFD operation Agreement: - For configuration-limited SBFD-capable BS, all declared RF channels that implement analogue filters shall be tested. - FFS RF channels to be tested for configuration-flexible SBFD-capable BS. Issue 1-1-9: MU and Test tolerance when testing existing Tx requirements Agreement: - Re-use existing MU for SBFD symbols testing requirements as a starting point. - FFS re-use existing TT for SBFD symbols testing requirements. Issue 1-2-5: Status of Tx when testing Rx requirements Agreement: When testing both conducted and OTA receiver requirements, transmitter shall be ON and transmitting.

In this contribution, we would like to share our views regarding this topic.

Discussion

Transient period between SBFD and non-SBFD

According to the WF, whether new test model is needed should be studied for the transient period between SBFD and non-SBFD. From our understanding, the existing TDD configuration can be used as baseline and necessary time-domain SBFD configuration should be included. The existing test procedure for transmitter transient period i.e., clause 6.4.2 in TS 38.141 can be used as starting point as well.

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Proposal 1: For the transition period between SBFD and non-SBFD symbols, reuse the existing TDD testing configuration and the existing test procedure for transmitter transient period as baseline. Necessary SBFD flavour should be added into testing configuration e.g. SBFD time domain configuration.

In-channel adjacent subband leakage

For this new RF requirement for SBFD, the existing test model should be updated since the DL signal is no longer transmitted in the whole carrier. Specifically, the number of RBs allocated for PDSCH should be the RB number assigned for DL subband i.e. NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case.

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-C and BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Proposal 2: For the In-channel adjacent subband leakage, reuse the test model NR-FR1-TM1.1 and NR-FR2-TM1.1 and change the NRB as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case. The test procedure for absolute ACLR limit can be used as baseline.

- For DU case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

- For UD case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB - NRB, SBFD, DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

- For DUD case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |
| # of PRBs PDSCH nRNTI=3 | NRB, SBFD, DL,2 |
| Modulation PDSCH nRNTI=3 | QPSK |
| Starting RB location of PDSCH nRNTI=3 | NRB - NRB, SBFD, DL, 2 |

In-channel adjacent subband blocking

Unlike the above new RF requirement for SBFD, we think the difference between it and the legacy in-band blocking is not that much, so we propose to use the existing test procedure for in-band blocking as basis and check if modification for SBFD is needed.

Proposal 3: For the In-channel adjacent subband blocking, use the existing test procedure for in-band blocking as baseline.

Conclusion

In this contribution we discussed on the conformance testing for new requirements for SBFD. We have the following proposals:

Proposal 1: For the transition period between SBFD and non-SBFD symbols, reuse the existing TDD testing configuration and the existing test procedure for transmitter transient period as baseline. Necessary SBFD flavour should be added into testing configuration e.g. SBFD time domain configuration.

Proposal 2: For the In-channel adjacent subband leakage, reuse the test model NR-FR1-TM1.1 and NR-FR2-TM1.1 and change the NRB as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case. The test procedure for absolute ACLR limit can be used as baseline.

- For DU case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

- For UD case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB - NRB, SBFD, DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

- For DUD case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |
| # of PRBs PDSCH nRNTI=3 | NRB, SBFD, DL,2 |
| Modulation PDSCH nRNTI=3 | QPSK |
| Starting RB location of PDSCH nRNTI=3 | NRB - NRB, SBFD, DL, 2 |

Proposal 3: For the In-channel adjacent subband blocking, use the existing test procedure for in-band blocking as baseline.

- References

- R4-2512665, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.

- R4-2515158, “Way Forward for SBFD BS conformance”, Ericsson, 3GPP TSG-RAN4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025.

- R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.