3GPP TSG-RAN WG4 Meeting #117                                          R4-2521660 Dallas, USA, November 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On BS conformance for legacy requirements for SBFD

Agenda Item:	6.13.2.2

Document for:	Approval

Introduction

During RAN4#116, more analysis on SBFD conformance testing are presented. Discussion progress can be found in WF [1]. In last meeting, good progress was made as captured in WF [2].

Issue 1-1-1: References and Definitions related to SBFD capable BS Agreement: - Discuss this when drafting CRs based on the work-split Issue 1-1-2: Manufacture declaration related to SBFD capable BSs Agreement: - Introduce new manufacture declarations related to SBFD capable BSs in TS 38.141-1 and TS 38.141-2 based on the work-split. - Remove multi-band operation from this release and align to Core specs. - For configuration-limited SBFD-capable BS, all RF channels that implement analogue filters shall be declared together with subband configuration declaration. - Add the declarations in existing table in clause 4.6 Issue 1-1-3: Testing applicability for SBFD operation Agreement: - If both DU/UD and DUD subband configurations are declared to be supported, both DU/UD and DUD subband configurations shall be tested. Issue 1-1-4: Test signal used to build Test Configurations when testing existing Tx requirements Agreement: - For configuration-limited SBFD-capable BS, Table 1 shall be used. FFS SCS in the table. - Table 1: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-limited. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; [30] kHz for FR1 [120] kHz for FR2-1. Subband configuration; All declared subband configurations. - For configuration-flexible SBFD-capable BS, use Table 2 as a starting point. FFS SCS in the table. - Table 2: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-flexible. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; [30] kHz for FR1 [120] kHz for FR2-1. Subband configuration; DUD; DU/UD. - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2.; - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL.. Issue 1-1-5: Test configuration Agreement: - Consider single carrier Issue 1-1-7: RF channel to be tested for SBFD operation Agreement: - For configuration-limited SBFD-capable BS, all declared RF channels that implement analogue filters shall be tested. - FFS RF channels to be tested for configuration-flexible SBFD-capable BS. Issue 1-1-9: MU and Test tolerance when testing existing Tx requirements Agreement: - Re-use existing MU for SBFD symbols testing requirements as a starting point. - FFS re-use existing TT for SBFD symbols testing requirements. Issue 1-2-5: Status of Tx when testing Rx requirements Agreement: When testing both conducted and OTA receiver requirements, transmitter shall be ON and transmitting.

In this contribution, we would like to share our views regarding this topic.

Discussion

Remaining issues

During last meeting, the SCS used for test signal configuration is FFS for both FR1 and FR2-1. From our understanding, since the applicable bands for SBFD in FR1 are TDD, it is straightforward to use 30 kHz as the SCS configuration for FR1 test signal. As for FR2-1, the 120 kHz SCS is commonly used in the real deployment. Therefore we think use 120 kHz as the SCS configuration for FR2-1 test signal is acceptable.

Proposal 1: For the test signal SCS configuration, confirm to use 30 kHz for FR1 and 120 kHz for FR2-1.

As RAN4 agreed to test all declared RF channels that implement analogue filters for configuration-limited SBFD-capable BS, whether to test all RF channels for configuration-flexible SBFD-capable BS is still FFS. In our view, it is acceptable to keep the consistency for testing between these two types of SBFD-capable BS considering that no loss in doing so and whether RF requirements under a sequence of scenarios can be met as expected can be verified.

Observation 1: For configuration-flexible SBFD-capable BS, testing simplification e.g. to test part of RF channels seems not a must.

In general, most of the test procedure can be reused except for new In-channel adjacent subband leakage and In-channel adjacent subband blocking requirement and the transient period between SBFD and non-SBFD.

Proposal 2: For the test procedure for SBFD, existing procedure can be reused except for new requirements including in-channel adjacent subband leakage, in-channel adjacent subband blocking and transient period between SBFD and non-SBFD.

Although most of the test setups will be reused between SBFD and non-SBFD symbols, whether there could be any impact on TT caused by simultaneous operating between Tx and Rx can be further considered. Otherwise, the existing TT can be reused for SBFD testing requirements.

Proposal 3: Pending on analysis for the impact on TT potentially caused by simultaneous operating between Tx and Rx to see whether existing TT can be reused for SBFD testing requirements.

Conclusion

In this contribution we discussed on the conformance testing for new requirements for SBFD. We have the following proposals:

Observation 1: For configuration-flexible SBFD-capable BS, testing simplification e.g. to test part of RF channels seems not a must.

Proposal 1: For the test signal SCS configuration, confirm to use 30 kHz for FR1 and 120 kHz for FR2-1.

Proposal 2: For the test procedure for SBFD, existing procedure can be reused except for new requirements including in-channel adjacent subband leakage, in-channel adjacent subband blocking and transient period between SBFD and non-SBFD.

Proposal 3: Pending on analysis for the impact on TT potentially caused by simultaneous operating between Tx and Rx to see whether existing TT can be reused for SBFD testing requirements.

- References

- R4-2512665, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.

- R4-2515158, “Way Forward for SBFD BS conformance”, Ericsson, 3GPP TSG-RAN4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025.

- R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.