3GPP TSG-RAN WG4#117	R4-2521666

Dallas, TX, USA, November 17th – 21st, 2025

Agenda item:	4.11.2.1

Source: 	Qualcomm

Title: 	Remaining aspects for (e)RedCap less than 5MHz RF

Document for:	Approval

# Introduction

Considerations on some remaining aspects for (e)RedCap less than 5MHz RF are provided in this contribution.

- Discussion

As per WF [1], the following aspects are open:

Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap

FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap

Issue 4-3: UE capability

FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

Our views are as follows.

Issue 4-1: We propose to apply same applicability for (e)RedCap 3MHz than for 3MHz for normal bands. In other words, if 3MHz is optional for non-(e)RedCap band, it shall be optional for(e)RedCap as well. Similarly, if 3MHz is mandatory for non-(e)RedCap band, it shall be mandatory for(e)RedCap as well. The rationale for this proposal is that currently there is discussion on-going on whether 3MHz should be mandated for some NR bands or not. In our view, it would not be meaningful to have different optional/mandatory rule for non-(e)RedCap and (e)RedCap.

Proposal 1: Agree that 3MHz (e)RedCap optionality/mandatoriness follows that of non-(e)RedCap for each band.

Issue 4-3: In our understanding, there are maximum 4 UE capabilities related to 3MHz which currently are not applicable for (e)RedCap. These UE capabilities are:

- support3MHz-ChannelBW-Asymmetric-r18

- support3MHz-ChannelBW-Symmetric-r18

- support12PRB-CORESET0-r18

- support12PRB-CORESET0-GSCN-41637-r18

Capability 4) is only for n100 while capabilities 1), 2), and 3) are general. In our view it’s clear that capabilities 2), 3), and 4) need to be modified by RAN2. The only capability which requires discussion in RAN4 is the capability 1). RAN4 should consider whether this capability is useful for (e)RedCap, as the whole purpose of 3MHz (e)RedCap was to support narrower BW than 5MHz. As of now, there seems to be no reason to start supporting asymmetric BW’s for (e)RedCap so hence we propose not to include that into RAN2 LS

Proposal 2: RAN4 should send and LS into RAN2 to ask them to modify capabilities support3MHz-ChannelBW-Symmetric-r18, support12PRB-CORESET0-r18, and support12PRB-CORESET0-GSCN-41637-r18 so that 3MHz can be supported for (e)RedCap

Proposal 3: Do not enable support3MHz-ChannelBW-Asymmetric-r18 for (e)RedCap

- Conclusion

Considerations on some remaining aspects for (e)RedCap less than 5MHz RF are provided in this contribution.

Proposal 1: Agree that 3MHz (e)RedCap optionality/mandatoriness follows that of non-(e)RedCap for each band.

Proposal 2: RAN4 should send and LS into RAN2 to ask them to modify capabilities support3MHz-ChannelBW-Symmetric-r18, support12PRB-CORESET0-r18, and support12PRB-CORESET0-GSCN-41637-r18 so that 3MHz can be supported for (e)RedCap.

Proposal 3: Do not enable support3MHz-ChannelBW-Asymmetric-r18 for (e)RedCap

- Reference

[1] R4-2515101, Way Forward for [116bis][323] NR_less_than_5MHz_RedCap_UERF, Sony