3GPP TSG-RAN WG4 Meeting # 117															 R4-2521681

Dallas, USA, 17th – 21st November, 2025

Agenda item:			7.8.4

Source:	Samsung

Title:	Discussion on RRM requirements for LB CA switching enhancement

Document for:	Discussion

1	Introduction

In this meeting, we will continue giving some analyses on the open issues for Rel-20 LB-CA on WF [1] agreed in last meeting.

2	Discussion

Sub-topic 1-1 General

2.1	Generic assumptions

Views collection from moderator summary.

- Proposal 1 (ZTE Corporation, Sanechips, Samsung): Reuse the MRTD requirement specified in R19 low band CA via switching - Proposal 1-1 (Samsung): The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching

In Rel-19, we thoroughly discussed what RRM core requirements shall be impacted to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching. To analyse the impact on RRM requirements of SDL SCC and FDD PCC, the group achieved a consensus on the generic assumptions, which are shown in Table 1.

Table 1. Assumption for Rel-19 LB CA via switching

|  | Assumption |
| --- | --- |
| MRTD | 3us |
| SSB | SSB based requirement as baseline |
| Band combinations | - RAN4 only defines requirements for CA with FDD PCC and one SDL SCC - Example band combinations: CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n67A, CA_n29A-n71A |

By comparing the two WIDs, the following conditions used to derive RRM requirements are the same:

- Deployment constraints except the limitation on no in-gap interference between DL carriers is considered in Rel-20 LB CA

- BC assumption that is only CA with FDD PCC and one SDL SCC is considered

- Semi-static switching pattern based on RRC configuration is considered to trigger the switching-based LB CA

In this sense, from our understanding, the generic assumptions defined in Rel-19 can be reused in Rel-20, including: The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching and SSB based SDL SCell operation.

Regarding the band combination assumption, since in last meeting, RF achieved the following agreements:

< R4-2515156> Issue 1: Work scope clarification - Do not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19 - Restrict the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier

In RRM, the same restriction on BC assumption can be also considered in Rel-20, and RRM to define requirements for CA with FDD PCC and one SDL SCC

Overall, from our understanding, the generic assumptions defined in Rel-19 can be reused in Rel-20, including: The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching; CA with FDD PCC and one SDL SCC.

Proposal 1: To define Rel-20 LB CA RRM requirement, the following generic assumptions defined in Rel-19 can be reused:

- The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching

- CA with FDD PCC and one SDL SCC

Proposal 2: For Rel-20 LB-CA via switching, RAN4 only defines requirements for CA with FDD PCC and one SDL SCC

Applicability of Rel-19 LB-CA RRM requirements

2.2 Rel-20 requirements

2.2.1 Issue 1-3-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC

Views collection from moderator summary.

- Proposal 1 (Apple, LG Electronics Inc., Samsung, Qualcomm Incorporated [Proposal 3]): In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC: - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL S`CC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

In Rel-19, the applicability of RRM requirement and switching pattern were confirmed, which are shown in Table 2

Table 2. Applicability analysis

| Applicability of switching pattern | - SCell which has been activated - SCell which is in activation procedure |
| --- | --- |
| Applicability of RRM requirement | For gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: - If all {the SSB occasions in SMTC window}or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching patterNo delay extension is allowed - If a fraction of all {the SSB occasions in SMTC window}or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching patterThe delay is extended in accordance to the fraction of available occasions - If no {the SSB occasions in SMTC window}or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching patterThe corresponding requirements do not apply |

To be more specific, in order to successfully perform L1/L3 measurement complying with different time relationship of measurement occasions and LB-CA switching pattern no matter for FDD PCell or SDL SCell, the restriction of overlapped occasions of measurement resources (i.e., SMTC window and L1 RS) and FDD PCell/SDL SCell reception duration is defined. Although in Rel-20 LB CA, we consider the switching behaviour between: FDD <-->FDD+SDL, requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC can be stretched from Rel-19 into Rel-20 LB-CA.

Proposal 3: In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Potential RRM requirement impact for Rel-20 LB-CA

2.2.2 Issue 1-4-1: SDL SCell related RRM requirement

<SCell activation>

Regarding SCell activation, not only SCell activation procedure itself, the activation of switching pattern is also involved, from this, there are four key aspects should be discussed carefully:

- Bullet 1: The timeline of switching pattern application, that is the time point of when UE shall apply the RRC configured switching pattern no matter for MAC-CE based SCell activation or RRC based direct SCell activation

- Bullet 2: How/When UE switches to SDL SCell no matter for MAC-CE based SCell activation or RRC based direct SCell activation

- Bullet 3: The timeline of SDL SCell activation itself (i.e., delay requirement) including the first available SSB reception and interruption

- Bullet 4: CSI measurement and reporting/L1-RSRP reporting

For bullet 1 and bullet 2, what matters most for the SCell activation procedure is the application time of switching pattern including TLBCA. While, in last meeting, the values of TLBCA were agreed for MAC CE based SCell activation and RRC based direct SCell activation, and the application times of enabling switching pattern in SCell activation were clear, accordingly. That is

<R4-2514794>, <R4-2512119 > Agreement: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based.

Since 1) we agreed that switching pattern applicability for RRM requirement considered in Rel-19 can be reused in Rel-20 LB-CA via switching, 2) based on the WID, the semi-static switching mechanism based on RRC configuration defined in Rel-19 for LB-CA via switching can be reused, 3) the fundamental idea of introducing TLBCA is related to pattern switching mechanism. In this sense, from our opinion, for the SCell which is in activation procedure, the application time of switching pattern including TLBCA defined in Rel-19 is still applicable for Rel-20 LB CA via switching.

Proposal 4: The application time of switching pattern including TLBCA for the SCell which is in activation procedure defined in Rel-19 is applicable for SDL SCell activation in Rel-20 LB CA via switching, containing

- MAC CE based SCell activation

- RRC based direct SCell activation

For bullet 3, since the most distinct part accounted in Rel-20 LB-CA (case 1 case 3) is that UE can receive on dual DL CA (FDD DL carrier and SDL carrier) simultaneously instead of on SDL carrier/ DL carrier only, due to the dual DL consideration, UE can truly enjoy the higher throughput enabled by SDL SCell scheduling, and the CA situation is similar to inter-band FR1 CA case. Besides, since the interruption or scheduling restriction during SDL SCell activation/deactivation and the corresponding requirement impact are pending on further discussion, we prefer that

Proposal 5: For the SCell activation/deactivation delay requirement identified for Rel-20 LB-CA via switching:

- Wait for more Rel-19 LB CA conclusions on e.g., the delay requirement and interruption/scheduling restriction

- How to clarify definitions of parameters (e.g., TfirstSSB/TfirstSSB_MAX) to reflect Rel-20 LB-CA enhancement

For bullet 4, in last meeting Rel-19 LB-CA, the relevant issue was extensively discussed, and following consensus was achieved, copied as below, the main reason is from the universal principle of the definition of TCSI_reporting in the current spec. is “…. first available downlink CSI reference resource, …in acquiring the first available CSI reporting resources”,

Issue 1-1-2: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA Agreement: The issue can be closed (without any additional clarification or spec change in RAN4).

In this sense, there is no need to discuss the same issue in Rel-20 LB CA as well, the requirement impact concluded in Rel-19 can be reused in Rel-20.

Proposal 6: In Rel-20 LB-CA via switching, for CQI/L1-RSRP reporting during SDL SCell activation in LB-CA requirement.

- The issue can be closed (without any additional clarification or spec change in RAN4).

<Interruption due to deactivated SCell measurement>

Recap from Rel-19 LB CA regarding interruption requirement for deactivated SCell measurement, the requirements are presented in the following Table 3

Table 3. RRM requirements impact (Interruption requirement for deactivated SDL SCell measurement)

| Requirement | Impact/Definition |
| --- | --- |
| Interruption length on PCC due to measurements on deactivated SDL | Tuning time in slot level + TSMTC_duation + retuning time in slot level - Tuning time from PCell to SCell/ Returning time from SCell to Pcell is related to LBCA-SwitchingGap-Duration-PCelltoSCell-r19 |
| Missed ACK/NACK rate | Up to X= Y/measCycleSCell, where Y=2* interruption length |

From our understanding, 1) no matter interruption length or missed ACK/NACK rate is not the same as what we concluded in Rel-19 LB CA anymore, since normal reception on FDD carrier is introduced ; 2) In Rel-20 LB CA, no matter single chain or dual chain reception, interruption will occur on the PCC UL and PCC DL,  even if for dual chain reception, the receiver chain will also be impacted due to the poor isolation between PLLs/VCOs. However, the definition of interruption on the PCC UL and PCC DL should to handle in different ways, since 1) in Rel-20 LB CA when UE to receive on SDL SCC, uplink transmission on the FDD carrier is not permitted, and 2)for DL interruption, we think the interruption length requirement is to some extent the similar one as for legacy inter-band case, and with this understanding, the interruption length on PCC due to measurements on deactivated SDL in Rel-20 LB CA is irrelevant to TSMTC_duation. And if the definition of interruption length is updated, the missed ACK/NACK rate will be updated, accordingly.

Observation 1: Interruption requirement for deactivated SDL SCell measurement including interruption length and missed ACK/NACK rate defined in Rel-19 LB CA is not applicable to Rel-20 LB CA

Observation 2: No matter which kind of RF reference architecture for Rel-20 LB CA is used, interruption will occur on the PCC UL and PCC DL.

Proposal 7: For Rel-20 LB CA interruption requirement of deactivated SDL SCell measurement

- RAN4 to discuss interruption requirement design of deactivated SDL SCell measurement, including interruption length on PCC (including UL and DL) and missed ACK/NACK rate

  - Interruption length on FDD DL and UL due to deactivated SCell measurement could be defined separately

2.2.3 Issue 1-4-2: FDD PCell related RRM requirement

Rel-20 LB CA dual DL reception mechanism presents a different challenge on defining RRM requirement for PCell, to be more specific, not to focus the time relationship of measurement resource occasions (SMTC or RSs for L1 measurement) and available reception duration of LB-CA switching pattern, that is fully overlapping, partially overlapping and non-overlapping, but to focus the case that loss of occasion (s) of reference signal reception on PCC caused by pattern switching (i.e., punctured by pattern switching), in this sense, the Rel-19 RRM requirement applicability to gap-less based L1/L3 measurement of an activated FDD PCC is not applicable any more, and the discussion on new RRM requirement applicability to gap-less based L1/L3 measurement of an activated FDD PCC is needed,  possible solutions can be 1) Extend the period to ensure UE can measurement a certain number of valid RS resources; 2) To define available RS resources taking switching pattern impacts into consideration.

Observation 3: For Rel-20 LB CA FDD PCC related RRM requirement, how to treat the case that loss of occasion (s) of reference signal reception on FDD PCC caused by pattern switching becomes new challenge.

Proposal 8: For Rel-20 LB CA, RAN4 to discuss the RRM requirement applicability to L1/L3 measurement of a FDD PCC, and to define the corresponding RRM requirement design for FDD PCell.

- RAN4 to discuss the applicability of legacy requirements taking switching pattern impacts into consideration

2.2.4 Issue 1-4-3: Active BWP switching for PCell and Scell

Since BWP switching requirement for LB CA is still suspending in Rel-19 LB CA, we prefer that keep the issue open in Rel-20 LB CA

Proposal 9: RAN4 to further discuss active BWP switching requirement for R20 LB-CA after completely concluding on active BWP switching requirement for Rel-19 LB-CA

2.2.5 Issue 1-4-4: Relation to Rel-19 LB CA via switching

In RF, the discussion on how to treat relation to Rel-19 LB CA via switching was recommended, copied as below

< R4-2515156 > Issue 9: Related to configuration - Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching

No doubt that it is important to clarify the relationship between the Rel-20 LB CA via switching enhancements and the existing Rel-19 framework. From our preliminary understanding, a UE may support only the Rel-19-type switching, only the Rel-20 dual-DL switching enhancements, or both, depending on its low-band Rx architecture and capability declarations. Besides, how to keep the specification consistent and manageable across release is not solid in RF. In this sense, we think there is no immediate need to treat the issue in RRM session, and we can wait for more conclusions achieved in RF.

Proposal 10: Relation to Rel-19 LB CA via switching in Rel-20 LB-CA RRM, postpone discussions and wait for Rel-20 RF LB-CA progress

2.3 Sub-topic 1-5 Potential RAN1 impacts

Two receiver architectures are on-going discussed in RF upping to UE implementation for Rel-20 LB-CA without any down-selection, i.e., single Rx chain architecture and dual Rx architecture. From our understanding, although the UE reference architecture is different from what we discussed in Rel-19, when UE to perform operation on the SDL carrier to complete a measurement/or data reception procedure, the essential functionality of the configured switching pattern is in principle the same as we discussed in Rel-19. In this sense, we think the requirements related to the switching pattern itself should be discussed in Rel-20 LB CA also, including: Interruption/scheduling restriction requirement for CC switching. In Rel-19 LB-CA RRM part, we achieved the following agreement for Rel-19 LB CA via switching,

< R4-2508259> Online Agreement on Tuesday: - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. - Note: the above agreement can be revisited if not aligned with RAN1 further agreement.

However, since interpretation on pattern switching behaviour is different between {case 1, case 2} and {case 1, case 3}, the Interruption/scheduling restriction requirement for CC switching defined in RAN1, copied as below may or may not be different between Rel-19 and Rel-20

TS 38.213 When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’.

In addition to interruption or scheduling restriction requirement, the same situation happend on definition of switching pattern specified in RAN1/RAN2.

TS 38.213 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration TS 38.331 LowBandCA-Switching field descriptions. gapDurationPCelltoSCell Indicates the duration of the switching gap for the switching from PCell to the SCell, in the unit of 15KHz SCS symbol. NW ensures that the switching gap is enough to cover at least the switching period.. gapDurationSCelltoPCell Indicates the duration of switching gap for the switching from the SCell to PCell, in the unit of 15KHz SCS symbol. NW ensures that the switching gap is enough to cover at least the switching period and TA.. switchingPattern Indicates which carrier is configured for the corresponding slot for low NR band carrier aggregation via switching between FDD and supplementary downlink (SDL) carrier. ‘0’ indicates FDD carrier (PCell), and ‘1’ indicates SDL carrier (SCell). The pattern starts from the beginning of SFN 0 of PCell...

Observation 4: The existing scheduling restriction/interruption requirements defined in RAN1 for Rel-19 LB CA switching may or may not be applicable for Rel-20 LB CA switching

Observation 5: The existing downlink carrier aggregation via switching behaviours defined in RAN1 for Rel-19 LB CA switching may or may not be applicable for Rel-20 LB CA switching

However, whether or not RAN4 need to define interruption and scheduling restriction depends on whether RAN1 will have such switching behaviour and rules for colliding defined in their spec. or not, before RAN1’s investigation, it is not appropriate to directly make a conclusion in RAN4 that the existing scheduling restriction/interruption requirements defined in RAN1 for Rel-19 LB CA switching are not be applicable for Rel-20 LB CA switching

Proposal 11: It is necessary to re-consider/modify interruption or scheduling restriction requirement for Rel-20 LB-CA switching in RAN1/RAN4

- Alt.1 RAN4 to define interruption or scheduling restriction requirement for carrier switching

- Alt.2 Postpone the discussion on interruption or scheduling restriction requirement in RAN4.

  - Ask RAN1 on the applicability of the existing collision handling rules between switching pattern and configured DL/UL resources, postpone sending the LS until more solid agreements are achieved in RAN4 or until RAN1 gets allocated TU

3	Conclusion

In this contribution, we provide our views focusing on Rel-20 LB-CA RRM aspects. Following observations and proposals were derived to be considered by this meeting.

Observation 1: Interruption requirement for deactivated SDL SCell measurement including interruption length and missed ACK/NACK rate defined in Rel-19 LB CA is not applicable to Rel-20 LB CA

Observation 2: No matter which kind of RF reference architecture for Rel-20 LB CA is used, interruption will occur on the PCC UL and PCC DL.

Observation 3: For Rel-20 LB CA FDD PCC related RRM requirement, how to treat the case that loss of occasion (s) of reference signal reception on FDD PCC caused by pattern switching becomes new challenge.

Observation 4: The existing scheduling restriction/interruption requirements defined in RAN1 for Rel-19 LB CA switching may or may not be applicable for Rel-20 LB CA switching

Observation 5: The existing downlink carrier aggregation via switching behaviours defined in RAN1 for Rel-19 LB CA switching may or may not be applicable for Rel-20 LB CA switching

Proposal 1: To define Rel-20 LB CA RRM requirement, the following generic assumptions defined in Rel-19 can be reused:

- The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching

- CA with FDD PCC and one SDL SCC

Proposal 2: For Rel-20 LB-CA via switching, RAN4 only defines requirements for CA with FDD PCC and one SDL SCC

Proposal 3: In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 4: The application time of switching pattern including TLBCA for the SCell which is in activation procedure defined in Rel-19 is applicable for SDL SCell activation in Rel-20 LB CA via switching, containing

- MAC CE based SCell activation

- RRC based direct SCell activation

Proposal 5: For the SCell activation/deactivation delay requirement identified for Rel-20 LB-CA via switching:

- Wait for more Rel-19 LB CA conclusions on e.g., the delay requirement and interruption/scheduling restriction

- How to clarify definitions of parameters (e.g., TfirstSSB/TfirstSSB_MAX) to reflect Rel-20 LB-CA enhancement

Proposal 6: In Rel-20 LB-CA via switching, for CQI/L1-RSRP reporting during SDL SCell activation in LB-CA requirement.

- The issue can be closed (without any additional clarification or spec change in RAN4).

Proposal 7: For Rel-20 LB CA interruption requirement of deactivated SDL SCell measurement

- RAN4 to discuss interruption requirement design of deactivated SDL SCell measurement, including interruption length on PCC (including UL and DL) and missed ACK/NACK rate

  - Interruption length on FDD DL and UL due to deactivated SCell measurement could be defined separately

Proposal 8: For Rel-20 LB CA, RAN4 to discuss the RRM requirement applicability to L1/L3 measurement of a FDD PCC, and to define the corresponding RRM requirement design for FDD PCell.

- RAN4 to discuss the applicability of legacy requirements taking switching pattern impacts into consideration

Proposal 9: RAN4 to further discuss active BWP switching requirement for R20 LB-CA after completely concluding on active BWP switching requirement for Rel-19 LB-CA

Proposal 10: Relation to Rel-19 LB CA via switching in Rel-20 LB-CA RRM, postpone discussions and wait for Rel-20 RF LB-CA progress

Proposal 11: It is necessary to re-consider/modify interruption or scheduling restriction requirement for Rel-20 LB-CA switching in RAN1/RAN4

- Alt.1 RAN4 to define interruption or scheduling restriction requirement for carrier switching

- Alt.2 Postpone the discussion on interruption or scheduling restriction requirement in RAN4.

  - Ask RAN1 on the applicability of the existing collision handling rules between switching pattern and configured DL/UL resources, postpone sending the LS until more solid agreements are achieved in RAN4 or until RAN1 gets allocated TU

Reference

[1]	R4-2514824	WF for [116bis][224] NR_LBCA_Sw_enh_RRM						RAN4#116bis

[2]	R4-2515156	Way Forward for [116bis][320] NR_low_band_CA_switching						RAN4#116bis