3GPP TSG-RAN WG4 Meeting #117	R4-2521684

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on L1-RSRP test case for SSB adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specify test case for L1-RSRP measurement when DRX is not used for SSB adaptation |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following agreements were achieved in last meeting for adaptation of common signal/channel transmission Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement Agreement: - Besides L3 intra-frequency measurement, specify the following test case for SSB adaptation - L1-RSRP measurement - L1-SINR measurement Issue 1-2: General principle Agreement: - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. Issue 1-3: SMTC and SSB configuration Agreement (in AH): - For L1-RSRP measurement test case: - SMTC periodicity from 20ms to 80ms, and from 80ms to 40ms. - SSB periodicity from 20ms to 80ms, and from 80ms to 40ms. Agreement - TC1: L3 intra-frequency measurement - FR1 - TC2: L1-RSRP measurement - PCell FR1 and SCell FR2 - TC3: L1-SINR measurement - FR1 Agreement - TC1: L3 intra-frequency measurement - FR1 - TC2: L1-RSRP measurement - PCell FR1 and SCell FR2 - TC3: L1-SINR measurement - FR1. |
| Consequences if not approved: |  | No test case for L1-RSRP measurement when DRX is not used for SSB adaptation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) A.5.6.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.5.6.3.X	SSB based L1-RSRP measurement when DRX is not used

A.5.6.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-RSRP measurement when SSB periodicity changes due to SSB adaptation. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1 and 9.5.7, with the testing configurations for NR FR1 PCell and NR FR2 SCells in table A.5.6.3.1.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15

Table A.5.6.3.X.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test (SCell)

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.6.3.X.1-2: Applicable NR configurations for FR1 SSB based L1-RSRP test (PCell)

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

A.5.6.3.X.2	Test parameters

There are two cells in the test, FR1 PCell (Cell 1) and FR2 SCell (Cell 2). The test parameters for the Cell 2 are given in table A.5.6.3.X.2-1 and table A.5.6.3.X.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1, T2 and T3 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured. The UE is configured to provide periodic L1-RSRP reports.

From the beginning of T2, UE receives a DCI format 2_9 indicating a change in SSB burst periodicity of the SSB #0 transmission on SCell, the periodicity of SSB #0 is changed from 20 ms to 80 ms.

From the beginning of T3, UE receives another DCI format 2_9 indicating a change in SSB burst periodicity of the SSB #0 transmission on SCell, the periodicity of SSB #0 is changed from 80 ms to 40 ms.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.5.6.3.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~4 |  | freq1 |
| Duplex mode | 1~4 |  | TDD |
| TDD Configuration | 1~4 |  | TDDConf.3.1 |
| BWchannel | 1~4 | MHz | 100: NPRB,c = 66 |
| Data PRBs allocated | 1~4 |  | 66 |
| PDSCH Reference measurement channel | 1,2 |  | SR.3.2 TDD |
|  | 3,4 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1,2 |  | CR.3.1 TDD |
|  | 3,4 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1,2 |  | CCR.3.1 TDD |
|  | 3,4 |  | CCR.3.7 TDD |
| SSB configuration for PCell | 1,4 |  | SSB.3 FR1 |
|  | 2,5 |  | SSB.3 FR1 |
|  | 3,6 |  | SSB.4 FR1 |
| SSB configuration for SCell | 1,2 |  | SSB.X2 FR2 for SCell during T1 SSB. 21 FR2 for SCell during T2 SSB.X3 FR2 except for SCell during T3 |
|  | 3,4 |  | SSB.X5 FR2 for SCell during T1 SSB.22 FR2 for SCell during T2 SSB.X6 for SCell during T3 |
| OCNG Patterns | 1~4 |  | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~4 |  | SMTC.10 |
| TRS Configuration | 1~4 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~4 |  | TCI.State.2 |
| DRX configuration | 1~4 |  | Off |
| reportConfigType | 1~4 |  | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| L1-RSRP reporting period | 1~4 | slot | 320 |
| T1 | 1~4 | s | 5 |
| T2 | 1~4 | s | 2 |
| T3 | 1~4 | s | 2 |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.5.6.3.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 4 | 1~4 |  | Rough |  |  |  |
| Note2 | 1~4 | dBm/15 kHz | -105 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -96 |  |  |  |
|  | 3,4 |  | -93 |  |  |  |
|  | 1~4 | dB | 0 | 0 | 0 |  |
| SSB_RP Note3 | 1,2 | dBm/SSB SCS | -96 | -96 | -96 |  |
|  | 3,4 |  | -93 | -93 | -93 |  |
| Io Note3 | 1,2 | dBm/95.04 MHz | -63.97 | -63.97 | -63.97 |  |
|  | 3,4 |  | -63.97 | -63.97 | -63.97 |  |
|  | 1~4 | dB | 0 | 0 | 0 |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

A.5.6.3.X.3	Test Requirements

During T1, UE shall send L1-RSRP report for SSB #0 every 320 slots. No later than X ms plus 320 slots from the beginning of time period T1, UE shall send L1-RSRP report SSB#0 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	1680 for UE supporting power class 1

-	1200 for UE supporting power class 2,3 or 4.

From the beginning of T2, UE receives a DCI format 2_9 indicating a change in SSB burst periodicity of the SSB #0 transmission on SCell, after Y ms from receiving the first SSB burst after the time point as defined in TS 38.213 plus 3 ms, UE shall send L1-RSRP report for SSB #0 with the updated periodicity while meeting the accuracy requirements defined in clause 10.1.20.1, where Y is

-	640 ms for UE supporting power class 3.

From the beginning of T3, UE receives another DCI format 2_9 indicating a change in SSB burst periodicity of the SSB #0 transmission on SCell, after Z ms from receiving the first SSB burst after the time point as defined in TS 38.213 plus 3 ms, UE shall send L1-RSRP report for SSB #0 with the updated periodicity while meeting the accuracy requirements defined in clause 10.1.20.1, where Z is

-	320 ms for UE supporting power class 3.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============End of change==============