3GPP TSG-RAN WG4 Meeting #117	R4-2521688

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.12.2

Source: 	Samsung

Title: 	Discussion on LLS simulation for BB error

Document for:	Discussion

- Introduction

In this meeting, we will give some LLS simulation discussions including simulation assumption and results for BB error based on WF [1] agreed in last meeting. The LLS simulation discussions are related to FR1.

- LLS simulation for BB error

Cell-specific parameters

- In the simulation, two cells are considered, which are Cell 1 (serves as an interfering cell and unknown to UE) and Cell 2 (serves as the cell under measurement and known to UE)

Table 1 Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel number | - | Channel 1 | Channel 1 |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| IOR | dB | Baseline: - 6.4 - 1.8 | Baseline: - 1.25 - 1 (SINR: -6dB; -3dB; FFS: 0dB; 3dB; -7dB; -8dB; -9dB ) |

General parameters

Table 2: General parameters (FR2)

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 30 GHz |
| DRX | No |
| BS antenna configurations | FFS：1/8/16/32 |
| UE receive antenna configuration | Baseline: 2 Rx Other options: |
| Subcarrier spacing SSB | Baseline: 120 kHz Other options: 240 kHz |
| Number of samples in L1 filtering | Baseline: 5 Other options e.g., 1, 3 |
| SS burst periodicity | Baseline: 5 ms Other options e.g., 10ms, 20 ms |
| Channel model | Fading channel Baseline: - TDL-C, DS=300ns Other options: CDL |
| UE speed | Baseline: - 30 km/h Other options: 60 km/h, 90 km/h, 120 km/h |
| FilterCoefficient in L3 filtering | Baseline: 0 Other options: Same as RAN2 assumption (4) |
| L1 filtering approach | Baseline: - Sliding Other options: Non-sliding |

Table 3: General parameters (FR1)

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz |
| DRX | No |
| BS antenna configurations | Baseline: 1 Tx or single layer transmissions Other options: |
| UE receive antenna configuration | Baseline: 2 Rx Other options: 4 Rx |
| Subcarrier spacing SSB | Baseline: 30 kHz Other options: 15 kHz |
| Number of samples in L1 filtering | Baseline: 5 Other options e.g., 1, 3 |
| SS burst periodicity | Baseline: 5 ms Other options e.g., 10ms, 20 ms |
| Channel model | Fading channel Baseline: - TDL-C, DS=30ns Other options: CDL |
| UE speed | Baseline: - 30 km/h Other options: 60 km/h, 90 km/h, 120 km/h |
| FilterCoefficient in L3 filtering | Baseline: 0 Other options: Same as RAN2 assumption (4) |
| L1 filtering approach | Baseline: - Sliding Other options: Non-sliding |

Initial results for FR1

- For L3 measurement accuracy, the CDF curves to be provided are as follows:

Delta SS-RSRP   = (estimated SS-RSRP – ideal SS-RSRP) 	[dB]

Delta SS-RSRQ   = (estimated SS-RSRQ – ideal SS-RSRQ) 	[dB]

Delta SS-SINR   = (estimated SS-SINR – ideal SS-SINR) 	[dB]

By comparing the numerical results in terms of the max (5%, 95%) from our initial results, we can find that the difference between the max (5%, 95%) of speed=30 km/h and speed=120 km/h for the same considered number of samples is less than 1 dB. That means speed variation has a limited impact on L3 measurement accuracy.

Observation 1:

- For FR1 L3 measurement accuracy, there is less than 1dB difference between different speeds for the same number of samples at the same SINR condition.

- For FR1, speed variation has a limited impact on L3 measurement accuracy for certain SINR condition.

Besides, for the same SINR condition, the L3 measurements with multiple samples can bring significant improvements in measurement accuracy, that means the discrepancy of BB error before and after multi-sample filtering is large, the BB error is suppressed after multi-sample filtering.

Observation 2: For FR1, the L3 measurements with multiple samples can bring significant improvements in measurement accuracy than one shot sample under the same SINR condition.

Proposal 1: Prefer that the distribution of BB error under certain SINR can be obtained through LLS assuming multi-shot measurement

Compared to AI/ML based BM, for AI MOB, L1 filtering and L3 filtering are both considered to obtain the cell level measurements. At the same time, since L3 filtered measurement results are not only related to the received measurement results from the physical layer, but also related to the old L3 filtered measurement results (e.g., Fn-1), RSRP prediction error with L3 filtering may be reduced compared with the one without L3 filtering. To evaluate the benchmark cases performance, the recommended value for L3 filtering parameter (i.e., FilterCoefficient) agreed in RAN2 is 4. Besides, in last meeting, we had the following agreements:

Agreement: - As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2. - Interested companies can also provide results for other sub-cases. - It is not precluded to add additional sub-case for FR2 in future meetings. - Further discuss how to define requirements based on the available simulation results. Ad-hoc agreements on Thursday: Agreement: As starting point, evaluate non-cluster (single-cell input and single-cell output) approach for frequency domain prediction as baseline. - Interested companies can also provide results for cluster approach. Further discuss the detailed simulation assumptions. - Further discuss how to define requirements based on the available simulation results.

That means the input is the data after L3 filtering. In this sense,

Proposal 2：RAN4 to discuss whether to/how to consider L3 filtering parameter (i.e., FilterCoefficient) when to obtain distribution of BB error through LLS simulations.

Proposal 3：If FilterCoefficient is not considered to obtain distribution of BB error through LLS simulations, suggest to add the BB error before L3 filtering.

Besides, we have the following concerns: 1) Upon deployment, AI models can encounter new data different from what the models were benchmarked on, 2) Measurement error may vary in a random way, and it is a combination of bias, variance and noise. In this sense, how to add the measurement error taking data uncertainty into consideration, and whether max (5%,95%) value from CDF is applicable/feasible to be considered into the BB error under fast fading channel need to be further discussed.

Proposal 4：RAN4 to discuss the applicability/feasibility of considering max (5%,95%) value from CDF at certain SINR into the BB error under fast fading channel consideration and how to add the BB error

- Conclusions

In this contribution, we provided our viewpoints on FR1 LLS simulation discussions including simulation assumption and results for BB error. The following proposals are obtained:

Observation 1:

- For FR1 L3 measurement accuracy, there is less than 1dB difference between different speeds for the same number of samples at the same SINR condition.

- For FR1, speed variation has a limited impact on L3 measurement accuracy for certain SINR condition.

Observation 2: For FR1, the L3 measurements with multiple samples can bring significant improvements in measurement accuracy than one shot sample under the same SINR condition.

Proposal 1: Prefer that the distribution of BB error under certain SINR can be obtained through LLS assuming multi-shot measurement

Proposal 2：RAN4 to discuss whether to/how to consider L3 filtering parameter (i.e., FilterCoefficient) when to obtain distribution of BB error through LLS simulations.

Proposal 3：If FilterCoefficient is not considered to obtain distribution of BB error through LLS simulations, suggest to add the BB error before L3 filtering.

Proposal 4：RAN4 to discuss the applicability/feasibility of considering max (5%,95%) value from CDF at certain SINR into the BB error under fast fading channel consideration and how to add the BB error

References

[1] R4-2514827, WF on NR_AIML_Mob_RRM, OPPO