3GPP TSG-RAN WG4 Meeting #117	R4-2521698

Dallas, US, November 17th – 21st 2025

Agenda Item:	7.2.4

Source:	Ericsson

Title:	On SBFD-SBFD co-existence

Document for:	Approval

# 1	Introduction

In RAN#109, a new RAN4 led Rel-20 WI [1] on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 was approved, which consists of two topics: Enhancement of co-location requirements and SBFD BS to SBFD BS adjacent channel coexistence. In this contribution, we present overview of SBFD BS to SBFD BS adjacent channel coexistence. The following text proposes a simulation approach for Rel-20 SBFD-SBFD co-existence simulation work. The focus is on defining simulation scenarios, parameters, and assumptions. The study should support requirement derivation and reflect realistic deployment cases, including both co-located and non-collocated scenarios.

# 2	Discussion

In Rel-20, the work on SBFD aims for adding two new requirements specifically for the potential deployment scenario of all operators deploying SBFD capable BS on top of Rel-19. To ensure consistency in requirement derivation, simulation work should align with prior assumptions made in Rel-18 and Rel-19. This includes band selection, simulation parameters, and scenario definitions. Due to time constraints and the need to address both co-location work and SBFD-SBFD co-existence work, a down selection of options is necessary.

Scenario: co-location and co-existence

The purpose of operators collaborating in order to deploy SBFD would be significantly undermined if there would be constraints on the operators also having to co-ordinate their BS sites in order to avoid being close to one another. In case there would be three operators within a band, co-ordination would become even more complex, since every operator would need to maintain at least a certain distance from every other operator. Rel-20 work should ensure full flexibility for operator site deployment. This should enable SBFD to function when operators BSs are very close to each other (for example, within 10 m of one another), and also when they are sharing the same tower.

- Both co-located and non-collocated scenarios are particularly important to support real-world deployment practices such as tower sharing and reuse of existing site grids (close to each other).

Include both 0% grid shift (co-located) and 5% grid shift (non-collocated) in the simulation scenarios.

Band

To ensure consistency with derivation of RF requirements for SBFD capable BS in Rel-19, the co-existence study should focus on the same band. The 4 GHz range (e.g., band n79) has been used extensively in Rel-19 work and is suitable for Rel-20 as well.

Use band n79 (4 GHz) as the reference band for initial SBFD-SBFD co-existence simulation.

- Other bands (e.g., n41, n104) can be further discussed.

SBFD carrier bandwidth

Considering limited time allocated to Rel-20 SBFD work, it is not feasible to analyze multiple bandwidths. A single, typical bandwidth should be selected for the simulation. 100 MHz is the largest carrier supported by NR FR1, and this is also aligned with assumptions made in previous studies in Rel-18/19.

- 100 MHz is the largest carrier supported by NR FR1, and this is also aligned with assumptions made in previous studies in Rel-18/19.

Use 100 MHz bandwidth for SBFD in the simulation study.

SBFD subband configuration

As shown in Figure 1, for example, to protect operator 2’s UL subband, suppression of inter-operator interference from operator 1 is needed. This necessitates both a transmitter analogue filter for the whole carrier on operator 1’s side and a receiver analogue filter for the UL subband on operator 2’s side, and vice versa. Therefore, one operator is required to equip two analogue filters (one for transmission and one for reception) that have different passbands and roll-offs per polarization per subarray for inter-operator interference suppression. Besides, to suppress self-interference, yet another subband RX analogue filter, that may be same as or different from the RX filter for suppressing inter-operator interference, is required to reduce the power level from its own transmitter(s)/DL subband(s), as shown in Figure 2. To summarize, for a single SBFD subband configuration, at least two analogue filters are required per polarization per subarray.

- To suppress inter-operator interference, one operator is required to equip both transmitter and receiver analogue filters per polarization per subarray. The two analogue filters have different passbands and roll-offs.

- To suppress self-interference, yet another subband RX analogue filter, that may be same as or different from the RX filter for suppressing inter-operator interference, is required per polarization per subarray for each operator.

- For a single SBFD subband configuration, at least two analogue filters are required per polarization per subarray for each operator.

Figure 1: Illustration of SBFD-SBFD co-existence and analogue filters for inter-operator interference suppression.

Figure 2: Illustration of analogue filter for self-interference suppression.

In our view, DUD will be the only feasible solution when there are more than two operators, although the two outermost operators could have the possibility to do UD/DU, but their TX analogue filter design will be limited by the operator(s) in the middle deploying DUD. Therefore, the most general and configuration-independent approach is to use DUD, regardless of how many operators are in the frequency band, and for SBFD-SBFD adjacent channel co-existence simulations, the configuration should be DUD-DUD.

- DUD will be the only feasible solution when there are more than two operators, although the two outermost operators could have the possibility to do UD/DU, but their TX analogue filter design will be limited by the operator(s) in the middle deploying DUD.

Focus on SBFD subband configuration DUD-DUD case only for Rel-20 co-existence simulations.

Simulation assumptions

In Rel-20, to properly define the additional requirements for both the transmitter and receiver, both in-band blocking and ACIR simulations are essential. The simulation assumptions should, as much as possible, reuse those defined in Rel-18 in order to minimize changes and maintain consistency with the previous SBFD work. However, considering limited time allocated to Rel-20 SBFD work, it is not feasible to analyze all options, and several updates are necessary (as shown below) to better reflect real-world deployment practices and the specific objectives of the Rel-20 study. In particular:

- Grid shift assumptions should be modified to represent practical deployment conditions such as tower sharing and reuse of existing site grids (close to each other).

- Subband configuration, antenna configuration, ACLR, and ACS settings should be revised.

- All other parameters from the Rel-18 simulation assumptions should remain unchanged to ensure continuity.

It is worth noting that in the Rel-18 simulation assumptions, the adopted array configuration was not correct and does not reflect the realistic case, which should be corrected in Rel-20 simulation work and use the IMT baseline instead. Additionally, ACLR and ACS should be removed from the Rel-20 simulation assumptions. The primary objective of the Rel-20 SBFD-SBFD coexistence study is to improve ACIR specifically for SBFD-capable BS. Therefore, in the ACIR simulation, the ACIR value should be swept to identify the level that maintains the target throughput. If possible, a reasonable ACIR is determined, it can be split between ACLR and ACS accordingly.

- In the Rel-18 simulation assumptions, the adopted array configuration was not correct and does not accurately reflect the realistic case, which should be corrected in Rel-20 simulation work and use the IMT baseline instead.

- ACLR and ACS should be removed from the Rel-20 simulation assumptions.

Reuse Rel-18 SBFD simulation assumptions, down select the options, and make necessary updates, e.g., grid shift, subband configuration, antenna configuration, ACLR and ACS.

# E.1	RAN4 co-existence simulation scenarios

Table E.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |

Table E.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 3 | SBFD DUD | SBFD DUD |  |

# E.2	RAN4 co-existence simulation assumptions

## E.2.1	Deployment

Table E.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro (Note 1) |
| --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. |
| Inter-BS distance | 500 m |
| Grid shift | Co-located: 0% Non-collocated: 5% |
| Carrier frequency | 4 GHz |
| Path-loss model | BS-to-UE: UMa + penetration loss, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828 UE-to-UE: UMi (h_BS = 1.5 m) + penetration loss, see TR 38.803 (Note 2). UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. |
| LOS probability | BS-to-UE: UMa, see TR 38.803 BS-to-BS: Option 1: UMa (h_UT = 25 m), see TR 38.803. Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828. UE-to-UE: UMi, see TR 38.803 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value |
| BS height | 25 m |
| UE height | 1.5 m |
| UE distribution | Uniformly UE distribution (Note 1) Baseline: 20% indoor and 80% outdoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model |
| Minimum BS-UE (2D) distance | 35 m |
| Minimum UE-UE (2D) distance | 3 m |
| DL power control | No |
| UL power control | Yes |
| Handover margin | 3 dB |
| BS mechanical down-tilt angle | 6 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes cluster-based UE dropping. See Table E.2.1-3 for cluster-based dropping. Note 2: For UE-UE penetration loss, the following is used to generate d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O-to-I building penetration loss: - d2D-in =Uniform(0,min⁡{25m,d2D2}) |  |

Table E.2.1-3: Cluster-based UE dropping parameters

| Parameter | Urban Hotspot |
| --- | --- |
| Cluster number per macro cell | 1 (Note 1) |
| Cluster area | Circular area with 25 m radius |
| UE distribution | UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor |
| Indoor and outdoor ratio | 80% indoor and 20% outdoor (Note 2) |
| Minimum (2D) distance | UE-UE: 1 m Cluster center – cluster center: 50 m Macro BS – cluster center: 60 m (Note 3) |
| UE dropping | Evenly random dropping into the cluster with minimum UE-UE distance limitation |
| Note 1: Cells fully or partly overlapping between the 2 networks share the same cluster. Note 2: O-to-I penetration losses are not calculated for pairs of indoor UEs deployed in the same cluster area. For pairs of indoor UEs in different cluster areas, a single penetration loss component is calculated. Note 3: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro BS to cluster centre should be respected also for BSs belonging to the other operator |  |

## E.2.2	Traffic

Table E.2.2-1: FR1 network traffic parameters

| Parameter | Urban Macro |
| --- | --- |
| Carrier bandwidth | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For SBFD {DUD}: 40 MHz |
| Scheduled channel bandwidth per UE (UL) | For SBFD {DUD}: 20 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as TDD |
| Traffic load | Full buffer |

## E.2.3	BS RF characteristics

Table E.2.3-1: FR1 BS RF parameters

| Parameter | Urban Macro (Wide Area) |
| --- | --- |
| Transmitter power (Total conducted power) | Option 1 (baseline): SBFD config 2: 49 dBm Option 2: SBFD config 2: 53 dBm |
| Antenna configuration | SBFD config 2 (baseline): (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,2.1)λ, (q3dB,j3dB)=(65,90) o SLAv=30 dB, Am=30 dB, GE,max=6.4 dBi, Msub=3, dv,sub= 0.7l, subtilt= 3 deg |
| Receiver noise figure model parameters | Baseline: P1=-43 dBm P2=-25 dBm F1=5 dB F2=14 dB Optional (Enhanced NF model): P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) |

# Conclusion

This paper recommends a simulation approach for SBFD-SBFD adjacent channel co-existence in Rel-20. By reusing Rel-18 and Rel-19 work and limiting the scope to the most representative configurations (DUD-DUD, 100 MHz, n79), RAN4 can efficiently proceed with requirement derivation and enable support for practical deployments, including tower sharing and reuse of site grids.

Observation 1	Both co-located and non-collocated scenarios are particularly important to support real-world deployment practices such as tower sharing and reuse of existing site grids (close to each other).

Observation 2	Other bands (e.g., n41, n104) can be further discussed.

Observation 3	100 MHz is the largest carrier supported by NR FR1, and this is also aligned with assumptions made in previous studies in Rel-18/19.

Observation 4	To suppress inter-operator interference, one operator is required to equip both transmitter and receiver analogue filters per polarization per subarray. The two analogue filters have different passbands and roll-offs.

Observation 5	To suppress self-interference, yet another subband RX analogue filter, that may be same as or different from the RX filter for suppressing inter-operator interference, is required per polarization per subarray for each operator.

Observation 6	For a single SBFD subband configuration, at least two analogue filters are required per polarization per subarray for each operator.

Observation 7	DUD will be the only feasible solution when there are more than two operators, although the two outermost operators could have the possibility to do UD/DU, but their TX analogue filter design will be limited by the operator(s) in the middle deploying DUD.

Observation 8	In the Rel-18 simulation assumptions, the adopted array configuration was not correct and does not accurately reflect the realistic case, which should be corrected in Rel-20 simulation work and use the IMT baseline instead.

Observation 9	ACLR and ACS should be removed from the Rel-20 simulation assumptions.

Proposal 1	Include both 0% grid shift (co-located) and 5% grid shift (non-collocated) in the simulation scenarios.

Proposal 2	Use band n79 (4 GHz) as the reference band for initial SBFD-SBFD co-existence simulation.

Proposal 3	Use 100 MHz bandwidth for SBFD in the simulation study.

Proposal 4	Focus on SBFD subband configuration DUD-DUD case only for Rel-20 co-existence simulations.

Proposal 5	Reuse Rel-18 SBFD simulation assumptions, down select the options, and make necessary updates, e.g., grid shift, subband configuration, antenna configuration, ACLR and ACS.

# References

RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”