3GPP TSG RAN4 Meeting #117							R4-2521747

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	Handling NTN and other verticals in RAN4 specifications

Agenda item:	8.12.2.4

Document for:	Approval

Introduction

In the current study item draft the following topics are listed under 6GR.

RP-250858.zip (New SID: Study on 6G Radio) RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles

Furthermore, at RAN4#116 a plan for the 6G study in RAN4 was presented by the RAN4 chair as captured in [1]. One of the topics was “Operation Efficiency” with a scope listed as:

- CR implementation improvements

- Specification improvements and modernization

- New tools for band combinations

- Other aspects

In this contribution we discuss the implementation of RAN4 specifications regarding aspects that go beyond the implementation of one single document. This paper aims at uniformize and discuss RAN4 approach to capture NTN and other verticals requirements in its specifications.

Discussion

Incorporating NTN into RAN4 specification

In Release 17, NTN was introduced in 3GPP specification as a consequence of the study and work initiated in Rel-16. This feature was not originally part of the 5G NR specifications from Release 15/16 and therefore was not a native feature (or a day-1 feature) in 3GPP specifications. The modifications needed to support NTN operation were not limited to one specific operation (e.g. timing, spectrum emissions, or demodulation). From RAN4 point of view, the NTN support required modifications that were pervasive across several parts of the specification. Sometimes small, sometimes big modifications were made to adjust the system to support the NTN systems. Because of that, in Release 17, RAN4 had to make decisions on how to accommodate the new feature into the specification, and the results were inconsistent across the different RAN4 documents, as evidenced by the list below:

- UE transmission and reception requirements (38.101 family): a new, segmented document, was created under the “-5” suffix. For the 38.101 family, this decision was in line with previous RAN4 segmentation of the document which had already split FR1, FR2 and interworking between FR1 and FR2 in their own segmented documents.

- RRM Requirements (38.133): the NTN requirements were introduced following the same structure of the RRM document, within the same document, using “mirror clauses” that ended in suffix “C”. Later in Rel-19, RedCap over NTN was introduced, and instead of re-using the suffix “C” with new clauses, new suffixes were chosen (“E” and “F”)

- BS Radio Transmission and Reception (38.104) and conformance testing (TS 38.141): new documents were created to detach completely the Satellite Access Node (SAN) from the base station documentation. The separation aims at make it clear that a SAN has some resemblance to a BS, but specificities that require it to be treated differently. The documents are under specification numbering TS 38.108 and TS 38.181

Pros and Cons of “separated documents”

As discussed above, in RF and demodulation, the solution adopted to incorporate NTN specification was to create mirror documents to the pre-existing ones, but capturing NTN specific requirements. For UE RF this was done within the same family of specification numbering (TS 38.101) while for BS RF wit was done using mirror documents that followed a similar structure (TS 38.104/TS 38.108 and TS 38.141/TS38.181).

This approach has some advantages. For example, when handling new bands that ought to be supported only by NTN UEs (or conversely when handling bands that were not supported by NTN operation), the separation of the documents made it clear which band numbers were to be supported for NTN or non-NTN operation. Besides, when Ku band was later introduced in Rel-19, and new numerologies were introduced in a previously unused frequency range (above 7 and below 20 GHz), the exceptional new frequency channels which supported multiple numerologies did not have any impact on legacy UE behavior.

On the other hand, separated documents limit how much of the specification can be “re-used”, as features and requirements in one of the documents are not automatically transferred to the other documents, which means that incoming features and enhancements in future Releases for the general requirements are not captured into NTN requirements, which tends to have greater impact over RRM requirements. Besides, it is not always straightforward clear that the general requirements are not applicable to NTN UEs, as no sentences of exceptionality are usually provided in the general documents.

Pros and Cons of new set of clauses

The use or not of suffixes are discussed in the next Section of this document. In this subsection, we discuss the approach used in the RRM document and its pros and cons. On the bright side, this maintains all requirements within a single document, and it is easy to capture the “exceptionality” of the general requirements by distinguishing which requirements are applicable to NTN and which are not.

On the other hand, it still provide significant maintenance issues and sometimes isolate NTN from new features and enhancements. This has been observed in the RRM document. Since Rel-17, new mobility enhancements have been introduced, but as they are added to the general requirements, they are seldom replicated into NTN clauses. Sometimes, this results on re-work. For example, in Rel-18, less than 5 MHz support was introduced for terrestrial UEs. In Rel-19, the work was replicated for NTN, which mostly re-used and relied upon the work done in Rel-18. RAN4 TUs and workload was dedicated to just replicate an existing feature with some small modifications.

Another problem is that this tends to creates large documents. Most of the NTN specification is rewritten from original (terrestrial) requirements, with some modifications and values or parameters and tables re-written to adapt to NTN.

Pros and Cons of adding NTN in the same set of existing clauses

Adding NTN requirements under the same general clauses to fully integrate NTN is an approach that was still not tried in RAN4, but it has been present in RAN1 and RAN2. Although this approach facilitates maintenance, as only one set of clauses affects all UEs, it makes the specification effort much more challenging, as all exceptions and specific requirements need to be captured at once. The chance to have a small modification introduced by a new feature to have unintended consequences over the NTN specifications (and vice versa) might be non-negligible.

Summary of options

The Sections above discussed how to handle NTN specifications in RAN4 document. Based on the discussion presented in this document we make the following observations and proposals:

The pros and cons of the different approaches to handle NTN specification are provided in Table 1.

Table 1. Summary of Pros and Cons of methods to capture NTN specification

|  | Evaluation of Options to handle NTN specifications in RAN4 |  |  |  |
| --- | --- | --- | --- | --- |
| Solution: | Separated Documents (same family numbering) | Separated documents (different numbering) | New set of “mirror” clauses (e.g. using suffix letters) | Under the same clauses and documents as TN requirements |
| Used in 5G? | In UE RF specification (family 38.101-X) | In BS RF documents (38.181 and 38.108) | In RRM (38.133) | Not used in RAN4, but used in RAN1 and RAN2. |
| Pros | - Easy to isolate specific work (e.g. dedicated bands, spectrum range) - Specs are “shorter” and easier to read invididually | Same as for separated documents using the same family numbering. | - Everything can be found within one document - Maintenance issues can be propagated within only one CR (reducing maintenance issues) - Easier to reference requirements | - Facilitate maintenance and propagation of enhancements to NTN requirements |
| Cons | - New features and enhancements do not propagate automatically to NTN clauses (re-work) - Not always clear that the general requirements are not applicable to NTN | - Same as for separated documents using the same family numbering. - Makes it harder to find the right specs | - Specs can become too long and difficult to handle - New features and enhancements do not propagate automatically to NTN clauses (re-work) | - Make specification work, especially in the first releases, more challenging - Easier to create unintended consequences by adding new features to general requirements that were not initially developed to NTN |

All methods above have their pros and cons. RAN4 has to carefully evaluate considering all specification documents, which solution is best applicable to NTN.

RAN4 shall evaluate, considering all specification documents, whether NTN requirements shall remain in the same document as TN requirements or whether they should be separated.

If RAN4 decides to specify TN and NTN in the same documents, RAN4 shall decide whether the standard approach is separating them by subclauses or include both in the same clauses.

RAN4 to decide whether the solution needs to be defined in a case-by-case approach or whether the same solution shall be applied to all specifications.

Handling “Verticals” and “suffixes” in the specification

In several cases, RAN4 has adopted a solution where clauses are “mirrored” under a suffix letter to indicate that those set of clauses are applicable to a single feature or a “vertical” solution. For example, in TS 38.101-1 and TS 38.133 those are the use cases where the suffix method was

Table 2: Copy of Table 2.3.2.2-1 in TS 38.101-1 (suffix-list)

| Clause suffix | Suffix use in TS 38.101-1 | Suffix use in TS 38.133 |
| --- | --- | --- |
| None | Single Carrier | General requirements (Single and multi carrier) |
| A | Carrier Aggregation (CA) | CCA (Clear Channel Assessment), RedCap |
| B | Dual-Connectivity (DC) | (e)Redcap, CCA (Clear Channel Assessment) |
| C | Supplementary Uplink (SUL) | NTN |
| D | UL MIMO | ATG, Redcap over NTN, Redcap |
| E | V2X | Redcap over NTN, ATG |
| F | Shared spectrum channel access | Redcap over NTN |
| G | Tx Diversity (TxD) |  |
| H | Carrier Aggregation (CA) with UL MIMO |  |
| I | (e)RedCap |  |
| J | ATG |  |
| K | Aerial UE (UAV) |  |
| L | Carrier Aggregation (CA) with Tx Diversity |  |
| M | LP-WUS/WUR |  |

It is worth noting that the Table above indicates several issues:

- Inconsistency across specifications: a reader of RAN4 specifications that is looking for ATG clauses, shall look for clauses under suffix J in TS 38.101-1 and for clauses with suffix D (in most cases) at TS 38.133. A similar problem is observed for RedCap. This lack of consistency makes RAN4 specification more confusing.

- Inconsistency within TS 38.133: While NTN specification thrived to maintain a single letter as suffix for the entire specification, other features (CCA, RedCap, RedCap over NTN and ATG) use different letters at different clauses to identify the feature specific requirements. The RRM specification is already quite long and this lack of consistency makes it harder to find the feature specific requirements with efficient scrolling and scanning.

RAN4 to investigate alternatives to the usage of suffix to identify feature specific or vertical specific requirements

If the suffix-method is adopted in 6G, RAN4 to adopt a consistent approach across all specifications and within the same specification (same letter always identifying the same feature).

Although in certain cases above (MIMO, CA, SUL) the suffixes were added to capture the needs of specific features, in other cases, they were used for identifying requirements for specific vertical applications. RAN4 needs to discuss whether vertical specific shall be maintained in the same document as the general requirements and what is the most efficient method to refer to them.

Discuss whether “vertical-specific” requirements shall be maintained in the same document as the general requirements or in a different document within the same family.

Conclusion

In this contribution we present our view on handling the specification of vertical solutions in RAN4 specifications. The following observations have been made:

Observation 1: The pros and cons of the different approaches to handle NTN specification are provided in Table 1.

Proposal 1: RAN4 shall evaluate, considering all specification documents, whether NTN requirements shall remain in the same document as TN requirements or whether they should be separated.

Proposal 2: If RAN4 decides to specify TN and NTN in the same documents, RAN4 shall decide whether the standard approach is separating them by subclauses or include both in the same clauses.

Proposal 3: RAN4 to decide whether the solution needs to be defined in a case-by-case approach or whether the same solution shall be applied to all specifications.

Proposal 4: RAN4 to investigate alternatives to the usage of suffix to identify feature specific or vertical specific requirements

Proposal 5: If the suffix-method is adopted in 6G, RAN4 to adopt a consistent approach across all specifications and within the same specification (same letter always identifying the same feature).

Proposal 6: Discuss whether “vertical-specific” requirements shall be maintained in the same document as the general requirements or in a different document within the same family.

- References

RP-250858, New SID: Study on 6G Radio

SP-250802, Study on Modernization of Specification Format and Procedures for 6G, Nokia et. al.

TR 21.802, Study on modernization of specification format and procedures for 6G

R4-2513056, LS on Study on Modernization of Specification Format and Procedures for 6G, TSG SA

R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT

RP-252842, Progress update and next steps for the Band Combination Database, Nokia