3GPP TSG-RAN WG4 Meeting#117                                R4-2521750

Dallas, USA, 17th – 21st, Nov. 2025

Agenda item:	8.4.4

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6GR U6GHz conformance testing

Document for:	Approval

Introduction

In the RAN#108 meeting, a new WID on Study on 6G Radio [1] was approved. In the last RAN4 meeting, there were some initial discussions on 6G NR BS RF requirement and agreements were captured in WF [8]. In this contribution, we would like to share some further understandings on 6GR U6GHz conformance testing.

- Objective: - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - RAN4#1116bis agreement: Issue 1-1-1: Starting point Agreement: 5G BS RF requirements should be considered as starting point when defining the 6G BS RF requirements. Issue 1-1-2: Requirements to be re-assessed Way Forward: For next meeting, companies are encouraged to evaluate how 5G BS RF requirements should be considered for 6G and provide their view using the table in Annex A as example. Note: The goal is to identify: - 5G requirements that might also be applicable to 6G. - 5G requirements which will need to be re-evaluated based on 6G system parameters but would not need a further study. - 5G requirements which should be further studied in the scope of the 6G SI, a prioritization might be made in next meeting. Issue 1-2-1: BS type 1-H enhancement Agreement: BS Hybrid beamforming type of architecture shall be studied. Requirements for cmWave bands Issue 1-3-3: Rx Requirements Agreement: For above 7 GHz, further study how BS Rx requirements should be specified, using FR2 approach or not, including if blocking should be specified based on FR2 methodology.

Discussion

- 2.1. Enhancement conformance testing for U6GHz EEIRP mask

- In the Rel-19, U6GHz EEIRP mask was introduced for NR BS and NCR with the following test beam distributed equally within the declared beam steer range as shown in Figure 2.1-1.

- EEIRPθHLi , θHHi=(θHHi−θHLi)×π180°MNsinθHHi−sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,

Table 2.1-1. The illustration of test beam set for U6GHz BS EEIRP mask

- In the Rel-19, the test beams is equally assumed in the specification based on the assumptions of target serving UE also equally distributed within the geographical coverage regions, then the proposed test beam pattern could well mimic the actual BS operation mode. However this might be not always applicable considering the emerging new service (e.g. sensing service or UAV service in the near future to support the low altitude economy, eVTOL, express Delivery based UAV, other sensing use case) as shown in Figure 2.1-3.

Table 2.1-2. The illustration of the operation beams equally distributed across different slots

- As shown in Figure 2.1-3, operation beam in red color in slot 2 in certain periodicity is intended to provide the service for sensing or UAV service, this is obvious that the weighting factor and probability of such kind of sensing/UAV beams is expected to be more sparse and different from the other communication beams. Therefore, for the conformance testing of U6GHz EEIRP mask in 6G phase, we propose to consider the unequally distributed test beams across the whole coverage regions (e.g. 6GR BS need to provide the sensing or UAV service with periodic beams steering upwards) in addition to the equally distributed test beams in Rel-19.

Table 2.1-3. The illustration of the operation beams unequally distributed across different slots (e.g. to provide the sensing or UAV service with periodic beams steering upwards)

- Proposal 1: for the conformance testing of U6GHz EEIRP mask in 6G, propose to consider the unequally distributed test beams across the whole coverage regions from the time domain (e.g. 6GR BS need to provide the sensing or UAV service with periodic beams steering upwards) in addition to the equally distributed test beams in Rel-19.

Conclusions

In this contribution, we shared some initial views on the 6GR U6GHz conformance testing and proposals are made as following:

- Proposal 1: for the conformance testing of U6GHz EEIRP mask in 6G, propose to consider the unequally distributed test beams across the whole coverage regions from the time domain (e.g. 6GR BS need to provide the sensing or UAV service with periodic beams steering upwards) in addition to the equally distributed test beams in Rel-19.

References

- RP-251881,New SID: Study on 6G Radio, Approved.

- R4-2511000,Further consideration on BS RF for Rel-19 LP-WUS, Noted

- RP-252952,New WID: UE RF enhancements for NR FR1, Phase 5, Approved.

- RP-25290, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Approved

- RP-252578,Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, Approved.

- TR 38.922

- R4-2316538c,Further discussion on RF requirement impacts from NES perspective, ZTE corporation

- R4-2514638, WF on 6G BS RF and co-existence, Ericsson, approved.