3GPP TSG-RAN WG4 Meeting#117                                R4-2521757

Dallas, USA, 17-21 Nov, 2025

Agenda item:	6.14.2

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on conformance testing for Rel-19 Ambient-IoT BS and CW

Document for:	Approval

Introduction

In the last RAN4 meeting, the core requirement for A-IoT BS and CW is declared as completed. In this contribution, we would like to share our further understanding on the conformance testing for A-IoT BS and CW node.

- ssue 2-5: A-IoT test configuration - Tentative Agreement: - Use the single carrier operation for A-IoT BS R2D conformance testing - FFS whether and how to test FDM operation for different devices with different R under same CW frequency operation case if needed - FFS whether use the single or multi carrier operation for A-IoT BS D2R conformance testing

Discussion

## 2.1. Test configuration

Similar as the standardization of other new RAT, RAN4 need to discuss the test configuration for A-IoT BS. In the last RAN4 meeting, there are two open issues left for further discussion. Regarding the FDM operation between different device in the uplink direction, the current REFSENS requirement is defined based on R=1 without any repetition and frequency shift. In other words, there are no core requirements for supporting such kind of FDM operation, therefore we propose to focus on R equal to 1 case.

Proposal 1: For conformance testing of uplink D2R, focus on the R equal to 1 without FDM between different devices.

In addition, there are single carrier or multiple carriers case for A-IoT R2D conformance testing. From our understanding, it is not feasible from device perspective due to the wide reception bandwidth from device side. In other words, the signal from different carriers will interference each other when the device conduct the envelope comparison/estimation.

Proposal 2: For test configuration for A-IoT R2D conformance testing, propose to consider the single carrier operation only.

Conclusions

In this contribution, we want to share some further views on Rel-19 A-IoT/CW conformance testing and proposals/observations are made as following:

Proposal 1: For conformance testing of uplink D2R, focus on the R equal to 1 without FDM between different devices.

Proposal 2: For test configuration for A-IoT R2D conformance testing, propose to consider the single carrier operation only.

References

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- R2-2506559, A-IoT MAC layer specification

- R4-2515058, Way Forward for [116bis][314] A-IoT_BS_CW, approved.