3GPP TSG-RAN WG4 Meeting #117	R4-2521770

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Simulation results for NR NTN Ku band demodulation

Agenda item:	5.10.3

Document for:	Information

# Simulation

## 2.1	PDSCH

Common parameters can be found in Table 11.2.2-1 in TS 38.101-5. Other parameters can be found in Table 2.1-1 below.

Table 2.1-1 Test parameters for PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 4 with feedback disabled |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 15 kHz SCS: 10 30 kHz SCS: 20 |
| cellSpecificKoffset-r17 (Note 1) |  | Slots/15kHz | 15 kHz SCS: 8 30 kHz SCS: 16 |
| Maximum number of HARQ transmission |  |  | disabled for all HARQ processes |
| Note 1: The number of slots between PDSCH and corresponding HARQ-ACK information contains the number of slots specified by cellSpecificKoffset-r17. |  |  |  |

Table 2.1-2 Minimum performance for Rank 1

| Bandwidth (MHz) / Subcarrier spacing (kHz) | PDSCH PRB number | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 / 15 | 270 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | NA |
| 50 / 30 | 133 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.2 |
| Note1: The Maximum throughput is based upon HARQ feedback disabled. |  |  |  |  |  |  |

Figure 2.1-1 Performance comparison among different channel models

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, Samsung