3GPP TSG-RAN WG4 Meeting #117	R4-2521773

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on 6G Demodulation Requirements

Agenda item:	8.7

Document for:	Discussion

# Introduction

In RAN4#116bis, the 6G Demod start the discussion on the scope and companies interested topics. Regarding the wide area of issues and limited session time, only one agreement is achieved on general test frame principle [1].

General performance requirement aspects

- Agreement

  - In 6G SI, RAN4 will study how to strive to develop a demod/test performance requirement framework for the minimum performance to better reflect real field conditions than 5G, if necessary, by taking into consideration of the requirement/test coverage/alignmentablity and the test feasibility, complexity and cost/resources.

In this contribution, more views are delivered for open issues for 6G demodulation.

# Discussion

## 2.1	General test framework

### 2.1.1 	Test definition

It is agreed in RAN4#116bis that new test framework should be studied for 6G demodulation to reflect real field conditions than 5G. But how to define “field condition” is not clarified here.

As our understanding, different operators would have different deployments, and their network conditions would be quite different. Furthermore, the real network condition would be a very wide range including numberless of situations with different possibility. The minimum demodulation requirements are not expected to check every corner case in all field network deployments. Companies should identify typical product configuration and propagation conditions first which occupies majority market and happens with very high probability, and then further choose the worst case from them according to demodulation feature algorithm. It is important for balancing the requirement coverage and test effort.

Based on this principle, only one deployment is normally chosen according to agreed RAN1 and/or RAN4 RF simulation assumptions, and then a typical channel model would be considered to align field observations on time, frequency and spatial fading configurations. To have a robust requirement, some configurations, such as Doppler, delay etc., might need some trail evaluations to have better values which can cover most of conditions. Then transmission signal and receiver configurations will be considered.  It is also possible adding dynamic configurations for some special cases, such as channel model drifting for UE moving etc., to have better estimation the real network environment at a certain level. Of course, the corresponding benefit and complexity should be investigated.

Proposal 1	RAN4 to further clarify the definition of “field condition” for minimum demodulation requirement regarding typical deployments, robust receiver algorithm verification and a certain level of dynamic environment.

RAN4 demod study timeline

- Proposals

  - Option 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

  - Option 2: RAN4 closely follow RAN1's discussion progress on other parameters related to 6G physical layer structure. Once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study.

  - Option 3: Postpone RAN4 discussion on demodulation requirements related to physical layer channel and procedure design until sufficient progress reached in RAN1 i.e., no early than Q2’26.

  - Option 4: 5GA Topics that were studied and proven useful should be introduced from day 1 in 6G, and RAN4 needs start discussions on corresponding test specifications to enable early adoption.

Proposals from companies mainly focus on the relationship between RAN1 and RAN4 demodulation timelines and show concerns on the overlapped period regarding to fixed release period. On one hand, RAN4 starting early could have more time to settle tricky issues but would face no completion of RAN1 discussion. On the other hand, the situation will be reversed, which would lead to only simplified demodulation requirements being introduced. To summarize the proposals for this issue, an objective effort estimation on RAN1 should be addressed early and coordinated with RAN4. It could be better to have a certain flexibility on the timeline of RAN4 demodulation to secure the feasible connection between RAN1 and RAN4 and also give enough time to RAN4 to derive good requirements according to estimation. To achieve this, more efficient and frequent coordination between RAN1 and RAN4 is necessary.

Proposal 2 	The higher efficient inter-group coordination and interactions between RAN1 and RAN4 is needed to have a more reasonable timeline for both groups.

Waveform and modulation study

- Proposals

  - Option 1: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

  - Option 2: RAN4 6G Demodulation study should cover following modulation schemes at least

    - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

    - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

    - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

  - Option 3: Develop testable specifications for waveform, frame structure, channel coding, and modulation as defined by RAN1, ensuring that these can be practically implemented and validated in real products.

SCS

- Proposals

  - Option 1: RAN4 6G Demodulation study could start with following SCS options.

    - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

    - For around 7GHz, 30kHz, 60kHz

    - For around 15GHz, 60kHz

    - For between 24.25GHz - 52.6GHz, 120kHz

The available proposals are based on 5G specifications, but it might be better to have a discussion on detailed configurations till RAN1 and RAN4 have conclusions on 6G parameters.

Proposal 3 	Postpone the parameter discussion for 6G Demod until RAN1 and RAN4 have conclusion on 6G system parameters.

ISAC study

- Proposals

  - Option 1: Study the demodulation for ISAC for 6G.

The sensing part of ISAC might not be quite relevant to demodulation since it mainly depends on the measurement, and the communication part of ISAC would be similar as normal scenario. According to current study, two parts are separately scheduled in time domain. In that case, the strong interference impact in ISAC might not be typical. However, it depends on RAN1 conclusion on frame structure and interference study. We propose to lower the priority of ISAC demodulation requirement study in 6G until RAN1 have conclusion on frame structure.

Proposal 4	Pending the discussion of ISAC in 6G Demod until RAN1 have conclusion on frame structure and interference study.

Conducted and radiated testing

- Proposals

  - Option 1: Conducted test method can be considered for FR1 frequency range, and OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements.

  - Option 2: Define test methodologies for FR3, including whether measurements are conducted or OTA.

The conducted or radiated test methodology should be discussed and decided by RF session. Although there is no sign that RF would change 5G test framework for FR1 and FR2, demodulation should follow RF conclusion on test type and then discuss corresponding demodulation test method.

Proposal 5	Follow RF conclusion on test type per feature per frequency range for demodulation conformance tests.

### 2.1.2 	Testability of broadcast channels

Broadcast and feedback-less channels/signals testing

- Proposals

  - Option 1: Study whether broadcast and feedback-less channels/signals can be assumed testable. RAN4 to check whether to recommend to RAN5 to define needed test solutions.

RAN4 UE demodulation requirements assume to do the conformance testing with the commercial chipset firmware by connecting the antenna port or OTA. This can verify the UE behaver in the commercial network and also avoid implementing the test mode in UE. For this purpose, RAN4 defines the demodulation test metrics by measuring feedback from the DUT (UE), e.g., the number of acknowledgments in HARQ-ACK, or CSI quantities. Although it does not require UE to implement the test mode, RAN4 cannot define demodulation requirements for the procedures not requiting feedback, for example, IDLE/INACTIVE mode procedures.

One exception is the PBCH demodulation requirements. Although UE does not send any feedback to gNB when UE decodes PBCH, RAN4 has defined the PBCH detection success rate requirements from 3G. One purpose of this requirement is to use it to derive MIB detection delay requirements in RRM.

There are some UE procedures we are interested in defining the requirements from the network performance point of view. One example is the paging reception performance, i.e., PDCCH demodulation in IDLE/INACTIVE. Since gNB can configure the coding rate of PDCCH, it is important to know a proper coding rate to avoid allocating unnecessarily large radio resources (e.g., larger AL in NR PDCCH).

We understand that implementation of test mode is an additional development cost, however RAN4 should consider defining the UE demodulation performance requirements even in IDLE/INACTIVE mode if it is justified to define the demodulation performance requirements..

Proposal 6	Consider defining the demodulation performance requirements for UE not in CONNECTED status even if it does not send the feedback from the UE, if it is justified to define the performance requirements.

## 2.2	Channel model

Channel type

- Proposals

  - Option 1: Clarify use of TDL and SCM models for 6G

    - Option 1A: Use rCDL baseline for MIMO.

    - Option 1B: Use CDL/rCDL as baseline for 6G.

    - Option 1C: Maintain TDL and CDL.

    - Option 1D: Continue TDL for simplicity.

    - Option 1E: Include xTDL.

    - Option 1F: Define default CDL propagation models.

    - Option 1G: Adoption of SCM should be justified for each test purpose.

    - Option 1H: Study practical MIMO correlation matrices for TDL.

    - Option 1I: Use rCDL baseline for MIMO and a limited number of TDL requirements for MIMO features with single layer transmission.

    - Option 1J: select one channel model (either TDL or CDL) for one specific feature.

      - Option 1Ja: The criteria of selection should be clarified and applied for all features.

The CDL-based rCDL-C model was derived in Rel-19 SI and will be further applied for Rel-20 demodulation requirements. The purpose of derived such kind of spatial channel model is to have more realistic channel model and then have more realistic product verification. Once the derived CDL-based model has been proved feasible for a certain feature requirement definition from performance and test perspective, we don’t see a reason why we need TDL-based model as baseline. The CDL-based model for baseline should be prioritized.

For practical correlation matrix study for TDL model, it might take similar effort as a new CDL model derivation. The antenna model, configuration and angular spread assumption should also be discussed and analyzed. The benefit is quite limited.

An exceptional condition would be there is no available CDL-based model and the feature is not sensitive to spatial fading (e.g., low rank transmission in a new deployment scenario), TDL-based model could be considered here to save effort for new model derivation.

In summary, the CDL-based channel model should be considered as baseline for 6G demodulation requirements. TDL models used in 5G could be maintained in 6G for some low rank transmission cases to save effort in case of no available CDL model can be applied.

Proposal 7	the CDL-based channel model should be considered as baseline for 6G demodulation requirements. TDL models used in 5G could be maintained in 6G for some low rank transmission cases to save effort in case of no available CDL model can be applied.

Specialized propagation channels

- Proposals

  - Option 1: Evaluate candidate channel model for DL and UL considering new use cases including AI, ISAC, NTN, HST.

  - Option 2: Initiate a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

For AI, it could be discussed in whole AI test framework, such as data driven tests or dynamic channel driven tests.

For ISAC, as we discussed above, it might not need new channel model for demodulation. As for sensing test if introduced, it might need consider new type of channel model based on RAN1 study, but it might not be in demodulation scope.

For NTN, CDL-based models, such as NTN-CDL-A/B/C/D, have been introduced by RAN1 which RAN4 could derive corresponding CDL channel model with proper randomness reduction for NTN scenario demodulation. No need to start with CDL-D.  But current NTN deployment is mainly low rank scenario and very complicated timing/doppler drifting scheme, the benefit of new CDL based channel model for NTN demodulation should be clarified at first.

HST deployment is mainly a LOS channel typical scenario, and current requirements are defined with AWGN channel with one layer transmission between a RRH and a HST receiver. The benefit of deriving new CDL models seems limited. The key issue for HST could be the timing, Doppler and power drift during the RRH and HST connection. In 5G, there are quite complicated drifting models introduced for DL especially. ATG scenario would be similar as HST.

In 6G, NTN and HST could reuse timing/Doppler/power drifting model in 5G, and the benefit of deriving new channel model might need more clarifications. For ATG scenario, it is possible to consider similar timing/Doppler drifting model definition for DL demodulation requirements.

Proposal 8	Benefits of new CDL channel model derivation for ISAC, NTN, HST and ATG should be clarified first if there are specific concerns.

Proposal 9	Reuse existing timing/Doppler/power drifting model for NTN and HST scenario in 6G.

Frequency related aspects of channel model

- Proposals

  - Option 1: Study new frequency range.

    - Option1A: Study new frequency ranges of 6G

      - Derive CDL for 7-15 GHz.

    - Option1B: Study FR2.

    - Option 1C: Evaluate necessity and study spatial channel model for other frequency ranges in 6GR.

  - Option 2: Study if channel model is agnostic to different carrier frequencies or separate models are needed.

  - Option 3: Discuss the number of antennas required for different frequency ranges.

The new frequency range, such as 7-15GHz, is defined for new deployment scenarios. RAN1 also have studied the corresponding propagation model for it. It is straightforward to have new channel models if new demodulation requirements will be introduced.

For FR2, as we pointed out in the previous meeting, the rank is limited to 1 or 2 by the OTA test setup which would lead to less sensitive to spatial fading. In 38.827, CDL model is derived for FR2 OTA MIMO UE RF test, but the benefit for demodulation should be further clarified.

The frequency band agnostic of CDL model issue was discussed in Rel-20 and companies agree that taking fixed Doppler value (Hz) for a model and variant the UE velocity vector according to center frequency. It is our understanding that the demodulation requirement should be frequency band agnostic and the current solution in Rel-20 can be followed.

For some special cases, such as high Doppler scenario, different deployment bands and corresponding SCS have limitations to handle same Doppler shift. In that case, different models could be derived accordingly. But it would be better to decouple antenna configurations from frequency bands within same frequency range. On the other hand, it is possible to have multiple test cases with multiple channel models to have a wider coverage, such as different combinations of antenna configuration with layers and MCS in 5G normal PDSCH/PUSCH demodulation requirements.

Proposal 10	Derive new channel models for new frequency range, such as 7 – 15GHz.

Proposal 11	Clarify the benefit of derive CDL model for FR2 demodulation first regarding the rank limitation of OTA tests.

Rel-20 NR SCM WI have agreement on band agnostic solution for CDL models.

Proposal 12	Take similar approach as in Rel-20 to handle CDL model band agnostic issue. Multiple CDL models can be derived for some special cases to have more feasible or wider coverage requirements.

Uplink aspects of channel model

- Proposals

  - Option 1: Study UL CDL for BS variants.

    - Option 1a: Study UL CDL for BS variants if key issues in DL CDL study are settled.

As we discussed in previous meeting, the different BS would have quite different antenna configurations and corresponding channel would be very different. It is not expected to take a channel model to test a BS will never face such kind of channel. Currently, there are many open issues even in DL CDL model investigation, so it should be better to consider UL model derivation later as Option 1a proposed.

Proposal 13	Take Option 1a for UL channel model issue.

Channel model alignment

- Proposals

  - Option 1: Study how to ensure the alignment of CDL implementation. Channel properties such as PSD, time correlation coefficient and frequency correlation coefficient may need to be aligned apart from SNR points alignment.

During Rel-19 SI on SCM, we found it beneficial to check channel properties in addition to performance which could help companies to check the alignment level of channel implementation. Regarding new CDL models would be derived in 6G, such as new UE antenna model or new frequency range, we think it is necessary to have such alignment. Once the alignment of channel properties and performance can be achieved, the model implementation procedure could be fixed if there is no special reason. There is no need to re-do this analysis every time after.

Proposal 14	Get alignment on channel properties and performance if new CDL models with major changes are derived in 6G.

PMI bias

- Proposals

  - Option 1: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

How to avoid PMI bias is also discussed in Rel-20 NR demodulation enhancement phase 6. But it is not decided whether to capture corresponding improvement study in Rel-20 WI scope. If this issue is agreed not to be considered in Rel-20 or it is considered but can’t be solved in Rel-20 period, it should continue studied in 6GR.

Proposal 15	Consider PMI bias issue in 6GR if it can’t be solved in 5G-A in Rel-20 period.

## 2.3 	Receiver assumptions

### 2.3.1	UE baseline receiver

Receiver assumption for UE

- Proposals

  - Option 1: MMSE-IRC as a baseline receiver.

  - Option 2: MMSE-IRC and R-ML as baseline receivers.

    - CMCC: with the prerequisite that the receiver is transparent to the network and does not require any PHY layer modification and additional assistance information.

  - Option 3: Cover advanced receivers (R-ML, soft-IC)

    - CT: Study the required information for advanced Rec for MU-MIMO.

  - Option 4: Study baseline and simplified structures.

  - Option 5: Study widely linear MMSE-IRC.

As for the UE baseline receiver, the discussion mainly focused on whether 6G continues to assume the IRC linear receiver as baseline, or introduce non-linear receiver (e.g. R-ML) as baseline or optional benchmarks?

During the last meeting’s discussion, a clear consensus emerges that MMSE-IRC should remain the primary baseline receiver for UE in the 6G demodulation study. It is widely regarded as a mature, power-efficient, and field-proven solution that ensures backward compatibility and implementation stability. At the same time, most companies acknowledge the growing relevance of non-linear receivers such as R-ML, especially for high-interference or high-rank MIMO scenarios. However, opinions differ on whether R-ML should be treated merely as an advanced optional feature or elevated to a co-baseline status.

Overall, all these views point toward a two-level receiver setup for 6G: use MMSE-IRC as the main baseline because it’s stable, efficient, and not too complex, and treat R-ML (or similar non-linear receivers) as an advanced option to test and compare performance versus power consumption. This setup would let 6G build smoothly on existing 5G systems while keeping room to add new features later as the physical layer evolves.

We think maintaining MMSE-IRC as the baseline offers immediate benefits — smoother migration from existing test frameworks, proven interoperability, and reduced certification risk. Meanwhile, including R-ML in the study phase allows measurable differentiation, helps characterize real-world interference behavior, and drives innovation in receiver design. To ensure practicality, any non-linear receiver studies should explicitly quantify complexity, power consumption, and reproducibility in laboratory environments, preventing excessive test costs or unrealistic certification demands.

In conclusion, the recommended path forward is to define MMSE-IRC as the 6G baseline receiver, while R-ML serves as the advanced benchmark for studying potential performance improvements in complex interference conditions. This dual-layer approach ensures that 6G demodulation requirements remain both forward-looking and practically implementable, achieving a balanced trade-off between innovation, performance, and operational feasibility.

Proposal 16	Define MMSE-IRC as the 6G baseline receiver, while R-ML serves as the advanced benchmark for studying potential performance improvements in complex interference conditions

Receiver assumption for BS

- Proposals

  - Option 1: MMSE-IRC as a baseline receiver.

  - Option 2: Study feasibility of considering higher than 8Rx scenarios.

2.3.2	BS baseline receiver

In RAN4#116-bis meeting, it was highlighted by many companies to consider a baseline receiver for 6G BS demodulation. The demodulation performance of a communication system primarily depends on the receiver’s capability to accurately estimate the transmitted information from a received signal affected by channel, hardware impairments and noise. The receiver algorithm plays a central role in this process, and its design directly impacts the achievable system performance and implementation complexity. An effective receiver algorithm should maintain robustness against various impairments, including inter-symbol interference (ISI), frequency offset, phase noise, etc, while achieving a practical trade-off between performance and computational complexity.

Observation: For 6G, the baseline receiver should exhibit robustness to key impairments such as ISI, frequency offset, and phase noise, while maintaining an optimal balance between demodulation performance and implementation complexity.

In 5G NR Rel-15, the default receiver defined for the base station (gNB) is a conventional Minimum Mean Square Error (MMSE) receiver. This receiver does not perform explicit inter-cell or intra-cell interference suppression, but it is mandatory in 5G NR specifications.

In Rel-19, demodulation requirements under interference conditions have been introduced for up to eight receive antennas (8Rx). The MMSE-Interference Rejection Combining (MMSE-IRC) receiver has been recognized as an optional feature in Release 19 and represents the current state-of-the-art implementation. The MMSE-IRC receiver may also be widely deployed in commercial networks. For Rel-19, LTE-based interference profiles have been utilized to define the BS demodulation requirements. However, for 6G, if MMSE-IRC is defined as the baseline receiver, it is recommended that more realistic interference modeling; better reflecting real-field deployment scenarios; be developed to ensure the performance evaluations are representative of practical conditions. This is further discussed in Sec. 2.5.

For 6G, it is desirable that the baseline receiver used in demodulation performance does not require any modifications to the physical layer design. Furthermore, the receiver should ensure robust performance, reasonable complexity, and broad vendor support. Given that discussions on the 6G physical layer (PHY) design are still ongoing in RAN1, it could be feasible to potentially consider MMSE-IRC receiver, given there are no PHY changes.

Proposal 17	Given that the 6G physical layer (PHY) design is still under discussion in RAN1, it is proposed that the MMSE-IRC receiver be considered as the baseline receiver algorithm for 6G base station demodulation performance evaluations, provided that no changes to the PHY design are introduced.

As highlight, until now BS demodulation requirements are defined up to 8Rx. In previous meetings, there have been discussions regarding a potential increase in the number of Rx antennas in the demodulation requirements, with the aim of reflecting more realistic field performance in the test setup. However, in our view, extending the configuration beyond 8 Rx using purely demodulation branches would not accurately represent field conditions. Furthermore, it would be useful for RAN4 to clarify the expected number of transmit antennas supported by 6G UEs, given that the current BS demodulation requirements assume up to 4 Tx antennas. Considering these factors, we do not see a clear benefit in increasing the number of Rx antennas for BS demodulation requirements. Such an extension would only add unnecessary complexity to the test setup and lead to higher testing costs, without providing meaningful performance insights. This is because it would require interconnections between every Tx and Rx antenna port, resulting in an exponentially increasing number of test equipment (TE) channel models and interconnections, making the setup complex.

Proposal 18	We propose to keep BS demodulation requirements up to 8Rx, as higher configurations would not reflect realistic field conditions and would only increase test complexity and cost.

## 2.4	TxEVM and SNR

TxEVM aspects

- Feature Lead notes

  - TxEVM related discussion has been captured in Ad-hoc minutes.

- Proposals

  - Option 1: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

    - Option 1A: Study impact of TX EVM for higher modulation order/ MIMO layers on Demodulation requirements.

    - Option 1B: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

    - Option 1C: Study whether the TxEVM requirements for the base station, at least in the simulations for deriving the demodulation requirements, could be tightened.

There was a proposal asking to study what EVM simulation assumptions should be used in demodulation and CSI requirements. From our view, in general demodulation and RF performance requirements shall remain decoupled, and Tx EVM requirements shall not be prematurely tightened or extended because it should maintain their current flexibility to avoid excessively restricting system design space.

In general, it is important to know that TE actually performs the test with the same EVM as the minimum requirement otherwise the DUT gets an additional margin that is not available in real operation. Or if not, when simulations are performed, they should include EVM that is the TE EVM. It may also relevant to other test setups, such as Tx power, MCS etc.

Specifically, for option 1C, the Tx EVM requirements (e.g., 3.5% for 256QAM) ensure minimum compliance across all equipment, not the actual achievable performance. Tightening these limits will not improve interoperability but will raise certification costs and restrict vendor design flexibility. While field measurements often show EVM < 1% or SNR > 30 dB, these reflect optimized deployments under favorable conditions. Standardization must consider worst-case configurations—multi-carrier, FR2, high-power, or OTA setups—where such tight limits are not always feasible. EVM is strongly linked to PA nonlinearity. Achieving lower EVM requires more complex DPD algorithms, higher back-off, and increased power consumption, which has already become separate topics for 6G.

As for option 1A&1B, the effectiveness of any Tx EVM study depends on the finalized PHY structure (e.g., waveform type, subcarrier spacing, beam granularity). Studying Tx EVM now risks generating results that will be obsolete once the architecture changes. Simulation data from 5G studies show that BLER degradation beyond 256QAM or 8Rx is dominated by channel estimation and inter-user interference. EVM plays only a secondary role, making it a low-priority study topic during the early phase of 6G.

TE vendors input is needed before any study

Proposal 19	Deprioritize the study of Tx EVM assumptions and requirements.

SNR aspects

- Feature Lead notes

  - SNR related discussion has been captured in Ad-hoc minutes.

- Proposals

  - Option 1: To ensure good field coverage, the SNR values in the demodulation requirements should follow the SSB SNR values that can be measured in existing 5G NR scenarios and those SSB SNR values that can be expected in upcoming 6G deployments.

  - Option 2: Study whether the coverage range for relevant field scenarios can be extended by defining demodulation requirements for larger SNR values as currently being used in 5G NR.

  - Option 3: Study whether the coverage range for relevant field scenarios can be extended for carrier aggregation. It should also be studied whether the SNR requirement should be dependent on the number of component carriers.

  - Option 4: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

The SNR aspect issue reflects a fundamental tradeoff between test comparability and realism.
From a practical standardization viewpoint, a tiered SNR definition should be adopted — maintaining the 20 dB baseline for consistency while introducing higher SNR extensions for 6G performance evaluation. As for option 1, it’s expected to be close to practical scenarios with some measured data/values so that to be a possible study direction. However, keep in mind that field measurement results include BS antenna gain for SSB, such as 18dBi antenna column gain of remote radio, but this gain is normalized in demodulation SNR definition. Therefore, more statistic data and analysis are needed on the mapping for SSB SNR values to demodulation requirements.Option 2 seems to be a possible direction for studying the applicable scenarios and the level of extended SNR range.

Proposal 20	Consider option 2 and study the applicable scenarios and the level of extended SNR range.

## 2.5	Interference modelling

Interference profile

- Proposals

  - Option 1: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios.

    - Option 1A: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios: gNB and UE configuration e.g., power class, antenna configuration Homogenous and heterogenous scenarios Asynchronization TDD or dynamic TDD scenario Semi-static/Dynamic SBFD operation in gNB.

    - Option 1B: For 6G Demodulation with interference modelling, further discussion, and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

  - Option 2: Study possible interference caused by MRSS, by interference cancellation or mitigation.

The interference profiles used for 5G DL/UL IRC receiver requirements are inherited from 4G studies. The deployment scenarios have been greatly changed in 5G and will be further changed in 6G. For example, more UE types are introduced from 5G with different Tx power and antenna types, more co-existing network types are considered in RF discussions, etc. Furthermore, the INR + TDL method-based interference modelling might not be good enough to address complicated interference conditions. It is necessary to have more practical interference modelling based on typical 6G deployment.

As for specific scenarios, the normal inter-cell and intra-cell inter-user interferences could be considered as the starting point including homogenous and heterogenous in synchronized deployments. The interference of MRSS can be also involved if the feasible migration receiver is agreed. But its impact might be smaller than LTE to NR regarding no CRS is used in NR. More complicated scenarios, such as asynchronization TDD or dynamic TDD scenario Semi-static/Dynamic SBFD operation in gNB, might need more input from RF discussion which can be lower prioritized.

Proposal 21 	Study the interference modelling for typical 6GR deployment which could start from inter-cell and intra-cell inter-user interferences in homogenous and heterogenous in synchronized deployments, then consider evaluate the impact of MRSS interference.

## 2.6	Performance and requirements

Demodulation testing

- Proposals

  - Option 1: For 6G demodulation study, use FRC style, MCS value, fixed rank, fixed channel bandwidth, fixed subframe configuration as a starting point.

  - Option 2: Study extending scope of demodulation tests with link adaptation.

  - Option 3: Discuss the additional margins and measurements uncertainty for requirements definition of 6GR.

  - Option 4: Discuss the SNR derivation procedure for 6GR, the span of ideal results span is <= [X] dB.

There was a discussion aiming to define the testing philosophy for 6G demodulation requirements — whether to retain the simplified, comparable structure used in 4G/5G or to evolve toward a more dynamic and realistic evaluation framework.

Our view is that Option 1 should serve as the starting point, using fixed FRC-style configurations to ensure test repeatability and vendor comparability. Option 4, however, could be developed in parallel, establishing a transparent and quantitative SNR derivation process (with a defined ideal result span) to bridge simulation and requirement levels. Option 4 can also be triggered by a specific topic or issue during any of the 6G SI/WI topics.

Proposal 22	Propose option 1 for the demodulation testing at the first stage and study option 4.

OLLA with link adaptation

- Feature Lead notes

  - Option 1 discussion has been captured in Chairman and Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on OLLA in the next meeting as several companies see value to study OLLA as part of 6G demod SI.

- Proposals

  - Option 1: Study to include OLLA in ATP requirements.

    - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point.

    - Option 1B: Study the feasibility to include OLLA in ATP requirements.

While the idea of including link adaptation and OLLA in demodulation testing aims to improve realism, it may not be appropriate or beneficial for the 6G study phase. First, OLLA and link adaptation belong to system-level and scheduler-level behaviors, not to the physical layer demodulation process itself. Their impact depends heavily on the network configuration, traffic mix, scheduling algorithm, feedback periodicity, and even vendor-specific optimization parameters. Embedding these higher-layer mechanisms into PHY-level demodulation tests would blur the boundary between physical-layer conformance and system performance evaluation, making the tests less standardized and less reproducible across vendors.

Second, OLLA relies on adaptive feedback loops and long-term BLER statistics, which require time-evolving simulations or closed-loop test setups to stabilize. Such setups are complex, time-consuming, and difficult to harmonize across labs. Introducing them would significantly increase test duration and cost, while providing little additional insight into the intrinsic demodulation capability of the UE or BS receiver. The purpose of demodulation tests should remain to isolate and evaluate receiver performance under controlled conditions—not to simulate full network adaptation dynamics.

Third, defining a common OLLA implementation across vendors is nearly impossible. Each manufacturer already employs proprietary algorithms tuned to their scheduler, HARQ behavior, and deployment strategy. Any “reference” OLLA model chosen by RAN4 would either be too simplified to reflect reality or too specific to one vendor’s design, undermining fairness and comparability of results.

Finally, link adaptation and OLLA are already well covered in system-level performance simulations and network evaluations, which are more suitable environments to study such adaptive mechanisms. Mixing them into conformance-style demodulation tests would duplicate effort, increase complexity, and risk diverting resources from more fundamental 6G receiver research (e.g., non-linear detection, interference suppression, or AI-based demodulation).

For these reasons, it would be more effective to keep 6G demodulation testing focused on receiver performance and channel conditions, while reserving link adaptation and OLLA studies for system-level work items or later performance evaluation phases. This separation preserves clarity, reproducibility, and efficiency in the early 6G study stage.

Proposal 23	Deprioritize the study of adding OLLA for demodulation requirement

CSI reporting test methodologies

- Proposals

  - Option 1: Streamline CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits.

  - Option 2: Study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing.

  - Option 3: Simplify the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ.

  - Option 4: Study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing.

  - Option 5: Study RI reporting requirements test metrics and test methodologies.

    - Option 5A: Investigate alternative metric that measures consistency and accuracy of Rank Indicator (RI).

During the last meeting, there are proposals discussing the deficiencies in legacy 5G CSI reporting test methodologies (CQI / PMI / RI) and proposes measurement/metric changes for 6G-era CSI reporting verification. Some companies emphasised simplification (move from multi-step to 1-step throughput/BLER-based tests; questioned throughput-ratio γ for PMI; re-think RI tests) and asked whether separate CQI/PMI/RI tests are needed beyond combined demodulation + link-adaptation testing. Companies also highlighted that current RAN4 settings are too relaxed (e.g., Rel-18 eTypeII Doppler codebook requirements do not show field gains vs Rel-16), that PMI test references (random precoder) may hide demodulation issues, and proposes alternative RI metrics (follow-RI vs median-RI) to verify accuracy/consistency of reported rank — in particular for >2Rx devices where current tests fail to require/report Rank-4.

We have also observed in Rel-19 PMI reporting requirements with more than 32 CSI-RS ports that the throughput ratio of follow PMI and random PMI is very large (close to Infinity) as the number of CSI-RS ports are increased. Since 6GR consider even larger antenna ports more than 128 CSI-RS ports, we expect the current PMI test metrics do not work for such a case.

From our perspective, we share the similar view that legacy 5G CSI reporting tests (CQI/PMI/RI) use multi-step procedures and reference metrics (e.g., throughput ratio γ vs random precoder, fixed-rank references) that can be misleading in spatially selective channels or for devices with >2 Rx. Current requirements are either overly complex (CQI 2-step) or too loose (PMI/RI thresholds allow poor reporting to pass), and do not always demonstrate the field gains targeted by advanced codebooks or higher rank reporting.

In the field, we have observed the wrong PMI reporting from the UE located in cell-edge. According to our analysis, some UE cannot derive the proper PMI under intercell interference environment. Since the current RAN4 PMI (and RI) reporting requirements are specified without inter-cell interference, we cannot verify the propose of PMI/RI reporting under inter-cell interference environment.

Many legacy metrics (γ vs random precoder; fixed-rank references) try to isolate one variable (precoding or rank) by comparing to a reference. But these references can interact strongly with demodulation, codebook granularity, mobility, and channel spatial selectivity. As one company pointed out, in high spatial-selectivity channels ‘random precoder’ may artificially reduce baseline throughput and inflate γ, or conversely hide deficiencies.

6GR will likely introduce richer CSI semantics (dense MIMO, distributed panels, more dynamic mobility patterns), so testing must: (a) be tied to target use-cases (mobility, spatial selectivity, panel topology), (b) include per-rank/per-codebook SNR operating points rather than one-size-fits-all thresholds, and (c) ensure metrics are robust to demodulation coupling. Having a better UE reporting leads to better downlink scheduling/throughput and less wasted overhead (e.g., unnecessary signaling, wrong rank/precoder assignments). The benefit is long-term network efficiency and user QoE.

Therefore, specifically, we propose to have some studies on the following CSI reporting tests,

- CQI: study the feasibility of adopting 1-step approach, for example: follow-CQI throughput/BLER requirements

- PMI: study the feasibility of setting absolute throughput/SNR thresholds instead of a relative metric; study the feasibility of the tests with specific scenarios that actually demonstrates field-relevant gains (e.g. specific Doppler windows, mobility profiles, inter-cell interference scenario, spatial channel model)

- RI: study the feasibility of an alternative metric of RI requirements, for example: setting throughput ratio as the test metric

Proposal 24	Propose to have some studies on the following CSI reporting tests,

- CQI: study the feasibility of adopting 1-step approach, for example: follow-CQI throughput/BLER requirements

- PMI: study the feasibility of setting absolute throughput/SNR thresholds instead of a relative metric; study the feasibility of the tests with specific scenarios that actually demonstrates field-relevant gains (e.g. specific Doppler windows, mobility profiles, inter-cell interference scenario, spatial channel model)

- RI: study the feasibility of an alternative metric of RI requirements, for example: setting throughput ratio as the test metric

In the meantime, RAN4 should also analyse and measure the potential risks for applying any new test methodology in:

- Feasibility of testing by TE vendors

- Cost/time consuming of a more complex test

- Applicability of old tests

If the pass requirements become stricter, some UEs might fail the tests at first, which could cause inconsistency among devices and increase testing costs. Vendors need to find a balance between making devices perform better in real networks and keeping the certification process manageable.

## 2.7	New TE functionalities

SRS based precoding

- Feature Lead notes

  - Option 1 discussion has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on SRS based precoding in the next meeting as several companies see value to study SRS based precoding as part of 6G demod SI.

- Proposals

  - Option 1: Study whether and how to define baseline SRS based precoding procedure in TE to enable aligned simulation assumptions.

This discussion focuses on whether and how to introduce a baseline SRS-based precoding procedure in TE to better align demodulation test assumptions with realistic system simulations. We are open for the study as it might improve realism for TDD and special channel scenarios. But keep in mind that if SRS-based precoding is introduced into the RAN4 requirements, the TDD and FDD test processes will be separated, which is not in line with the principle of consistent design. RAN4 has never previously introduced the "dynamic channel measurement + precoding" process into standard testing. Implementing SRS estimation, channel reconstruction, and beamforming calculations at the TE layer is highly complex and lacks a unified reference model.

We propose RAN4 to keep fixed or PMI based precoding as the baseline for comparability and simplicity, and study SRS-based precoding as an optional or exploratory item to evaluate its potential benefits for simulation alignment. In the meantime, TE vendors’ inputs are very much important to add the feasibility during the study.

Proposal 25	RAN4 to keep fixed or PMI based precoding as the baseline.

Time/frequency/phase offset precompensation

- Feature Lead notes

  - Discussion of Option 1 has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on time/frequency/phase offset precompensation in the next meeting as several companies see value to study time/frequency/phase offset precompensation as part of 6G demod SI.

- Proposals

  - Option 1: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., applying timing offset reports (CJT).

Since RAN4 demodulation requirements aim to verify the UE behavior regarding a specific new feature, RAN4 usually defines demodulation requirements assuming the time/frequency offset compensation has been done. For example, for NTN and ATG, time/frequency offset compensation is necessary to be done before demodulation part, only some small error e.g. 0.1ppm will be left for further processing. RAN4 has never taken scheduling into account since it will need a deep involvement of two sides which is against the target of an isolated test. Lacking a matured and aligned model is another barrier as it will need a comprehensive simulation of system level among different companies.

Proposal 26	Deprioritize the study until more inputs from TE vendors are received regarding the feasibility of dynamic TE decisions.

## 2.8	UE classification and applicability

UE classification

- Proposals

  - Option 1: Study demod requirement handling for UE classifications and agree on questions such as a baseline set of requirements for all devices vs. individual requirements for each UE classification.

Applicability rules

- Proposals

  - Option 1: Discuss improving Demodulation spec in 6GR by replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

  - Option 2: Implement a capability-aware test applicability framework that considers device functionality. For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases.

Device types

- Proposals

  - Option 1: Wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards discuss how device types can be covered in the test framework of RAN4.

In current UE demodulation specifications, it is concerned by readers that so many applicability rules combined with very complicated device capabilities. It is very confused to understand how and which requirement should be applied for a UE feature. That’s the reason behind companies’ proposals to have simpler and better organized requirement structure.

The UE device types are studied by RAN1 and would be defined later. It is possible to also have a baseline capability list for all or most UE types which could be used for mandatory requirement definition at the beginning of 6GR. Other capabilities can be defined for a certain of UE types as enhanced capabilities, and the corresponding applicability rules can be used. RAN4 can further study the specification structure based on it.

Proposal 27	RAN4 study the structure of applicability rules based on RAN1 conclusions on UE device type definition and capability classification.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Rel-20 NR SCM WI have agreement on band agnostic solution for CDL models.

Observation 2	Observation: For 6G, the baseline receiver should exhibit robustness to key impairments such as ISI, frequency offset, and phase noise, while maintaining an optimal balance between demodulation performance and implementation complexity.

Observation 3	TE vendors input is needed before any study

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to further clarify the definition of “field condition” for minimum demodulation requirement regarding typical deployments, robust receiver algorithm verification and a certain level of dynamic environment.

Proposal 2 	The higher efficient inter-group coordination and interactions between RAN1 and RAN4 is needed to have a more reasonable timeline for both groups.

Proposal 3 	Postpone the parameter discussion for 6G Demod until RAN1 and RAN4 have conclusion on 6G system parameters.

Proposal 4	Pending the discussion of ISAC in 6G Demod until RAN1 have conclusion on frame structure and interference study.

Proposal 5	Follow RF conclusion on test type per feature per frequency range for demodulation conformance tests.

Proposal 6	Consider defining the demodulation performance requirements for UE not in CONNECTED status even if it does not send the feedback from the UE, if it is justified to define the performance requirements.

Proposal 7	the CDL-based channel model should be considered as baseline for 6G demodulation requirements. TDL models used in 5G could be maintained in 6G for some low rank transmission cases to save effort in case of no available CDL model can be applied.

Proposal 8	Benefits of new CDL channel model derivation for ISAC, NTN, HST and ATG should be clarified first if there are specific concerns.

Proposal 9	Reuse existing timing/Doppler/power drifting model for NTN and HST scenario in 6G.

Proposal 10	Derive new channel models for new frequency range, such as 7 – 15GHz.

Proposal 11	Clarify the benefit of derive CDL model for FR2 demodulation first regarding the rank limitation of OTA tests.

Proposal 12	Take similar approach as in Rel-20 to handle CDL model band agnostic issue. Multiple CDL models can be derived for some special cases to have more feasible or wider coverage requirements.

Proposal 13	Take Option 1a for UL channel model issue.

Proposal 14	Get alignment on channel properties and performance if new CDL models with major changes are derived in 6G.

Proposal 15	Consider PMI bias issue in 6GR if it can’t be solved in 5G-A in Rel-20 period.

Proposal 16	Define MMSE-IRC as the 6G baseline receiver, while R-ML serves as the advanced benchmark for studying potential performance improvements in complex interference conditions

Proposal 17	Given that the 6G physical layer (PHY) design is still under discussion in RAN1, it is proposed that the MMSE-IRC receiver be considered as the baseline receiver algorithm for 6G base station demodulation performance evaluations, provided that no changes to the PHY design are introduced.

Proposal 18	We propose to keep BS demodulation requirements up to 8Rx, as higher configurations would not reflect realistic field conditions and would only increase test complexity and cost.

Proposal 19	Deprioritize the study of Tx EVM assumptions and requirements.

Proposal 20	Consider option 2 and study the applicable scenarios and the level of extended SNR range.

Proposal 21 	Study the interference modelling for typical 6GR deployment which could start from inter-cell and intra-cell inter-user interferences in homogenous and heterogenous in synchronized deployments, then consider evaluate the impact of MRSS interference.

Proposal 22	Propose option 1 for the demodulation testing at the first stage and study option 4.

Proposal 23	Deprioritize the study of adding OLLA for demodulation requirement

Proposal 24	Propose to have some studies on the following CSI reporting tests,

	CQI: study the feasibility of adopting 1-step approach, for example: follow-CQI throughput/BLER requirements

	PMI: study the feasibility of setting absolute throughput/SNR thresholds instead of a relative metric; study the feasibility of the tests with specific scenarios that actually demonstrates field-relevant gains (e.g. specific Doppler windows, mobility profiles, inter-cell interference scenario, spatial channel model)

	RI: study the feasibility of an alternative metric of RI requirements, for example: setting throughput ratio as the test metric

Proposal 25	RAN4 to keep fixed or PMI based precoding as the baseline.

Proposal 26	Deprioritize the study until more inputs from TE vendors are received regarding the feasibility of dynamic TE decisions.

Proposal 27	RAN4 study the structure of applicability rules based on RAN1 conclusions on UE device type definition and capability classification.

# References

[1]	R4-2514645, WF on [116bis][106] 6G Demod, MTK