3GPP TSG RAN WG4 #117	R4-2521775

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	4.22.2

Source: 	Ericsson

Title:	Remaining issues on LP-WUS RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 continued the discussion of core maintenance requirements for the LP-WUS/WUR and made the following agreement [1].

### Issue 1-1-3 LP-WUR operation with RedCap ### Agreement: ### The MR wake up delay of the 2Rx RedCap UE applies for 1Rx RedCap UE ### Issue 1-1-9: On the serving cell and interference cell LP-SS with opposite sequences for the case when {M, L}={1,6} ### Agreement: ### Capture the following note in LP-WUR core requirements: ### Note: Higher margin may be required for UE to consider an entry or exit criteria fulfilled when LR related thresholds are configured, if the serving cell and neighbour cell are using opposite binary sequences for LP-SS, and LP-SS are transmitted on the same time and frequency resource in this two cells.

In this contribution, we continue the discussion on the remaining issues of LP-WUS.

- Remaining issues

Low mobility

In Rel-16 RRM relaxation, except not in cell edge criteria, RAN4 also defined the low mobility criteria. In Rel-19 LP-WUS, RAN2 is still discussing whether to consider the low mobility criteria based on LR. From our understanding, whether to introduce LR-based low mobility criteria is up to RAN2. Therefore, RAN4 needs to wait RAN2’s final conclusion about whether considering low mobility for Rel-19 LP-WUS and further decide the related requirements.

Proposal 1: RAN4 to wait RAN2’s final conlusion about whether to consider low mobility criteria in LP-WUR.

Measurement behaviour after UE exiting case 1 (case 3)

In last meeting, there was one remaining issue related to transition period. When UE exits from case 1, UE cannot check whether RRM relaxation condition is met or not since MR just wakes up and no MR based measurement has been executed. Therefore, UE shall follow the legacy measurement behaviour first and check whether the RRM relaxation criteria is met or not when UE finishes the measurement.

Issue 1-1-7: On MR measurement behaviour after exiting case 1 (case 3) due to corresponding conditions cannot be met - Proposals: - Scenario 1: exit from case 1 due to corresponding condition cannot be met: - Option 1: UE follows legacy measurement and meet the corresponding requirements. (oppo Huawei) - Option 2: Up to UE implementation and no requirements apply. (oppo) - Option 3: UE shall meet the requirements for Case 3 (if the related relaxation criteria are configured); (xiaomi) - Scenario 2: exit from case 3 due to corresponding condition cannot be met: - Option 1: After existing Case 3, UE shall meet the legacy requirements. (xiaomi vivo) - P1: RAN4 to discuss transition requirement among normal measurement(Rel-15, Rel-16 measurement) and Rel-19 MR RRM measurement relaxation, offloading (Ericsson)

Proposal 2: When UE exits from case 1, UE shall follow legacy MR measurement requirements.

- Summary

In this contribution, we discussed the remaining issues on LP-WUS operation. Based on the discussions, we have made the following proposals and observations:

Proposal 1: RAN4 to wait RAN2’s final conlusion about whether to consider low mobility criteria in LP-WUR.

Proposal 2: When UE exits from case 1, UE shall follow legacy MR measurement requirements.

- References

[1]. R4-2514811, “WF for RRM requirements for LP-WUS/WUR”, vivo