3GPP TSG RAN WG4 #117	R4-2521776

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.16.2

Source: 	Ericsson

Title:	Discussion on remaining issues for LP-WUR performance requirement

Document for:	Discussion

# Introduction

In last meeting, RAN4 finished most of the performance part test cases on Rel-19 LP-WUS WI. In this contribution, we’ll discuss the maintenance part of the test case design [1].

- General aspects

Test case of entry condition

In last meeting, RAN4 agreed to define the test cases for exit condition, however, whether to define the test case for entry conditions is FFS.

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 Ad hoc meeting agreement: - RAN4 to define at least test cases for exit conditions. - FFS on whether to define test cases for evaluation of entry conditions.

In our understanding, it’s important to verify UE’s behaviour if the entry condition is not met but UE enters the RRM relaxation or offloading mode. Currently, when UE enters offloading mode, UE won’t perform RRM measurement by MR. If UE doesn’t follow the criteria set by NW, it’s a big risk for cell reselection in IDLE mode. Therefore, RAN4 should define a test case to verify UE behaviour when the LP-WUS entry condition is not met.

Proposal 1: RAN4 to define the test cases to verify the entry condition as follow.

- When the entry condition is not met, the UE which enters RRM relaxation or offloading mode shall fail the test case.

- Summary

In this contribution, we have initially discussed the LP-WUS performance part. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to define the test cases to verify the entry condition as follow.

- When the entry condition is not met, the UE which enters RRM relaxation or offloading mode shall fail the test case.

- References

[1]. R4-2514811, “WF on RRM requirements for NR_LPWUS”, vivo