3GPP TSG RAN WG4 #117	R4-2521778

# Dallas, USA, Nov. 17 – 21, 2025

Agenda Item:	6.15.2.2

Source: 	Ericsson

Title:	Discussion on remaining issues for SSB adaptation performance requirements

Document for:	Discussion

# Introduction

In last meeting, the following agreements are achieved about SSB adaptation test case.

Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement Agreement: - Besides L3 intra-frequency measurement, specify the following test case for SSB adaptation - L1-RSRP measurement - L1-SINR measurement Issue 1-2: General principle Agreement: - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. Issue 1-3: SMTC and SSB configuration Agreement (in AH): - For L1-RSRP measurement test case: - SMTC periodicity from 20ms to 80ms, and from 80ms to 40ms. - SSB periodicity from 20ms to 80ms, and from 80ms to 40ms. - For other test cases: - SMTC periodicity from 80ms to 20ms. - SSB periodicity from 80ms to 20ms. Issue 1-4: How to verify SSB adaption Agreement: - For L3 Intra-frequency serving Cell measurement: - UE is configured with Event A1 or A2. - Condition of serving Cell is changed to satisfy the Event threshold at the instance after DCI 2-9 received + the transition period. - It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

In this contribution, we’ll discuss the test case design for L1 based SSB adapation.

# Test case design

Currently, only DCI format 2_9 can be used for SSB adaptation. The processing time for SSB adaptation is defined as follow. Therefore, when RAN4 defines the L1 based SSB adaptation test case, RAN4 should guarantee to verify such DCI processing time.

Issue 1-1-1: Processing time for SSB adaption Agreement RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation. - FR1: 2ms FR2: 3ms

Proposal 1:  For L1-based SSB adaptation, RAN4 shall define the test case to verify the DCI processing time.

- Summary

In this contribution, we have discussed the performance part of SSB adaptation. Based on the discussions, we made the following proposals and observations:

Proposal 1:  For L1-based SSB adaptation, RAN4 shall define the test case to verify the DCI processing time.

- References

- R4-2514809, WF on Netw_Energy_NR_enh_RRM_Part2, Huawei