3GPP TSG-RAN WG4 Meeting #117										R4-2521784

Dallas, Texas, USA, 17th to 21st November 2025

Agenda Item:	6.8.3.1

Source:	Ericsson

Title:	On Transmitter Spurious Emission Co-existence discussion

Document for:	Approval

# 1	Introduction

LS from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence Tx spurious emission requirement levels for BS. It was acknowledged by RAN4 in the response LS [2].

The co-existence spurious emission requirement was originally derived in 1999 for UTRA. The technical background at that time was very much focused on the introduction of UTRA FDD/TDD operating at 2 GHz. The assumptions used as references to the original requirement definition can be found in a contribution [3] presented at RAN4#113 in Orlando. Over the years RAN4 have defined new operating bands below and above 2 GHz using the same requirement level. In current specification, bands are defined within the range 410 MHz up to 7125 MHz with the same requirement level, except for a few occasions (e.g. protection of GSM). Therefore, it is now time to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

The objective can be summarized as:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

In this contribution, we share our simulation evaluations and findings to trigger further discussions on open issues, along with a draft LS response to ETSI MSG TFES in Annex.

# 2	Discussion

In this contribution, we outline our approach for conducting system-level evaluations to assess the requirements for spurious emissions considering technical aspects related to AAS base stations. Section 2.1 addresses the modeling of antenna gain for out-of-band unwanted emissions and Section 2.2 presents simulation results that support our findings.

## 2.1	Modelling of antenna gain for unwanted emissions

In Release-19, a roll-off profile was finally introduced in TR 38.922 providing a reference curve for modeling decorrelation in certain deployment scenarios. This topic has also been discussed in other standardization bodies, including ITU-R and ECC, though no firm conclusions have been reached so far.

The equation describes the composite far field antenna gain for an AAS in the presence of partial spatial correlation – particularly relevant for unwanted emissions.

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1

As can be seen in the equation, the composite antenna pattern is the sum of the subarray element gain and an array gain component which depends on the array geometry, beamforming vector and element correlation. This model accounts for spatial decorrelation due to spectral separation from the carrier, which affects the coherent summation in the array factor. The key addition in this model is the ρ(f) where f=offset from carrier center, which models the roll-off of coherent gain as a function of offset from the carrier center frequency.

Many companies have raised concerns that assuming the correlation factor ρ(f) to be either 0 or 1 outside the desired carrier band is too simplistic and unrealistic for unwanted emissions from an Active Antenna System (AAS).

The unwanted emissions consist of a mix of two components:

- Correlated intermodulation distortion (IMD) products, which are partially coherent across antenna elements because they originate from the same nonlinear processes.

- Uncorrelated wideband transmitter noise, which is random and independent at each antenna element.

Because of this mixture, the correlation factor ρ(f) does not simply jump from 1 (fully correlated) inside the carrier to 0 (fully uncorrelated) immediately outside it. Instead, ρ(f) gradually decreases with frequency offset from the carrier edge as noise becomes more dominant and coherence diminishes. Assuming ρ(f)=1 outside the carrier tends to overestimate the coherent array gain, potentially underestimating interference levels, while assuming ρ(f)=0 can be too conservative by ignoring any partial correlation that exists.

Selecting ρ therefore balances between underestimating array gain by assuming too little correlation and overestimating it by assuming too much, making it crucial to choose a value that reflects the actual mixture of noise and distortion in the system.

The full array doesn’t make sense for integer multiples of the fundamental frequency as the antenna array and beamforming are primarily optimized for the fundamental frequency. Thus, the antenna efficiency drops reducing the effective radiated power at integer multiples regardless of the array size. We can also not ignore the fact that the physical size of the antenna elements is fixed but the wavelength λ changes with frequency which further drastically changes the radiation pattern, impedance input and efficiency. These combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

As a tradeoff due to above reasons, we can look at two choices of correlation factor ρ – 0.1 (almost incoherent) and 0.5 (partial coherent), as a way to model the partial coherence between array elements.

For the modelling of antenna gain for unwanted emissions, RAN4 to assume correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations.

In the agreements from last meeting [4], an alternate model was proposed by one of the companies into the existing 3GPP model, to consider a scaling factor based on the harmonic order applied to the element spacing of fundamental frequency. It is difficult to ascertain if this is the most appropriate way to model the harmonic response but can be a good starting point for further discussions.

## 2.2	Simulation Results

In this section, we discuss the results based on correlation factor 0.5 and 1. The results are obtained deriving the coupling loss between BS-to-UE and BS-to-BS from a two-network deployment where the aggressor network is at 2 GHz and the victim network is at 6.4 GHz.

### 2.2.1	BS-to-BS coupling with AAS Macro at 2 GHz and AAS Macro at 6.4 GHz

Figures 2.2.1.1-1 and 2.2.1.2-1 depict the coupling loss CDF between the aggressor AAS Macro BS operating at 2 GHz, and the victim AAS Macro BS receiver operating at 6.4 GHz depending on the Network grid shift 100% or 50%.

The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 2 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – based on correlation factor for 0.5 and 1.

  - Propagation model – TR 38.901

  - Antenna height – 25m

  - Antenna model – existing 3GPP model based on fundamental frequency radiation pattern

  - Without 3dB polarization gain

- Victim

  - Transmitting frequency – 6.4 GHz

  - Antenna model – AAS with 8x16 array and 3x1 subarray antenna configuration

  - Gain at Rx – full array gain

  - Propagation model – TR 38.901

  - Antenna height – 25m

  - Antenna model – existing 3GPP model based on fundamental frequency radiation pattern

  - BS NF is 6 dB (based on n104)

We evaluate the coupling loss CDF from the aggressor BS and to a victim UE/ BS, assuming grid shift 100% and 50%. In Rel-99, the calculation from the cell arrangement point towards 100% grid shift if we have to maintain the same grid structure. However, it should be noted that the work in Rel-99 do not consider the AAS BS aspects. We acknowledge the agreements from last WF [4] to derive the requirements from 100% grid shift but at the same time we need to depict a real deployment which resembles 50% grid shift. This is the reason why we have also shared results with 50% grid shift to be captured in the TR for information to the readers.

#### 2.2.1.1	Network grid shift is 100%

Figure 2.2.1-1 Coupling loss CDF from aggressor BS Macro to victim BS Macro, when GS is 100%

#### 2.2.1.2	Network grid shift is 50%

Figure 2.2.1.2-1 Coupling loss CDF from aggressor BS Macro to victim BS Macro, when GS is 50%

Tables 2.2.1-1 and 2.2.1-2 summarize the coupling loss values for correlation factor 0.5 and 1 for different CDF %tiles. The Release-99 approach to evaluate the minimum coupling loss, which refers to 100%tile on the CDF curve was calculated towards the wanted UE or a neighbouring BS. However, with AAS antennas it has large variations due to adaptive beamforming adjusting the beam patterns towards multiple UEs and different UE positions to maximize the reception and transmission of signal. This 100%tile sitting at the queue of the CDF is very sensitive to simulation and can fluctuate across different simulation runs, which might lead to variability in observed MCL values. This would have made sense in Release-99 with the assumption of passive antennas with a fixed radiation pattern. But with AAS antennas, this would capture an extreme simulation specific outlier and not reflecting the general network behaviour.

For the requirements derivation, consider the coupling loss CDF %tile as 99.99%tile or 99.95%tile instead of the 100%tile minimum coupling loss value.

Table 2.2.1-1 Coupling loss CDF %tile with correlation factor 0.5

| CDF %tile | Coupling Loss with 0.5 correlation factor |  |
| --- | --- | --- |
|  | Grid shift 100% | Grid shift 50% |
| 99.9 | -80 | -71.6 |
| 99.95 | -74.8 | -71.2 |
| 99.99 | -70.8 | -70.5 |

Table 2.2.1-2 Coupling loss CDF %tile with correlation factor 1

| CDF %tile | Coupling Loss with 1 correlation factor |  |
| --- | --- | --- |
|  | Grid shift 100% | Grid shift 50% |
| 99.9 | -79.4 | -71.6 |
| 99.95 | -79 | -70.2 |
| 99.99 | -72.3 | -68.7 |

The BS-to-BS coupling with 100% grid shift and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~4 dB and ~8 dB respectively.

### 2.2.2	BS-to-UE coupling with AAS Macro at 2 GHz and Isotropic UE at 6.4 GHz

Figure 2.3.2-1 depicts the CDF of the coupling loss between the aggressor BS deployed at 2 GHz and the victim UE associated to a 6.4 GHz network deployment.

Proposed detailed assumptions can be found in Annex A along with the simulation methodology. The assumptions to highlight are as follows –

- Aggressor

  - Transmitting frequency – 2 GHz

  - Antenna model – AAS with 8x8 array and no subarray antenna configuration

  - Gain at Tx – based on correlation factor for 0.5.

  - Propagation model – TR 38.901

  - Minimum BS - UE distance (2D) – 3 m

  - Antenna height – 10m

  - Antenna model – existing 3GPP model based on fundamental frequency radiation pattern

  - Without 3dB polarization gain

- Victim

  - Transmitting frequency – 6.4 GHz

  - Isotropic UE with 0 dBi gain and without beamforming

  - Propagation model – TR 38.901

  - UE NF is 9 dB (based on n104)

For BS-to-UE coupling, it is not clear the motivation behind adding the additional 3B polarization gain in the context of this study. Applying this artificial gain would not be meaningful, as it would artificially enhance the coupling level leading to an overestimation of the interference coupling. The effective gain towards the victim should derived from the directional radiation pattern of the two polarizations, without introducing the additional gain that is only justified for co-polarized serving links.

For BS to UE, do not consider the additional 3 dB polarization gain.

Figure 2.2.2-1 Coupling loss CDF from aggressor BS Micro to victim UE

Tables 2.2.2-1 summarize the coupling loss values for correlation factor 0.5 for different CDF %tiles.

Table 2.2.2-1 Coupling loss %tile with correlation factor 0.5

| CDF %tile | Coupling Loss with 0.5 correlation factor |
| --- | --- |
| 99.9 | -85.8 |
| 99.95 | -84.5 |
| 99.99 | -81.7 |

The BS-to-UE coupling without polarization gain and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~28 dB and ~31 dB respectively.

With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~4 dB/ 8 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

Observation 2	The BS-to-BS coupling with 100% grid shift and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~4 dB and ~8 dB respectively.

Observation 3	The BS-to-UE coupling without polarization gain and correlation factor 0.5 assumption at 99.99%tile and 99.95%tile indicates a potential correction of ~28 dB and ~31 dB respectively.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the modelling of antenna gain for unwanted emissions, RAN4 to assume correlation factor ρ = 0.5 as a tradeoff between performance and hardware limitations.

Proposal 2	For the requirements derivation, consider the coupling loss CDF %tile as 99.99%tile or 99.95%tile instead of the 100%tile minimum coupling loss value.

Proposal 3	For BS to UE, do not consider the additional 3 dB polarization gain.

Proposal 4	With BS-to-BS coupling being the dominant scenario, there should be a similar adjustment of ~4 dB/ 8 dB compared to Rel-99 values for both BS-to BS and BS-to-UE co-existence spurious emission requirements.

# References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[3]	  R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

[4]	R4-2515171, “WF for Tx spurious co-ex evaluation”, Moderator

# Annex: draft LS response to ETSI MSG TFES

TSG-RAN Working Group 4 (Radio) meeting #117	R4-25xxyyy

Dallas, Texas, USA, 17th to 21st November 2025

Title:	Draft LS to ETSI MSG TFES on Transmitter spurious emissions: co-existence requirements for high frequency bands

Response to:	LS R4-2414917 on Transmitter spurious emissions: co-existence requirements limits for high frequency bands

Release:	-

Work Item:	-

Source:	3GPP RAN WG4

To:	3GPP TSG RAN

Cc:	-

Contact person:	Shubham Bhargava

shubham.bhargava@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

-

# 1	Overall description

3GPP RAN WG4 has received the incoming LS from ETSI MSG TFES in R4-2414917 on the TX spurious emissions asking RAN4 to re-evaluate co-existence requirements limits for high frequency bands.

3GPP RAN WG4 would like to inform that the topic of TX spurious emissions for co-existence requirements limits for high frequency bands has been discussed.

3GPP RAN WG4 is documenting the methodology and relevant simulation assumptions in the technical report (TR 38.921) on the IMT parameters. The assumptions for the derivation are captured in Clause 10. The following should be noted:

- While some assumptions are carried forward from Release-99 study in TR 25.942, others are assumed keeping current deployment conditions.

- With no consensus on a number of issues – modelling of out of band antenna gain and percentile on Coupling Loss CDF curves, RAN WG4 has provided recommendations and left it to the reader discretion.

# 2	Actions

To TSG RAN

ACTION: RAN WG4 asks TSG RAN to consider the assumptions and results provided in TR 38.921 Clause 10 for the re-evaluation of Transmitted spurious emission co-existence work.

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN4 Meeting #118		9 – 13 February 2026		Gothenburg, EU

TSG RAN4 Meeting #118bis	13 – 17 April 2026			EU

TSG RAN4 Meeting #119		18 – 22 May 2026			China

TSG RAN4 Meeting #120		24 – 28 August 2026		Maastricht, EU

TSG RAN4 Meeting #120bis	12 – 16 October 2026		Korea

TSG RAN4 Meeting #121		16 – 20 November 2026		Calgary, Canada