# 3GPP TSG-RAN WG4 Meeting #117		R4-2521787

Dallas, Texas, USA, 17th to 21st November 2025

Agenda Item:	8.4.3

Source:	Ericsson

Title:	On 6G BS co-existence discussion

Document for:	Discussion

# 1	Introduction

The discussion related to 6G co-existence evaluation started in the RAN4#116bis meeting in Prague. There has been an initial agreement for co-existence aspects captured in WF [1].

In this contribution, we provide further discussion points related to the 6G co-existence work highlighting some critical areas.

# 2	Discussion

The agreements from RAN4#116bis meeting has been highlighted below -

The list of candidate frequencies ranges from 0.7 GHz to 30 GHz. However, most companies have expressed a preference to focus on the 7 GHz band for co-existence evaluations. It was indicated RAN4 has conducted relevant work documented in TR 38.921 for the 6.425 - 7.025 GHz and in  TR 38.922 for the 7.125 – 8.400 GHz which can be taken as inputs and thereby reducing the workload an avoiding duplication of effort for the new co-existence evaluations.

One can argue that during the study item, the choice of co-existence simulation assumptions related to BS-UE antenna characteristics, number of BS antenna elements, UE maximum outpower and others can be updated to better reflect current deployment conditions. Such refinements could yield co-existence results that differ from those in earlier studies. However, it is important to recognize that the ACLR – ACS results from the earlier RAN4 work have already been communicated to external organizations - ITU-R WP5D, through related LS replies.  These results are being used as input to ITU-R sharing and compatibility studies for spectrum allocation and protection criteria between mobile and incumbent services.

If RAN4 were to change the simulation assumptions now and even if those changes led to different ACLR - ACS results, it would create inconsistency with the on-going ITU-R work, which depends on the previously agreed parameters. Such discrepancies could undermine the credibility and alignment between RAN4 and ITU-R studies, and therefore any modifications to the assumptions must be considered very carefully. This does not mean that there are no limitations to the previous co-existence studies. In TR 38.921, the co-existence studies for 7 GHz assumed BS array antenna parameters without sub-array configuration. However, this limitation can be addressed with the use of BS antenna parameters from TR 38.922 of 8 GHz which considered sub-array antenna configurations.

The BS antenna parameters of 8 GHz from TR 38.922 which include sub-array configuration can be used for the 7 GHz frequency if needed.

During the IMT SI for 15 GHz documented in TR 38.922, it was agreed to study the in-band blocking requirements during the WI phased primarily due to time-constraints. It was also noted that this parameter is not needed by ITU-R WP5D for the sharing and compatibility studies and hence no value was communicated in the related LS. However, the in-band blocking requirements should be studied during 6G co-existence work.

In the IMT SI for 15 GHz, it was proposed to study in-band blocking requirements during the work item phase.

RAN4 to re-use the co-existence study outcomes from the IMT studies documented in TR 38.921 and TR 38.922 where necessary, unless significant impacts are identified.

The intention is not to restrict the co-existence studies but rather derive simulation assumptions and perform studies where necessary. One example is the LP-WUS feature in 5G. the same radio jammer between main receiver and LR is agreed as a working assumption in RAN4, In the end, two types of LR are specified and one type LR relaxes both noise figure more and also selectivity claiming the limited dynamic range of the filter capability.  As the LP-WUS signal is new physical signal and no coexisting study has been done for this feature, it is unclear how selectivity can be relaxed without any concrete coexisting. Therefore, the coexisting study should be done at 6G during the study phase. If the receiver is blocked by the jammer, this creates a coverage hole in the network, and the feature should ensure the coverage in a holistic sense.

RAN4 to set a proper selectivity for Wake Up receiver during the 6G co-existence study.

Another example is the recent LS from ECC PT1 with specified technical questions related to operation handling and modelling AAS BS below 1 GHz, received at RAN4#116bis meeting in Prague [2]. ECC PT1 have asked RAN4 the following questions:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band.

Based on the discussion in the WF [3], RAN4 has agreed to inform RAN and seek guidance on how to deliver answers to all technical questions raised by ECC PT1 based on a workplan. However, it should be noted that such activities are not planned in Rel-20 timeline, and one way could be to leverage the RAN4 6G BS RF study. There has been a tentative agreement for Question 1 and 2 in the last WF [3], but it should be noted that such requirement levels for unwanted emission were derived for non-AAS operating around 2 GHz as part of UTRA technical background. With the consideration of AAS BS below 1 GHz, this necessitates the re-evaluation to check current requirement levels for BS ACLR/ ACS and also Transmitter spurious emission co-existence levels with relevant simulation assumptions.

With the consideration of AAS BS operating below 1 GHz, this necessitates the re-evaluation to check current requirement levels for BS ACLR/ ACS and also Transmitter spurious emission co-existence levels with relevant BS antenna size and deployment parameters – ISD, grid shift and others.

If we look more carefully at the questions, Question 3 is not straightforward and probably requires further extensive studies. Hence, this means the work needs to be carefully planned to derive relevant simulation assumptions for operation below 1 GHz and establish array antenna model for out-of-band characteristics. In our companion paper [4], we have shared some initial thoughts on antenna array configuration and array model assumption which can be taken as a starting point. However, proper work plan need to be presented with the dedicated TU allocation to answer Question 3.

RAN4 to re-evaluate the check current requirement levels for BS ACLR/ ACS and also Transmitter spurious emission co-existence levels with relevant BS antenna size and deployment parameters – ISD, grid shift and others, with considerations of AAS BS operating below 1 GHz within the scope of 6G BS co-existence work.

There is an on-going co-existence work in Rel-19 to re-evaluate Tx spurious emission co-existence levels according to the request from ETSI/ TFES focusing on bands in the upper region of FR1 with priority above 5.9 GHz. It is reasonable to expect that 6G networks will reply heavily to AAS BS with beamforming capabilities. This further emphasizes the fact that with the use of AAS beamforming capabilities, the legacy simulation assumptions are no longer representative of realistic 6G deployments (re-used for LTE and NR) and therefore need to be revisited for 6G. The on-going Rel-19 work can be used as a foundation to evaluate such requirements in 6G as the new assumptions consider –

- System level evaluations based on CDF points (still in discussion in Rel-19 work)

- Dynamic behavior of AAS with updated deployment parameters

Similarly, the receiver in-band blocking should be re-evaluated considering 6G deployment conditions. These levels similar to additional spurious emission co-existence levels were originally derived for UTRA (re-used for LTE and NR). For 6G, it is expected that the AAS BSs will use large array configurations with sub-array structures where interference is spread across many receiver branches and each individual receiver sees less interference as compared to a non-AAS BS using a single column passive antenna. Hence, the in-band blocking requirement should be re-evaluated to reflect this more realistic, distributed interference scenario instead of old single-antenna assumptions. Some details have also been shared in our companion contribution [5].

The Rel-19 work to re-evaluate Tx spurious emission co-existence levels can be used as foundation for BS co-existence requirements, based on updated AAS BS and deployment assumptions.

RAN4 to re-evaluate in-band/ out-of-band blocking related requirements considering AAS BS architectures and current deployment conditions, rather than on legacy assumptions from Rel-99 UTRA time evaluations.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The BS antenna parameters of 8 GHz from TR 38.922 which include sub-array configuration can be used for the 7 GHz frequency if needed.

Observation 2	In the IMT SI for 15 GHz, it was proposed to study in-band blocking requirements during the work item phase.

Observation 3	With the consideration of AAS BS operating below 1 GHz, this necessitates the re-evaluation to check current requirement levels for BS ACLR/ ACS and also Transmitter spurious emission co-existence levels with relevant BS antenna size and deployment parameters – ISD, grid shift and others.

Observation 4	The Rel-19 work to re-evaluate Tx spurious emission co-existence levels can be used as foundation for BS co-existence requirements, based on updated AAS BS and deployment assumptions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to re-use the co-existence study outcomes from the IMT studies documented in TR 38.921 and TR 38.922 where necessary, unless significant impacts are identified.

Proposal 2	RAN4 to set a proper selectivity for Wake Up receiver during the 6G co-existence study.

Proposal 3	RAN4 to re-evaluate the check current requirement levels for BS ACLR/ ACS and also Transmitter spurious emission co-existence levels with relevant BS antenna size and deployment parameters – ISD, grid shift and others, with considerations of AAS BS operating below 1 GHz within the scope of 6G BS co-existence work.

Proposal 4	RAN4 to re-evaluate in-band/ out-of-band blocking related requirements considering AAS BS architectures and current deployment conditions, rather than on legacy assumptions from Rel-99 UTRA time evaluations.

# References

[1]	R4-2514638, “WF on 6G BS RF and co-existence”, 3GPP RAN4#116bis, Ericsson

[2]	R4-2513058, “Parameters for AAS BS operating in bands below 1 GHz”, ECC PT1

[3]	R4-2515071, “Way Forward for [116bis][324] LS_BDaT reply to CEPT”, CATT

[4]	R4-2521049, “Draft LS to RAN on request of information related to AAS BS operation below 1 GHz”, Ericsson

[5]	R4-2521435, “BS RF considerations for 6G”, 3GPP RAN4#117, Ericsson