3GPP TSG-RAN4 Meeting #117	R4-2521788

Dallas, US, Nov. 17 - 21, 2025

Source:	Nokia

Title:	Enhanced test methodologies to cover NTN Ka and Ku bands

Agenda item:	7.3.3

Document for:	Discussion

Introduction

Based on the agreement in the WF from RAN4 #116-bis meeting [1] and the summary in [2], in this paper, we’d like to present our views on the related issues of OTA tests in NTN Ka and Ku bands.

# WF in [1]

### Sub-topic 2-10: Beamlock

RAN4 further check whether beam steering is needed for VSAT conformance testing.

Summary in [2]

### Sub-topic 2-2: OTA range type

- Proposals:

- Proposal 2 from CATT in R4-2513209

  - Proposal 1:  Consider IFF via CATR test method as first priority and NFTF test method as second priority for VSAT device OTA testing.

- Proposal 2 from Vivo in R4-2513572

  - Proposal 2: RAN4 should study the feasibility and complexity of extending FR1 AC system and FR2 IFF/DFF system to cover NTN Ka/Ku frequency range.

- Proposal 1 from Huawei, Hisilicon in R4-2513580

  - Proposal 1: remove DFF from consideration as the far field distances required by a quiet zone of 60cm at Ku and Ka bands are too large to implement in anechoic chambers.

- Proposal 2 from Huawei, Hisilicon In R4-2513580

  - Proposal 2: develop test procedures for both IFF and NFTF.

- Proposal 3 from CAICT in R4-2513812

  - Proposal 3: RAN4 is recommended to analyze the minimum required range length for DFF-based OTA testing of VSATs and assess its feasibility of using existing chamber designs.

- Proposal 2 from Keysight Technologies in R4-2514328

  - Proposal 2: Define the IFF methodology as the baseline for NTN VSAT OTA testing

### Sub-topic 2-3: Quiet zone size

- Proposals:

- Proposal 3 from Vivo in R4-2513572

  - Proposal 1: RAN4 should specify a reasonable QZ for NTN VSAT testing to minimize the burden of test system upgrade.

- Proposal 2 from Samsung in R4-2513876

  - Proposal 2: RAN4 needs to discuss the radiating aperture diameter assumption and QZ size for the evaluation of minimum range length and SNR range.

- Proposal 1 from Eutelsat Group in R4-2514266

  - Proposal 1: To facilitate applicability to the most common small VSAT type it is proposed to use a 45 cm x 45 cm antenna aperture requiring a 63.6 cm QZ.

- Proposal 4 from Keysight Technologies in R4-2514328

  - Proposal 4: VSAT vendors/NTN operators to provide maximum antenna aperture for each NTN VSAT type

Summary in [2]

### Sub-topic 2-4: DUT size and weight

- Proposals:

- Proposal 3 from Keysight Technologies in R4-2514328

  - Proposal 3: VSAT vendors/NTN operators to provide the maximum VSAT device size and weight limits for each NTN VSAT type

- Moderator Recommendation:

- Inputs required

### Sub-topic 2-9: Beam peak search

- Proposals:

- Proposal 4 from Samsung in R4-2513876

  - Proposal 4:	RAN4 needs to discuss if beam peak search for VSAT testing can be skipped and only focus on the single set of measurement grid.

- Proposal 2 from Eutelsat Group in R4-2514266

  - Proposal 2: The extent of beam pointing performance testing requires further discussion as this is linked to ephemeris data and VSAT propagator performance.

- Proposal 5 from Keysight Technologies in R4-2514328

  - Proposal 5: VSAT vendors to provide the worst-case antenna assumptions and beam steering assumptions for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement angular grid spacing as well as the measurement uncertainty for BP searches, TRP, and off-axis EIRP emission density limits/gain measurements.

- Moderator Recommendation:

- Discussion required

### Sub-topic 2-01: Beampointing stability

- Proposals:

- Proposal 4 from Eutelsat Group in R4-2514266

  - Proposal 4: Pointing stability should be considered as an operational requirement of the system and not be tested in 3GPP.

- Proposal 9 from Keysight Technologies in R4-2514328

  - Proposal 9: Define the pointing stability as a system-level requirement rather than a radiated conformance test requirement as it cannot be tested in conventional OTA test systems

- Moderator Recommendation:

- Discussion required

Summary in [2]

### Sub-topic 2-13: Antenna pattern measurement

- Proposals:

- Proposal 5 from Eutelsat Group in R4-2514266

  - Proposal 5: Antenna pattern is required to be measured in the UL and DL for regulatory purposes.

- .Proposal 6 from Eutelsat Group in R4-2514266

  - Proposal 6: Due to lack of RF connectors, the antenna gain can only be measured via the L or S band modem port.

- Moderator Recommendation:

- Discussion required

### Sub-topic 2-14: Spectrum emission mask

- Proposals:

- .Proposal 7 from Eutelsat Group in R4-2514266

  - Proposal 7: The spectrum mask should be assessed at the DUT level using EIRP and not on the antenna gain.

- Proposal 14 from Keysight Technologies in R4-2514328

  - Proposal 14: NTN Operators/VSAT vendors to clarify whether these spectrum mask measurements should be based on antenna gain patterns or rather EIRP

- Moderator Recommendation:

- Use EIRP to measure spectrum emissions

Discussion

Beamlock and Beam peak search

Beamlock is used in the FR2 OTA tests in the current standard. It will help to prevent that UE may change the beam during the OTA measurement for related RF specifications. This also means to keep the beam pointing in one direction all the time.

There is no need to use beamlock for single VSET antenna test, since the boresight will always be in the same direction relatively to the physical VSET antenna. So, only the mechanical steer can move the beam. However, for the electronical beam steering VSET setup, the beamlock will be needed, since the phase of the feeding signals to the antenna (array) need to be stable enough to maintain the beam in lock status.

Beamlock is used in FR2 OTA tests in the current standard to prevent the UE from changing beams during OTA measurements for related RF specifications. This ensures that the beam remains pointed in a single direction throughout one measurement during the test.

For a single VSET antenna test, beamlock is not required, as the boresight remains fixed relative to the physical VSET antenna; only mechanical steering can move the beam. However, for electronically steered VSET setups, beamlock is necessary because the phase of the feeding signals to the antenna array must remain unchanged to keep the beam locked in the desired direction.

Beamlock is not needed for single VSET antenna tests, as the boresight remains fixed relative to the physical VSET antenna.

For electronically steered VSET setups, beamlock is required because the phase of the feeding signals to the antenna array must remain unchanged to maintain the beam in a locked position.

Use beamlock for electronically steered VSET; beamlock is not required for mechanically steered VSET.

The beam search can be skipped for mechanically steered VSET antennas, but it should not be skipped for electronically steered VSET antennas.

Beampointing stability, and antenna pattern measurement

During the beamlock period, the UE must maintain a certain level of beam pointing stability. If the stability is insufficient, test results can be affected by variations in the beam peak power during measurements. For example, the minimum peak EIRP in OTA measurements and the spherical coverage in current FR2 OTA tests may be impacted. Currently, there are no specifications defining beam pointing stability, and this aspect remains implementation-agnostic.

There is currently no specification defining beam pointing stability; this aspect remains implementation-agnostic.

However, antenna pattern measurement may help to limit such beam point stability issue. Say, if the antenna pattern is created with certain beamwidth as narrow a narrow beam, but due to the beam pointing stability, the tested antenna patterns can have a wider beam pattern, which may considered as a wide beam. The gain of the beam also impacted. So, we may consider using the beam pattern requirement to implicitly control the beam pointing stability issues.

Antenna pattern measurements may help mitigate beam pointing stability issues. For example, if a very narrow beamwidth antenna pattern is designed, insufficient beam pointing stability can result in a measured pattern that appears wider, effectively behaving as a wider beam. This also impacts the beam gain. Therefore, beam pattern requirements could be used to implicitly control and limit beam pointing stability issues.

Beam pattern requirements can be used to implicitly control beam pointing stability issues. If beam pointing stability is low, measurements of a narrow antenna pattern may appear as a wider beam pattern.

Define antenna radiation pattern requirements to implicitly constrain beam pointing stability.

OTA range type: test procedure for IFF and NFTF

It is acceptable to study both test methods. If the new NFTF test method requires physical modifications to the current TE chamber, we should assess the feasibility of reusing the existing chamber. The goal is to enable reuse for testing NTN Ka and Ku bands without causing significant delays in OTA test method deployment.

If the NFTF test methods require significant modifications to the current TE chamber, we should consider using IFF as the starting point.

Be open to new test methods to ensure OTA testing keeps pace with market needs. However, if the new methods require extensive changes to the current TE chamber system, IFF should be used as the starting point, with further study on how to adapt the new test methods to the existing test system.

Quiet zone size and DUT size

The size of the quiet zone depends on both the DUT size and the aperture of the antenna under test. Therefore, we first need to collect information on the DUT sizes to determine whether the quiet zone needs to be enlarged compared to its current dimensions.

Gather information on DUT sizes and antenna apertures to determine whether they can fit within the current quiet zone or if the quiet zone needs to be enlarged.

Spectrum emission mask

The spectrum emission mask is related to the spectrum of the transmitted signals and is independent of the antenna used for transmission. Therefore, it is not directly an OTA-related issue. However, we would like to consult with other companies to determine whether this can be considered an OTA test and to understand the underlying principle.

Consult with other companies to clarify whether the spectrum emission mask is considered an OTA-related test.

Conclusion

The following observations and proposals were made:

- Beamlock is not needed for single VSET antenna tests, as the boresight remains fixed relative to the physical VSET antenna.

For electronically steered VSET setups, beamlock is required because the phase of the feeding signals to the antenna array must remain unchanged to maintain the beam in a locked position.

- Use beamlock for electronically steered VSET; beamlock is not required for mechanically steered VSET.

The beam search can be skipped for mechanically steered VSET antennas, but it should not be skipped for electronically steered VSET antennas.

There is currently no specification defining beam pointing stability; this aspect remains implementation-agnostic.

Beam pattern requirements can be used to implicitly control beam pointing stability issues. If beam pointing stability is low, measurements of a narrow antenna pattern may appear as a wider beam pattern.

Define antenna radiation pattern requirements to implicitly constrain beam pointing stability.

If the NFTF test methods require significant modifications to the current TE chamber, we should consider using IFF as the starting point.

Be open to new test methods to ensure OTA testing keeps pace with market needs. However, if the new methods require extensive changes to the current TE chamber system, IFF should be used as the starting point, with further study on how to adapt the new test methods to the existing test system.

Gather information on DUT sizes and antenna apertures to determine whether they can fit within the current quiet zone or if the quiet zone needs to be enlarged.

Consult with other companies to clarify whether the spectrum emission mask is considered an OTA-related test.

- References

- R4-2514820, WF on NR_UE_OTA_Enh

- R4-2513534, Topic summary for [116bis][228] NR_UE_OTA_Enh