3GPP TSG-RAN4 Meeting #117	R4-2521789

Dallas, US, Nov. 17 - 21, 2025

Source:	Nokia

Title:	NR UAV Coexistence

Agenda item:	7.7.3

Document for:	Discussion

Introduction

Based on the agreement in the WF from RAN4 #116-bis meeting [1] in this paper, we’d like to present our views on the related issues of NT UAVs.

Issue 2-2-6: UAV UE antenna assumptions

- Agreements:

  - As the starting point, omni-directional is assumed for 2GHz, and phase array is assumed for 4GHz.

  - The antenna parameters for antenna array in the table below are used as the starting point.

| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| --- | --- |
| Element gain (dBi) | 5 dBi |
| Horizontal/vertical front-to-back ratio (dB) | 30dBc |
| Antenna polarization | Linear ±90º |
| Antenna array configuration (Row × Column x Polarization) | [(2x2x2)] |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |

Issue 3-2-1: Maximum output power

- Agreements:

  - RAN4 to specify maximum output power for UAV higher power UE considering the following two options:

    - Option 1: Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG)

    - Option 2: Introduce a new power class for UAV UE for up to 33dBm max total conductive output power for the example bands (Similar as legacy TN UE power class definition)

    - Option 3: Other options are not precluded.

Discussion

UAV UE Antenna Assumption and MOP

In 38.101-1, UE antenna patterns are assumed to be omnidirectional. The maximum output power (MOP) is defined based on power class—for example, PC3 to PC1 correspond to 23 dBm, 26 dBm, 29 dBm, and 31 dBm. These output powers are measured as conductive power at the antenna feed point. The RF requirements are then defined based on this setup with an omnidirectional antenna.

A question arises when using a directional antenna, which can add, for example, 10 dB in the main radiation direction. In this case, the overall transmitted EIRP is much higher than that of an omnidirectional antenna, potentially impacting RF performance.

For UAV UEs employing beam-based radiation patterns with the same MOP as an omnidirectional antenna, the impact on interference, MPR, and other RF requirements can be significant. The main-lobe beam gain increases the transmitted power in the beam direction, thereby greatly increasing interference. To address this, mechanisms such as MPR need to be studied and potentially defined separately for directional-beam UEs.

Using a beam-based antenna significantly increases the transmitted EIRP in the main-lobe direction, potentially by up to 10 dB, compared to an omnidirectional antenna.

The RF requirements should distinguish between omnidirectional and beam-based antennas, as beam-based antennas can increase the transmitted EIRP in the main-lobe direction by up to 10 dB compared to omnidirectional antennas.

Conclusion

The following observations and proposals were made:

- Using a beam-based antenna significantly increases the transmitted EIRP in the main-lobe direction, potentially by up to 10 dB, compared to an omnidirectional antenna.

- The RF requirements should distinguish between omnidirectional and beam-based antennas, as beam-based antennas can increase the transmitted EIRP in the main-lobe direction by up to 10 dB compared to omnidirectional antennas.

- References

- R4-2515159, WF on NR_UAV