3GPP TSG-RAN4 Meeting #117	R4-2521800

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR Test case for direct SCell activation with EMR in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_PH5 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | EMR based direct SCell activation requirements are tested for FR2 SCell with different UE capabilities considered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### A.7.5.3.X1	EMR based SCell activation of unknown SCell in FR2 in RRC Inactive

##### A.7.5.3.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based direct SCell activation delay is within the requirements stated in clause 8.3.2A, when the SCell in FR2 is unknown by the UE but the EMR measurements are available  at the time of activation.

In the test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as SCell in FR2 on NR RF channel 2. The supported test configurations for PCell and SCell are provided in table A.7.5.3.X1.1-1 and table A.7.5.3.X1.1-1A respectively. The general test parameters are given in table A.7.5.3.X1.1-2 and cell-specific parameters for PCell and SCell are given in table A.7.5.3.X1.1-3 and table A.7.5.3.X1.1-4 respectively.

Table A.7.5.3.X1.1-1: supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.7.5.3.X1.1-1A: supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4, and T5  respectively.

During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2. UE is configured with inter-frequency measurement reporting for Cell 2 in:

- measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or idleInactiveNR-MeasReport-r16 only, or

- measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements.

- The UE is configured with beam level reporting for early measurements.

The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

At the beginning of T2, Cell 2 becomes detectable but the cell reselection threshold is not occurring. Signal level of Cell 2 is set to the value given in table A.7.5.3.X1.1-4. The duration of T2 is set to a fixed value according to the table A.7.5.3.X1.1-2.

At the beginning of T3, the signal level of the Cell 2 is set to a value according to the table A.7.5.3.X1.1-4.

The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

During T2 and T3, UE is in RRC_IDLE mode.

The time when TE sends the paging message is defined as the starting point of T4.

During T4, the UE shall send a valid measurement report with SSB index of Cell 2 to the PCell in RRCResumeComplete message

At the beginning of T5 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2.

T5: At the beginning of T5, the UE receives RRC message for direct addition and activation of the SCell (Cell 2). The point in time at which the RRC message is received at the UE antenna connector is denoted as slot #n.

The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+Ndirect/(NR slot length) where Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3 ms (TCI state is not indicated within Tactivation_time) or

Ndirect = TRRC_Process + T1 + THARQ + Tactivation_time + TCSI_Reporting (TCI state is not indicated within Tactivation_time).

Table A.7.5.3.X1.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 FR1 |
| SCell |  | Cell 2 | Secondary cell on NR RF channel number 2 FR2 configured by RRC during the test. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | ms | 0 |  |
| Time alignment error between cell2 and cell1 | ms | £ Time alignment error as specified in TS 38.104 [13] clause 7.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of idle/inactive and reselection measurements by performing validity check, or if the measurements meet the accuracy requirements. |
| T4 | s | 1 | During this time UE shall send a valid measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 5 | During this time SCell (Cell 2) is configured via RRC signalling and the UE completes the activation of SCell (Cell 2). |
| Filter coefficient |  | 0 | L3 filtering is not used |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length, where k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k1 should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [26]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.X1.1-3: Cell specific test parameters for NR PCell

| Parameter |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  | Config 3 |  | SSB.2 FR2 |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  |  |
|  | Config 3 |  | 10 |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |  |  |
|  | Config 3 |  | 5 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -58.96 |  |  |  |  |
|  | Config 3 | dBm/ 19.08 MHz | -52.87 |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 [26] clause 5.2.2.5. |  |  |  |  |  |  |  |

Table A.7.5.3.X1.1-4: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |  |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |  |  |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR2 |  |  |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR2 |  |  |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |
| reportConfigType |  |  | N/A |  |  |  |  |
| reportQuantity |  |  | N/A |  |  |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |  |  |
|  |  | dB | -infinity | 17 | 7 |  |  |
|  |  | dB | -infinity | 17 | 7 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -infinity | -87 | -97 |  |  |
|  | ConfigSCell 3 |  | -infinity | -84 | -94 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -infinity | -87 | -97 |  |  |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36 MHz | -76.05 | -58.96 | -68.26 |  |  |
|  | ConfigSCell 3 | dBm/ 19.08 MHz | -69.96 | -52.87 | -62.17 |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |  |  |

##### A.7.5.3.X1.2	Test Requirements

The UE shall complete the SCell activation no later than at slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

The UE shall report non-zero CQI for SCell from slot n + THARQ+Tactivation_time+TCSI_ReportingNR slot length and onwards throughout time period T5.

Tactivation_time  is defined in clause 8.3.2A in TS 38.133 for FR2 SCell as Tactivation_time is TFirstSSB_MAX + 15*TSMTC_MAX  + 5ms

The observed SCell activation delay fulfiiling the SCell activation delay requirements specified in clause 8.3.2A in TS 38.133 is counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

==============End of change==============