3GPP TSG-RAN WG4 Meeting #117	R4-2521817

Dallas, US, Nov. 17 – 21, 2025

Source: 	ZTE Corporation, Sanechips

Title: 	Considerations on 6G RAN4 UE & BS RF specification enhancement

Agenda item:	8.12.2.1

Document for:	Approval

- 1 Introduction

An SID on 6G Radio was approved and further revised in [1] to initiate the study on developing a 6G radio access technology. RAN4 6G topic planning based on 6G SID was provided in [2].

Agreement:

- By taking the identified issues in the feature lead’s summary into consideration, in RAN4#117, a dedicated singe sub-agenda item will be introduced for the spec improvement. Interested companies are encouraged to structure their contributions such that the spec specific proposals and the general proposals for all RAN4 spec can be separately presented.

- Interested companies are encouraged to continue populating the list of identified issues to be addressed in 6G as necessary.

- A dedicated sub-agenda will be introduced on CR handling improvement from RAN4#117

A special thread [116bis][111] was set for RAN4 6G operation efficiency in RAN4#116bis. During the meeting, companies provide their initial views on the topic for 6GR RAN4 operation efficiency [3] and a WF was approved in [4] with the following agreement.

In this contribution, we give our further considerations on UE spec improvement.

- 2	Discussion

Issue 1-1-2: Issues for UE RF specifications

- Observed issues

  - Issue 1: framework

    - The latest release requirements are the superset of all previous releases.

    - NR spec mixes mandatory and optional feature requirements in one spec

  - Issue 2: band combinations

    - Huge band combination tables in MS Word format.

    - Tens of thousands of band combinations have been specified in UE specifications.

  - Issue 3: Suffix approach

    - For TS 38.101-1, suffix-ed requirements for different features make specs complex.

    - Rules on applicability of suffix requirements and general requirements are not clear or not strictly followed.

    - Different approaches are adopted for different features in the structure; too many undefined (sub-)clause suffixes.

    - Too many (sub-)clause suffixes not defined for RF requirements.

  - Issue 4: Readability

    - Poor readability caused by recursively 3rd-level heading in UE RF specs.

    - Some RF requirements are quite complicated and difficult to understand.

    - Readability not good enough for engineers who only want to implement mandatory features and making maintenance difficult.

In last meeting, the following issue on UE RF specifications was observed during the previous 5G RAN4 standardization activities and may provide valuable insights for ongoing discussions.

2.1	UE specification improvement

2.1.1	UE RF spec structure

Regarding to the structure of UE RF spec in NR, it is observed that the structure is organized by the requirements with different features packing into the second level sub-clauses. The features are currently defined in the spec with the suffixes shown as Table 2.1.1-1. For shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause. For V2X, suffixes A and F are used for indicating at 3rd level clause.

Fig 2.1.1-1 Spec structure with feature example

Table 2.1.1-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |
| M | LP-WUS/WUR |

In the current UE RF spec, only chapters 1~4 are common to all features in the spec, while in other chapters, they are recursively categorized into second or third level features. If we take 38.101-1 chapter 6 as an example, for “General” clause, it consists of more than 9 sub-clauses with “General” even for reserved sub-clause, which looks quite strange and causes poor readability.

Observation 1: In 5G UE RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. However, there are also some second level features further recursively categorized into 3rd level features. This causes the spec poor readability.

Proposal 1: In 6GR UE RF spec, the recursive multi-level feature sub-clauses should be avoided.

Fig 2.1.1-2  Illustration of zip file structure for TS 38.101-1

Fig 2.1.1-2 illustrates the zip file structure for TS 38.101-1. The sub-files packaged in the zip file for the spec are out of order. If we assign a serial number to these sub-files, the readability of the specification will be improved.

Observation 2: In 5G UE RF spec, currently the sub-files packaged in the zip file are out of order. If we assign a serial number to these sub-files, the readability will be improved..

Proposal 2: In 6GR UE RF spec, if multiple sub-files are packaged in the zip file, a serial number to these sub-files are suggested.

To optimize the specification structure, we notice that one of the main issue affects the readability of the spec is that the requirements are distributed by large number of features. If we want to check some requirements with a certain feature, supposed to check the basic feature for single carrier, we have to open all the sub-files in the zip file and try to find the requirements out of different features since the feature is distributed into different sub-files which belongs to different sub-clauses. To optimize the structure in 6GR, if we re-organize the zip file by features instead of by the clauses, then the requirements for a certain feature within a sub-file will be an independent part. In each sub-file, if a feature is further needed to be categorized into different sub-features, then a second level sub-clause can be setup. Figure 2.1.1-3 illustrates the above optimization concept.

By introducing the above mentioned optimized structure for RF spec, the structure will be more concise. For the first level clause, the feature will be reflected in the sub-file suffice. For example, if the constituent sub-file name is “TS 38.101-1_A.doc”, it means this sub-file specifies the CA specific requirements. In the first level clause within the sub-file, there is no need to repeat the feature suffice since all the requirements specified in this sub-file belongs to the same feature. Thus the structure of the sub-file is simplified. If for the feature such as “Transmitter power for shared spectrum channel access” which has further sub-feature “for CA”, “for NR-DC” and “for UL MIMO”, then a second level of sub-clause with suffix “A”, “B” and “D” will be used. With that, in most cases, in a sub-file there is no need to have a suffix for feature. The suffix for feature will be limited to one level. Note that the common part is for all features and it will be specified in a separate file.

Proposal 3:  For the structure of 6GR UE RF specification, it can be optimized with the guidelines as below.

- Re-organize the specification zip file by the features, each of the constituent sub-file specifies a certain feature, such as CA, DC, Redcap etc.

- A common sub-file for the basic aspects will be specified for all features.

- In each sub-file for a certain feature, the clauses could be further specified with a second level sub-clause to reflect the requirements of the feature if needed.

Fig 2.1.1-3  Illustration of optimized structure for TSxx.101-1 in 6GR

2.1.2	UE RF requirements for different features

In TR 38.846 [5], during the study of simplification for band combination there are some analysis on Tx/Rx RF requirements among different features. Currently, the main RF requirements related to specific band combinations include maximum output power (MOP), spurious emission for UE-to-UE coexistence, REFSENS and REFSENS exceptions due to harmonic/harmonic mixing/cross band isolation/IMD interference. Obviously, RF requirements for different features on the same frequency range combination have some similarities and dependencies. Especially, the RF implementations are similar and RF architectures can be reused for different features on the same frequency range combination. It can be seen that CA_nA-nB (NR CA), DC_nA-nB (NR-DC), DC_A_nB (EN-DC), DC_B_nA (EN-DC), DC_nB_A (NE-DC), DC_nA_B (NE-DC) (different features on same frequency range combination) could use same RF implementation.

2.1.2.1	Tx requirements for different features

- Maximum output power

Table 2.1.2.1 illustrates CA/DC band combination PC3 MOP requirements (TS 38.101-1 v19.3.1 & 38.101-3 v19.3.0) for the same frequency range “n1A-nXA” as an example. From the table, it can be seen that most of the combinations can be specified in the common part spec, for example for frequency range combination “n1A-nXA”, in current NR spec there will be 7 entries in the different feature part. However, if we use the spec structure as proposed in Proposal 3, only one entry is needed in the common part spec, which largely reduces the redundancy in 6GR spec. In the table, we see only the greyed rows need to be specified in the feature specific part (“n1-n18”, “21_n1”, “1_n38”, “1_n50”, “1_n51”, “68_n1”, ”n1-n74”, “n1-n105”).

Table 2.1.2.1-1: PC3 MOP requirements for the same frequency range with different features

| Uplink NR CA Configuration | Uplink NR DC Configuration | Uplink EN-DC Configuration | Uplink NE-DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | DC_n1A-n3A | DC_1A_n3A DC_3A_n1A DC_1A_n80A DC_3A_n84A | DC_n3A_1A | 23 | +2/-3 |
| CA_n1A-n5A |  | DC_1A_n5A |  | 23 | +2/-3 |
| CA_n1A-n7A | DC_n1A-n7A | DC_1A_n7A DC_7A_n1A |  | 23 | +2/-3 |
| CA_n1A-n8A |  | DC_1A_n8A DC_8A_n1A | DC_n8A_1A | 23 | +2/-3 |
| CA_n1A-n18A |  |  |  | 23 | +2/-3 |
|  |  | DC_19A_n1A |  | 23 | +2/-3 |
| CA_n1A-n20A | DC_n1A-n20A | DC_1A_n20A DC_20A_n1A |  | 23 | +2/-3 |
|  |  | DC_21A_n1A |  | 23 | +2/-3 |
| CA_n1A-n26A |  | DC_1A_n26A |  | 23 | +2/-3 |
| CA_n1A-n28A | DC_n1A-n28A | DC_1A_n28A DC_28A_n1A | DC_n1A_28A | 23 | +2/-3 |
|  |  | DC_1A_n38A DC_38A_n1A |  | 23 | +2/-3 |
| CA_n1A-n40A |  | DC_1A_n40A DC_40A_n1A |  | 23 | +2/-3 |
| CA_n1A-n41A | DC_n1A-n41A | DC_1A_n41A DC_41A_n1A |  | 23 | +2/-3 |
|  |  | DC_42A_n1A |  | 23 | +2/-3 |
| CA_n1A-n46A | DC_n1A-n46A |  |  | 23 | +2/-3 |
|  |  | DC_1A_n50A |  | 23 | +2/-3 |
|  |  | DC_1A_n51A |  | 23 | +2/-3 |
|  |  | DC_68A_n1A |  | 23 | +2/-3 |
| CA_n1A-n71A |  | DC_1A_n71A |  | 23 | +2/-3 |
| CA_n1A-n74A |  |  |  | 23 | +2/-3 |
| CA_n1A-n77A | DC_n1A-n77A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | DC_n77A_1A | 23 | +2/-3 |
| CA_n1A-n78A | DC_n1A-n78A | DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | DC_n78A_1A | 23 | +2/-3 |
| CA_n1A-n79A | DC_n1A-n79A | DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
| CA_n1A-n102A | DC_n1A-n102A |  |  | 23 | +2/-3 |
| CA_n1A-n105A |  |  |  | 23 | +2/-3 |
| … | … | … | … | … | … |

- Spurious emission for UE-to-UE coexistence

Generally, if frequency range combination have same spurious emission requirements for UE to UE coexistence, there is no need to specify them is different feature parts. Once an uplink frequency range combination is verified for one feature, the combination for other features could also be considered as meeting the requirements. Thus, if the spurious emission requirements for different features with the same frequency range, it can be specified in the common part spec.

Observation 3: In 5G UE RF spec, some Tx requirements for the same frequency range with different features are duplicated in different sub-clauses even if the requirements are the same.

Proposal 4: In 6GR UE RF spec, it is proposed to merge the same Tx requirements for multiple features and specified in the common part spec.

2.1.2.2	Rx requirements for different features

- REFSENS exceptions due to 2UL inter-modulation

For inter-band frequency range combination, the MSD due to IMD for NR CA, NR DC, EN-DC and NE-DC are shown in Table 2.1.2.2-1 based on TS 38.101-1 v19.3.1 & 38.101-3 v19.3.0 for “n1-nX” as an example. Similar to the Tx requirements, it can be seen from the table that most of the combinations can be specified in the common part spec. In this “n1-nX” case, only the greyed row need to be specified in the CA feature spec part.

Table 2.1.2.2-1: REFSENS exceptions due to 2UL inter-modulation for the same frequency range with different features

Observation 4: In 5G UE RF spec, some Rx requirements for the same frequency range with different features are duplicated in different sub-clauses even if the requirements are the same.

Proposal 5: In 6GR UE RF spec, it is proposed to merge the same Rx requirements for multiple features and specified in the common part spec.

2.2	BS specification improvement

In the Rel-19, RAN tasked RAN4 to build the framework for the simplification for co-existence and co-location requirement for RAN4 BS specification as listed in Table 2.2-1 in order to reduce the lots of duplicated CRs for any new NR/LTE bands introduction [6].

Table 2.2-1: The list of BS specifications

| Specification (TS) |  |
| --- | --- |
| 38.104 | NR BS Core |
| 38.141-1 | NR BS Conf - Conducted |
| 38-141-2 | NR BS Conf - OTA |
| 38.106 | NR Repeater - Core |
| 38.115-1 | NR Repeater -Conf Cond. |
| 38.115-2 | NR Repeater -Conf OTA No impact |
| 38.174 | IAB Core |
| 38.176-1 | IAB Conf Conducted |
| 38.176-2 | IAB Conf OTA |
| 37.105 | AAS BS Core |
| 37.145-1 | AAS BS Conf Conducted |
| 37.145-2 | AAS BS Conf OTA |
| 37.104 | MSR BS Core |
| 37.141 | MSR BS Conf |
| 36.104 | E-UTRA BS Core |
| 36.141 | E-UTRA BS Conf |
| 36.106 | E-UTRA Repeater Core |
| 36.143 | E-UTRA Repeater Conf |

Proposal 6: It is proposed to leverage the Rel-19 RAN task for the simplification for co-existence and co-location requirements for 6GR BS specification.

In addition, in the 6G phase regardless the support of these network node in 6G day1 or day2, the various network nodes are also expected to be specified similar as 5G NR, then if we still follow the legacy approach to define the RF requirements for different network nodes in the different specifications, then it would also face the similar situation as 5G NR BS specification e.g. duplicated information for TRP measurement, EVM measurement, test mode/configuration and test procedures across the different specifications, in order to simplify the workload and maintenance burden for the BS specification, we also propose to consider how to capture the same requirements or test procedures across different network nodes if there are many similarities just with some item/notation difference.

Proposal 7: It is proposed to discuss how to capture the same requirements (e.g. TRP measurement, EVM measurement, test mode/configuration, OTA test chamber) or test procedures across different network nodes specifications if there are many similarities just with some items/notation difference.

2.3	Other aspects related to operation efficiency

2.3.1	New tools for band combination

In 5G, it is highlighted that tens of thousands of band combinations have been specified in UE specifications. The combinations are inefficiently stored in the tables in Word files. At RAN#102, it was decided to develop a new tool for handling CA band combinations within RAN4.

To offload the burden of huge numbers of band combination, it is decided in RAN4 that the CA/EN-DC/NE-DC band combinations will be transitioned from Docx table to band combination database (https://forge.3gpp.org/rep/ran4/carrieraggregation) in Rel-20.

Proposal 8: It is suggested that the band combinations in 6GR follow Rel-20 5G not to store the band combinations in the specifications but store in the database.

Currently in 5G, there are some guidelines on requesting new band combinations. The proponents should propose all the necessary fallback modes together with the proposed band combinations. All the next level fallback configurations shall be reported and the status of all the fallback configurations shall be declared. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination. A band combination configuration can only be considered as completed when all of the fallback configurations are completed. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Although rapporteurs and other companies are encouraged to check the status of all fallback configurations, there are still many fallback band combinations missing in the specification. To check the missing fallback band combinations is a quite time-consuming task. If an automatic checking fallback tool can be developed, the RAN4 efficiency of band combination will be greatly improved.

Proposal 9: In 6GR, it is proposed to develop an automatic checking tool for fallback band combinations in RAN4.

2.3.2	About spec framework

In last meeting, it is highlighted that the following spec framework is valuable for follow-up discussion.

  - Issue 1: framework

    - The latest release requirements are the superset of all previous releases.

    - NR spec mixes mandatory and optional feature requirements in one spec

For the spec release framework, in our understanding if we separate the 6GR specification into a series of feature-based sub-files, the latest release requirements should be the superset of all previous releases within the scope of the certain feature. In other words, the requirements for a certain feature is self-contained. Regarding to the issue of mandatory or optional feature requirements, it is not feasible to separate them into different specs since it will cause tremendous complexity in 6GR spec.

Proposal 10: If the 6GR specification is decided to categorize into a series of feature-based sub-files, from the perspective of a certain feature, it should keep the latest release requirements as the superset of all previous releases within the feature. It also applies to the common part sub-file for all feature.

Proposal 11: It is not suggested to separate mandatory and optional feature requirements in 6GR unless a reasonable solution is found.

Another issue related to spec framework is about how to develop the 3GPP spec. For four generations, 3GPP specifications have been developed using Microsoft Word with the guidelines outlined in TR 21.801 across different working groups. However, due to the increasing number of specifications, increasing number of Change Requests (CRs) and increasing specification files sizes, the handling of specifications is becoming more and more difficult which leads to low efficiency in readability.

To ensure a more reliable, modern and efficient approach to the new Generation specifications in 3GPP, an SID was established for the study in 6G. The purpose of the SI is to assess the existing specification formats, working methods, and requirements for any improvements.

Observation 5: Recently some efforts have been made in 3GPP to check if it is feasible to use

-    Markdown as the tool for specifying specifications.

-    LaTeX as the tool for writing formulas.

Although the final decision on whether using Markdown or LaTeX in 6GR will be decided by SA/RAN plenary, it is suggested RAN4 to study the possible influence to RAN4. It should be noted that, in NR spec there are many tables having complex format. We should consider in RAN4 how to handle the complex table if 3GPP decide to use a new tool to develop the spec.

Proposal 12: Although final decision on whether using Markdown or LaTeX in 6GR will be decided by SA/RAN plenary, it is suggested RAN4 to study the possible influence to RAN4. At least how to handle the complex table format in the spec should be considered in RAN4.

- 3	Conclusion

In this contribution, we give some further considerations on the 6GR RAN4 operation efficiency. Issues on UE RF spec structure, RF requirements for different features and other aspects related to operation efficiency have been discussed. The conclusions are summarized as below:

Observation 1: In 5G UE RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. However, there are also some second level features further recursively categorized into 3rd level features. This causes the spec poor readability.

Proposal 1: In 6GR UE RF spec, the recursive multi-level feature sub-clauses should be avoided.

Observation 2: In 5G UE RF spec, currently the sub-files packaged in the zip file are out of order. If we assign a serial number to these sub-files, the readability will be improved..

Proposal 2: In 6GR UE RF spec, if multiple sub-files are packaged in the zip file, a serial number to these sub-files are suggested.

Proposal 3:  For the structure of 6GR UE RF specification, it can be optimized with the guidelines as below.

- Re-organize the specification zip file by the features, each of the constituent sub-file specifies a certain feature, such as CA, DC, Redcap etc.

- A common sub-file for the basic aspects will be specified for all features.

- In each sub-file for a certain feature, the clauses could be further specified with a second level sub-clause to reflect the requirements of the feature if needed.

Observation 3: In 5G UE RF spec, some Tx requirements for the same frequency range with different features are duplicated in different sub-clauses even if the requirements are the same.

Proposal 4: In 6GR UE RF spec, it is proposed to merge the same Tx requirements for multiple features and specified in the common part spec.

Observation 4: In 5G UE RF spec, some Rx requirements for the same frequency range with different features are duplicated in different sub-clauses even if the requirements are the same.

Proposal 5: In 6GR UE RF spec, it is proposed to merge the same Rx requirements for multiple features and specified in the common part spec.

Proposal 6: It is proposed to leverage the Rel-19 RAN task for the simplification for co-existence and co-location requirements for 6GR BS specification.

Proposal 7: It is proposed to discuss how to capture the same requirements (e.g. TRP measurement, EVM measurement, test mode/configuration, OTA test chamber) or test procedures across different network nodes specifications if there are many similarities just with some items/notation difference.

Proposal 8: It is suggested that the band combinations in 6GR follow Rel-20 5G not to store the band combinations in the specifications but store in the database.

Proposal 9: In 6GR, it is proposed to develop an automatic checking tool for fallback band combinations in RAN4.

Proposal 10: If the 6GR specification is decided to categorize into a series of feature-based sub-files, from the perspective of a certain feature, it should keep the latest release requirements as the superset of all previous releases within the feature. It also applies to the common part sub-file for all feature.

Proposal 11: It is not suggested to separate mandatory and optional feature requirements in 6GR unless a reasonable solution is found.

Observation 5: Recently some efforts have been made in 3GPP to check if it is feasible to use

-    Markdown as the tool for specifying specifications.

-    LaTeX as the tool for writing formulas.

Proposal 12: Although final decision on whether using Markdown or LaTeX in 6GR will be decided by SA/RAN plenary, it is suggested RAN4 to study the possible influence to RAN4. At least how to handle the complex table format in the spec should be considered in RAN4.

- 4	Reference

[1]	RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] 	R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair

[3]  R4-2514518, Topic summary for [116bis][111] 6G operation efficiency, Moderator

[4] 	R4-2514648, Way forward for [116bis][111] 6G operation efficiency, CATT

[5]  TR 38.846, Study on simplification of band combination specification for NR and LTE

[6] 	R4-2504741, Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex