3GPP TSG-RAN4 Meeting #117						                  R4-2521846
Dallas, Texas, USA, 17 November – 21 November 2025

Source:	Ericsson

Title:	6G spectrum and band combination considerations

Agenda item:	    		8.5

Document for:	Approval

# Introduction

This contribution provides views on 6G spectrum considerations and on band combination improvements.

# Spectrum considerations

## Frequency range

In last RAN4#116bis meeting, RAN4 had some initial discussions on the definitions of the Frequency Range(s) for 6G. Several options were proposed, from defining one unique range from 410 MHz to 52.6 GHz to extending the existing ranges FR1/FR2 and introducing a new FR3 frequency range.

To facilitate the discussion, RAN4 defined ([4]) a list of criteria to converge on the frequency ranges definition:

In the scope of the recent IMT study in the 7125-8400 MHz / 14.8-15.35 GHz ([3]), RAN4 assumed that UE RF architecture principles used in FR1 will also be applicable to the 7.125-8.4 GHz frequency range, UE would not support large uniform  rectangular phased array type of antenna.  The size of the panels of this type of antenna at such frequency would be too big to be implemented in a handheld UE design. The same assumption was also taken in the previous IMT study ([2]), where RAN4 studies the frequency range 10.0-10.5GHz.

As the UE RF type of architecture would not change, at least up to 10.5 GHz, the currently specified RRM requirements for FR1 would still be applicable up to 10.5 GHz.

Also, up to 10.5 GHz, BS type 1-H type of architecture would still be considered, especially after that RAN4 agreed to study further enhancement of this type 1-H by considering hybrid beamforming type of architecture (added to BS RF 6G SI scope in last RAN4 meeting ([5]).

For those reasons, we would propose to extend the current upper limit of the FR1 frequency range from 7.125 GHz to 10.5 GHz (at least), meaning that, in the 6G scope, FR1 frequency range should be defined from 410 MHz up to 10.5 GHz.

Proposal 1: Consider extending the FR1 frequency range upper limit at least up to 10.5 GHz.

Above 10.5 GHz and below 24.25 GHz (currently FR2 lower limit), to our knowledge, only the 14.8-15.35 GHz frequency range is currently considered for IMT allocation (WRC-27 agenda item). In the 10.5-15.35 GHz frequency range, it looks again particularly challenging considering UE design with phased array antenna as captured in TR 38.922 ([3]). Still, if different types of UE architectures (e.g. FWA with large uniform phased array antenna) might co-exist in this frequency range, RAN4 should first check how RRM requirements (FR1 and/or FR2) would apply in this frequency range before taking any conclusion on extending FR1 higher up, extending FR2 down or creating a new frequency range FR3.

Anyway, RAN4 should not reproduce what was done with NTN and the introduction of the Ku-bands. Due to the need of supporting both FR1 and FR2 numerologies, those bands were specified both in FR1 and FR2, with the consequences that FR1-NTN was extending up to 14.5 GHz and FR2-NTN was extending down to 10.7 GHz. This had some major drawbacks in RRM where notes were introduced to clarify that current FR1 requirements are also applicable to FR2. This was considered as acceptable in the context of NTN Ku-bands for a quick fix considering that only a small subset of requirements was defined. But similar approach shall not be reproduced for 6G if all RRM FR1/FR2 requirements could be impacted  by handling a frequency range overlapping FR1 and FR2. If any such overlapping frequency range would be needed for 6G, it would be preferable to consider a new FR3 frequency range instead, corresponding to this overlapping frequency range, and specify then new RRM requirements for that frequency range.

Proposal 2: In case of RAN4 identifies the need for an overlapping of FR1 and FR2 frequency ranges, RAN4 should instead consider introducing a new frequency range FR3 corresponding to that overlapping range.

With NR, the FR2 frequency range has been defined from 24.25 GHz up to 71 GHz, sub-dividing this range in 2 sub-ranges: FR2-1 (from 24.25 GHz up to 52.6 GHz) and FR2-2 (from 52.6 up to 71 GHz). For the time being, there is no band targeted above 71 GHz in the 6G scope. There is so no need then to extend the upper FR2 limit or define a new frequency range above 71 GHz for the time being.

Most likely, the lower limit of FR2 frequency range could be extended, assuming that UE and BS with FR2 type of architecture would also be considered in frequencies lower than 24.25 GHz. The FR2 RRM requirements, which are based on beam sweeping factor, would still be applicable then. But it remains unclear down to which frequency the FR2 range could be extended down.

Proposal 3: Further study if FR1 should further extend up to 15.35 GHz, if FR2 should be extended down to 10.5 GHz or if a new frequency range FR3 shall be specified.

## Band naming

In last RAN4#116bis meeting, RAN4 made the following agreement on band naming convention:

We think it was really key to keep the same “concept” that was used for LTE and NR, keeping the same band numbering between RATs to avoid any very messy situation where the band number would have a different frequency range definition depending on the context, which RAT is considered.

Regarding the prefix to be used to distinguish between LTE, NR and 6G bands, we don’t have a strong opinion as long as this prefix is kept very short. We would suggest then using the prefix “s”, one could consider it would stand for “sixth generation”.

Proposal 4: Use the “s” prefix to differentiate 6G bands from LTE/NR bands (keeping the same band numbering between RATs).

## Regulatory Survey

In the scope of the 7-24 GHz frequency range study, RAN4 made a regulatory landscape survey, captured in TR 38.820 ([1]). Since then, WRC-23 concluded on the allocation of the 10.0-10.5 GHz frequency range to IMT (in Region 2). Except from this, no other update of interest for RAN4 has been made in ITU.

As the 6G study is supposed to conclude by RAN#116 (June 2027), RAN4 should not expect any outcome from WRC-27 in that timeframe (WRC-27 dates is scheduled October 18- November 12, 2027). It would then be premature to study again the regulatory landscape in 4-24GHz, RAN4 would miss some key information. RAN4 should better wait for the conclusions of WRC-27 or if, in the meantime, an Administration decides to early regulate any frequency range in the 7-24 GHz.

Observation 1: It would be premature to investigate further regulations in the 7-24 GHz frequency range in the scope of the 6G study.

Proposal 5: Postpone further regulatory survey in the 7-24 GHz frequency range until Regulators published a new decision.

# Band Combination considerations

## Improvement of MSD handling

In 5G MSD values are nowadays chosen based on existing similar (close) combinations, and in the cases where similar combinations don´t exist new MSD values are typically recommended by UE vendors. For HPUE it has now been agreed to use LUT (Look-Up Tables) to derive HPUE MSD values from each band combination’s existing PC3 MSD values. However, the existing PC3 MSD values used as base to derive new values and for the look-up tables are not consistent and stringent between EN-DC and NR CA, and/or between similar/close band combinations.

It would be good for 6G to consider better guidance on how to set the PC3 MSD baseline values using values and suggestion in a table format. Co-existence studies still need to be performed for 2DL and 3DL combinations, but actual MSD values chosen for intermodulation, harmonic and crossband issues found could use some more rules and guidance.

More strict guidance and rules can be achieved by designating bands into well-defined band groups. The split into defined band groups could be lower low bands (600-800), upper low bands (800-1000), lower mid bands (1400-1700), mid bands (1700-2200), upper mid bands (2200-2700), lower high bands (3300-4200) and upper high bands (4400-7200).

Proposal 6: Study how to improve and simplify selection of MSD values based on a proposed band group concept of lower low bands (600-800), upper low bands (800-1000), lower mid bands (1400-1700), mid bands (1700-2200), upper mid bands (2200-2700), lower high bands (3300-4200) and upper high bands (4400-7200)

## BCS concept

In retrospect It would have been preferable if we in 5G used EN-DC type of configuration tables format also for NR CA. If BCS’s (Bandwidth Combination Sets) were not used for NR CA it would have saved us a lot of work; overly complex, unreadable and sometimes faulty configuration tables, missing fallbacks, etc.

Not using BCS’s would mean that NR CA combinations are assumed to support all channel BW’s defined for each of the bands in the band combinations. This can create potential issues when new channel BW’s are added for existing bands after a band combination is defined. But in practice, this has not been seen as a major issue and has also worked well for EN-DC.

For intra-band it is proposed to instead of BCS concept introduce a max value for channel BW.

Proposal 7: Don’t specify BCS’s and specific channel BW’s per band for 6G Intra-band and Inter-band combinations. Introduce a maximum channel BW value for intra-band combinations.

## Transfer of existing 5G combos

To save many years of work already spent defining band combinations, there is a point in transferring existing 5G NR CA band combinations directly into 6G.

Since the BCS concept is to be avoided in 6G, at least the existing BCS 4 and 5 combinations could be transferred into 6G. But it also needs be studied how and if transitions can be done for band combinations not yet defined as BCS 4 and 5 and where additional studies and co-existence checks might be needed before such a transfer.

Proposal 8: Study how existing NR CA band combinations can be transferred into a 6G specification baseline

## JSON database

Proposal 9: JSON database is to be used instead of configuration tables for all 6G band combinations.

## Procedural improvements

Block approval of PC3 band combinations contributions are recommended to be continued. However, it is preferred that HPUE baskets are also aligned with this way of working.

It would also be beneficial if there are considerations also to merge HPUE baskets WI’s with the relevant PC3 baskets WI’s.

Observation 2: Discuss further how to align HPUE and PC3 work items and the meeting procedurals for them.

## Potential removal of ΔTIB and ΔRIB values

It can be questioned if the definition of ΔTIB and ΔRIB insertion loss values is still needed and whether these values are really used for UE testing.

Also, there are no rules on how to define them, and they are typically selected based on the RF architecture considered at the time of when the combination was introduced. Since these assumptions have changed over time with the introduction of new front-end components, the currently defined ΔTIB and ΔRIB are not aligned.

Proposal 10: Check the relevance of continue to define the ΔTIB and ΔRIB insertion loss values.

# Conclusion

Considerations on 6G spectrum were provided with the following observations and proposals.

Observation 1: It would be premature to investigate further regulations in the 7-24 GHz frequency range in the scope of the 6G study.

Observation 2: Discuss further how to align HPUE and PC3 work items and the meeting procedurals for them.

Proposal 1: Consider extending the FR1 frequency range upper limit at least up to 10.5 GHz.

Proposal 2: In case of RAN4 identifies the need for an overlapping of FR1 and FR2 frequency ranges, RAN4 should instead consider introducing a new frequency range FR3 corresponding to that overlapping range.

Proposal 3: Further study if FR1 should further extend up to 15.35 GHz, if FR2 should be extended down to 10.5 GHz or if a new frequency range FR3 shall be specified.

Proposal 4: Use the “s” prefix to differentiate 6G bands from LTE/NR bands (keeping the same band numbering between RATs).

Proposal 5: Postpone further regulatory survey in the 7-24 GHz frequency range until Regulators published a new decision.

Proposal 6: Study how to improve and simplify selection of MSD values based on a proposed band group concept of lower low bands (600-800), upper low bands (800-1000), lower mid bands (1400-1700), mid bands (1700-2200), upper mid bands (2200-2700), lower high bands (3300-4200) and upper high bands (4400-7200)

Proposal 7: Don’t specify BCS’s and specific channel BW’s per band for 6G Intra-band and Inter-band combinations. Introduce a maximum channel BW value for intra-band combinations.

Proposal 8: Study how existing NR CA band combinations can be transferred into a 6G specification baseline

Proposal 9: JSON database is to be used instead of configuration tables for all 6G band combinations.

Proposal 10: Check the relevance of continuing to define the ΔTIB and ΔRIB insertion loss values.

# References

- TR 38.820, Study on the 7 to 24 GHz frequency range for NR

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz

- R4-2514626, WF on 6G Spectrum, Feature lead (Nokia)

- R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)