3GPP TSG-RAN4 Meeting #117	R4-2521849

Dallas, Texas, USA, 17 November – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR 38.101-3 correcting EN-DC intra-band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correcting EN-DC intra-band combinations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting Notes for DC_(n)71AA Since n66 is not a TDD band, removing the TDD Note 4 from DC_66A-(n)66AA and DC_66A-(n)66AA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corrections are not made |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3B, 5.5A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (note 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)3AA DC_(n)3CA | DC_(n)3AA6 | Yes6 |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)7AA | DC_(n)7AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA | DC_(n)38AA6 | Yes6 |
| DC_(n)40AA7 | DC_(n)40AA6,7 | Yes3,6 |
| DC_(n)41AA DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_(n)41AA | Yes3 |
| DC_(n)48AA | DC_(n)48AA6 | Yes6 |
| DC_48A-(n)48AA | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA | DC_(n)48AA6 | Yes6 |
| DC_(n)48DA | DC_(n)48AA6 | Yes6 |
| DC_(n)66AA | DC_(n)66AA6 | Yes |
| DC_66A-(n)66AA | DC_(n)66AA6 | Yes6 |
| DC_(n)71AA | DC_(n)71AA10 | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 5: For TDD bands, the minimum requirements only apply for non-simultaneous Rx/Tx between all carriers. NOTE 6: Only single switched UL is supported NOTE 7: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.1-1 is applicable to the same EN-DC configuration. NOTE 8: The PC1.5 Uplink EN-DC configuration supported in Table 6.2B.1.1-1 is applicable to the same EN-DC configuration. NOTE 9: The UE does not include the intraBandENDC-Support and intraBandENDC-Support-UL (absent) for these configurations if supported. NOTE 10: Uplink DC_(n)71AA is not applicable to BCS2. |  |  |

---Text omitted---

Table 5.3B.1.3-2: EN-DC configurations and bandwidth combination sets defined for mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration /Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3CA5,6 | DC_3A_n3A2 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25 |  | 50 | 0 |
|  |  |  | 10, 20, 25 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)3CA7 | DC_(n)3AA2 DC_3A_n3A2 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25 |  | 50 | 0 |
|  |  |  | 10, 20, 25 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48CA5,6 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5,6 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48A-(n)48AA5 | DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_66A-(n)66AA5 | DC_66A_n66A2 | See CA_66A-66A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5,10,15,20, 25, 30, 40 |  | 80 | 0 |
|  |  |  | 5,10,15,20, 25, 30, 40 | See CA_66A-66A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_66A-(n)66AA7 | DC_(n)66AA2 DC_66A_n66A2 | See CA_66A-66A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5,10,15,20, 25, 30, 40 |  | 80 | 0 |
|  |  |  | 5,10,15,20, 25, 30, 40 | See CA_66A-66A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Rx/Tx between all carriers. NOTE 5: The UE supporting these configurations indicates intraBandENDC-Support-UL = ‘non-contiguous’ with intraBandENDC-Support absent. NOTE 6: The minimum requirements also apply for the intra-band non-contiguous fallback resulting from releasing an Scell within the sub-block bandwidth of the downlink configuration. NOTE 7: The UE supporting these configurations indicates intraBandENDC-Support-UL = ‘both’ with intraBandENDC-Support absent. |  |  |  |  |  |  |

---End of changes---