# 3GPP TSG RAN WG4 Meeting #117 														           R4-2521857

# Dallas, USA , November 17th – 21st, 2025

Agenda Item:	4.19.3

Source: 	Ericsson

Title:	Core maintenance for Rel-19 Subband full duplex evolution

Document for:	Discussion

# Introduction

The agreements on SBFD operation during RAN4#116 bis meeting are captured in the approved WF [1].

This contribution discusses our views on the two LSs regarding supporting SBFD and CA operation in [2] and DC operation in [3].

# Discussion

## 2.1 Core requirements maintenance for SBFD operation

Based on our understanding from the RAN2 LS to RAN1 in [2], currently there is no specification in stage-2 TS38.300 that states that SBFD operation with CA is supported.

Also based on the LS [2], RAN2 have not reached a conclusion on how to capture SBFD operation with CA in stage-2 specification.

RAN2 has discussed stage-2 specification impact to support SBFD with CA according to the agreement at RAN1 #120. Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case. However, RAN2 has not reached a conclusion how to capture SBFD operation with CA in the stage-2 specification (TS 38.300).

- Observation 1: RAN2 has not reached conclusion on supporting SBFD operation with CA.

Existing Scell activation requirements are defined assuming that there are corresponding band combinations supported in the RF specification. However, when the Rel-19 SBFD work item started, there was no TU allocated for the band combination discussion at all within RAN4. This indicates there is no band combination specified for SBFD Scell.

- Observation 2: No band combination for SBFD CA has been specified due to no TU allocation in RAN4 when work item started.

Due to the fundamental assumption that Rel-19 SBFD aware UE is half-duplex, to support CA, how the UE will handle transmission in UL in Scell while receiving DL in Pcell is unclear. Another example is the simultaneous TX/RX UE capability is mandatory to support the band combinations that have been specified.

- Observation 3: Fundamental UE capability to support simultaneous TX/RX for SBFD CA is completely missing.

- Observation 4: RAN1 agreement ‘FFS how to handle half-duplex CA’ which also indicates RAN1 has not concluded how SBFD operation with CA can be supported.

Issue 2-6: SCell activation <Agreement>: - Existing SCell activation requirements apply also for SBFD SCell.

Based on our observations above, this agreement needs to be further revisited.

Due to the CSI-RS based L1 measurement impact, the potential CSI reporting timeline in existing Scell activation requirement may also be impacted, we see further analysis is needed to conclude whether existing Scell activation requirements applies. Due to the work item is finished, we don’t see the possibility of specifying any proper requirements.

- Proposal 1: RAN4 revisit the agreement for SBFD Scell activation requirement due to there is no conclusion that SBFD in Scell is supported and legacy requirement can not be applied.

Issue 2-8: Simultaneous configuration of SBFD and Dual Connectivity <Way forward>: This issue has been discussed in RAN4 but no agreement has been achieved in this meeting.

Regarding support SBFD and DC, based on our understanding of the RAN2 LS to RAN1 and RAN4 [3] there is no conclusion whether SBFD and DC is supported.

RAN2 would like to thank RAN1 for the LS on simultaneous configuration of SBFD and DC in R1-2504858. RAN2 has evaluated the specification impact to the scenario of simultaneous configuration of SBFD and inter band NR-DC as indicated in the LS, and made the following agreement: RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion..

- Observation 5: No conclusion from RAN2 that SBFD and DC is supported.

Technically, it is similar as SBFD and CA, the band combination from RAN4 is completely missing during Rel-19 work item phase. How to handle half-duplex UE handling simultaneous MCG and SCG cell group has not been discussed at all.

- Observation 6: No band combination for SBFD DC have been specified due to no TU allocation at RAN4 when work item started.

- Observation 7: Fundamental UE capability to support simultaneous TX/RX for SBFD DC is completely missing.

Since the work item for SBFD core part is finished, we think it is very unreasonable to discuss the support of SBFD and DC due to no clear conclusion from RAN1 and RAN2 and no fundamental band combination and UE capability being specified.

- Proposal 2: SBFD and DC have not been concluded to be supported and fundamental RAN4 requirements and specification change is needed.

# Summary

The following have been proposed in the current contribution:

	Observation 1: RAN2 has not reached conclusion on supporting SBFD operation with CA.

	Observation 2: No band combination for SBFD CA has been specified due to no TU allocation in RAN4 when work item started.

	Observation 3: Fundamental UE capability to support simultaneous TX/RX for SBFD CA is completely missing.

	Observation 4: RAN1 agreement ‘FFS how to handle half-duplex CA’ which also indicates RAN1 has not concluded how SBFD operation with CA can be supported.

	Observation 5: No conclusion from RAN2 that SBFD and DC is supported.

	Observation 6: No band combination for SBFD DC have been specified due to no TU allocation at RAN4 when work item started.

	Observation 7: Fundamental UE capability to support simultaneous TX/RX for SBFD DC is completely missing.

	Proposal 1: RAN4 revisit the agreement for SBFD Scell activation requirement due to there is no conclusion that SBFD in Scell is supported and legacy requirement can not be applied.

	Proposal 2: SBFD and DC have not been concluded to be supported and fundamental RAN4 requirements and specification change is needed.

# References

- R4-2515170 “WF on SBFD evo maintenance”, Huawei, October ,2025

- R2-2507746 “LS on SBFD and CA”, INTERDIGITAL, October ,2025

- R2-2506492 “Reply LS on simultaneous configuration of SBFD and DC”, ZTE, October ,2025