# 3GPP TSG RAN WG4 Meeting #117													                     R4-2521871

# Dallas, USA, 17-21 November, 2025

Agenda Item:	6.10.3

Source: 	Ericsson

Title:	On performance requirement for Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. The RAN4#116bis WF is listed in [2].  In this paper we provide our view for the remaining open issues.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

Issue 2-1-2: Applicability rules and number of test cases Agreements: - Further discussion applicability rules, taking the table containing TCs for EMR based SCell activation as reference. TC No. (CR No.); Scenario. 1; Normal SCell activation + FR1 + Configuration 1. Normal SCell activation + FR1 + Configuration 2. Normal SCell activation + FR1 + Configuration 3. 2; Normal SCell activation + FR2 + Configuration 1. Normal SCell activation + FR2 + Configuration 2. Normal SCell activation + FR2 + Configuration 3. 3; Direct SCell activation at SCell addition + FR1 + Configuration 1. Direct SCell activation at SCell addition + FR1 + Configuration 2. Direct SCell activation at SCell addition + FR1 + Configuration 3. 4; Direct SCell activation at SCell addition + FR2 + Configuration 1. Direct SCell activation at SCell addition + FR2 + Configuration 2. Direct SCell activation at SCell addition + FR2 + Configuration 3. 5; PUCCH SCell activation + FR1 + Configuration 1. PUCCH SCell activation + FR1 + Configuration 2. PUCCH SCell activation + FR1 + Configuration 3. 6; PUCCH SCell activation + FR2 + Configuration 1. PUCCH SCell activation + FR2 + Configuration 2. PUCCH SCell activation + FR2 + Configuration 3. Way forward: - Discuss the following applicability rules as a starting point: - Agree the following testing applicability rules: - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped. - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped. - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped. - Other applicability rules are not precluded. - Note: Any agreed testing applicability rules will be captured in clause A.3.X in 38.133.

For Rule 1, we understand the logic behind it is that if the MAC CE based DL Scell activation for UE supporting Rel-18 EMR test case can be skipped since for PUCCH Scell activation regardless of whether UE has valid TA or not, the MAC CE based Scell activation will be verified together with Rel-18 EMR reporting mechanism.

For Rule 2: We don’t see why the RRC based Direct Scell activation can be skipped due to the reason that UE have verified the MAC CE based activation requirement.

For Rule 3: we think it is okey to skip the Rel-18 EMR test case since what the EMR test case have verified is at different condition UE may report, and the report is based on the accuracy.

- Proposal 1: Rule 1 and Rule 3 can be followed to balance the test scope and test efficiency.

Issue 2-1-5: Report mechanism of EMR results in the test cases - Proposals - Option 1: (Nokia) - Support UE reporting in RRCReconfigurationComplete message in INACTIVE test case(s) - Support UE reporting in UE information request / response in IDLE-mode tests cases.

In our view, this is more related to how EMR test case shall be followed. In Rel-18 there is no specific change in regarding the report mechanism rather than what should be reported. The enhancement is about to report cell reselection measurement results etc not which message shall the UE report. We don’t think it is necessary to test this not new behaviour in Rel-19 test case.

- Proposal 2: No need to verify the legacy report mechanism in the test case.

Issue 2-2-2: Details of test case

From the UE procedure aspect, performance evaluation begins when the UE receives the measurement configuration for EMR measurement and concludes when UE sent the CSI report for a successful Scell activation.

From a performance perspective, if the UE sends a valid measurement report 5 seconds prior to receiving the Scell activation command, the target cell is considered known. The known condition will apply, and specific delay activation requirements will be verified during the test. Conversely, if the UE does not send a valid measurement report before the activation command, the target cell is treated as unknown, and the unknown activation requirements will be verified during the test case.

From a test case timeline perspective, fast Scell activation for UEs supporting Rel-18 EMR requires a combination of two existing test cases, as illustrated in Figure 1.

Figure 1. Test case timelines to combine existing test cases.

- Observation 1: The EMR/Cell reselection test case and the Scell activation test case follow relatively independent timelines.

We think the T1 and T2 for the EMR test case blue timeline in Figure 1 can be merged since no UE behavior is verified during this time slot, it is just making sure UE connected to one cell and have received the EMR measurement configuration.

For legacy Scell activation test case, the green timeline in Figure 1, both activation and deactivation delay have been verified, we think it is okey to only verify the activation delay, T1 and T2 can be merged into one time duration.

The total procedure consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively as the figure below illustrates:

- T1: During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2 (SCell to be activated). UE is configured with measurement reporting for Cell 2. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

- T2: At the beginning of T2, Cell 2 becomes detectable. Signal level of Cell 2 and the duration of T2 are set to fixed values.

- T3: At the beginning of T3, the signal level of Cell 2 is set to another value. The duration of T3 equals to measIdleValidityDuration-r18.

- T4: The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report within the duration of T4.

- T5: At the beginning of T5, the TE sends a message for activation of the SCell (Cell 2). The activation command can be either RRC_reconfiguration or MAC CE command. The point in time at which the activation command is received at the UE antenna connector is denoted as slot #n. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.2A.

Figure 2. Proposed test case timelines for Fast Scell activation with UE supporting Rel-18 EMR

- Proposal 3: Total test procedure consist 5 successive time periods for fast Scell activation with UE support EMR .T1: from UE camp in Pcell to receive RRC_Release configuration. T2: from Scell become detectable to signal level of Scell change to another level. T3: From Scell signal level change to end of validity duration. T4: from UE receive Paging to UE receives Scell activation command. T5: From UE receive activation command  to UE send the CSI report.

# Conclusion

	Proposal 1: Rule 1 and Rule 3 can be followed to balance the test scope and test efficiency.

	Proposal 2: No need to verify the legacy report mechanism in the test case.

	Proposal 3: Total test procedure consist 5 successive time periods for fast Scell activation with UE support EMR .T1: from UE camp in Pcell to receive RRC_Release configuration. T2: from Scell become detectable to signal level of Scell change to another level. T3: From Scell signal level change to end of validity duration. T4: from UE receive Paging to UE receives Scell activation command. T5: From UE receive activation command  to UE send the CSI report.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-25124801, WF for NR_RRM_Ph5_part 2, CATT, Octorber,2025