3GPP TSG-RAN4 Meeting #117	R4-2521876

# Dallas, USA, 17-21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6219 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI-RS based CBD requirements with SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on RAN4 116bis meeting agreement from WF R4-2515170: Issue 2-5: Clarification on the meaning of L1=0 when M =1 <Agreement>: - For the case UE does not support SBFD capability or SBFD is not configured, clarify that L1 is not applicable. - For the case UE supports SBFD capability and SBFD is configured, clarify L1 for the case of M=1 with one of the following alternative. - Apply the way of L1 counting for M = 3 to M = 1. The legacy UE which do not support SBFD, the requirement table shall not be applicable to have the L or L1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the L1 in CSI-RS based CBD requirements with SBFD issue above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based CBD requirements with SBFD is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.5.6	Requirements for CSI-RS based candidate beam detection

#### 8.5.6.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set q1 configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.6.2. The requirements in this clause apply when UE is required to perform candidate beam detection on no more than 1 serving cell per band unless otherwise specified. For UE supporting intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided for the configured FR1 intra-band non-contiguous CA, the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

For UE supporting intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v19xy is “type 4”

3>	the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers

2> if nonCollocatedTypeNR-CA-v19xy is “type 1”

3>	the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 1 serving cell per band

2> else if nonCollocatedTypeNR-CA-v19xy is not provided

3>	the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 1 serving cell per band

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers

2>else

3> the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 1 serving cell per band

#### 8.5.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set q1 estimated over the last TEvaluate_CBD_CSI-RS period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS period provided CSI-RS Ês/Iot is according to annex B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in table 8.5.6.2-1 and 8.5.6.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320 ms.

The value of TEvaluate_CBD_CSI-RS is defined in table 8.5.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in table 8.5.6.2-2 for FR2 with scaling factor N, where

-	N = 2, 4 or 6 for serving cell in FR2-1 if the UE supports fastBeamSweepingMultiRx-r18 according to the conditions in clause 3.6.19

-	N=8 for other cases in FR2-1, and

-	N=12 for FR2-2.

For the value of L1,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network,

2>	L1is not applicable;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network,

2>	If DRX is not configured,

3> L1 is the number of occasions of the CSI-RS resource for candidate beam detection that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_CBD_CSI-RS;

2>	If DRX is configured,

3> L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource for candidate beam detection is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_CBD_CSI-RS.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

For a UE supporting LB-CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrentMeasGap-r17 and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are configured,

-	an CSI-RS resource occasion for candidate beam detection is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8 and 9.1.10,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable> 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W,

For UEs supporting LB CA via switching and configured with low band CA via switching:

-	for the CBD monitoring on PCell, Ntotal also includes SSB resource occasions that overlap with the SDL ON duration within the window,

-	for the CBD monitoring on SCell, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CBD-RS resource occasions within the window W,

-	 that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or

-	that are not overlapped with SCell ON duration corresponding to the LB CA switching pattern, for the CBD on PCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the CBD on SCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, and

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion nor any SMTC occasion within the window W, and

-	an CSI-RS resource occasion for candidate beam detection is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

-	TL1 is periodicity of the target CBD-RS, and

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	for UE not supporting LB CA via switching, switching pattern periodicity is not applicable.

-	CBD requirement in this clause is not applied when Noutside_MG = 0.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s), concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when candidate beam detection RS is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P =Psharing factor, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod,, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSMin(xRP, TSMTCperiod), when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=31-TCSI-RSxRP,, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the CBD-RS resource outside GAP is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a GAP is configured only and the GAP is not NCSG,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps the GAP occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG GAP only is configured,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG only, an CBD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured, a CBD-RS resource or an SMTC occasion is not considered to be overlapped by a GAP occasion if the GAP occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

NOTE:	The overlap between CSI-RS for CBD and SMTC means that CSI-RS for CBD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 8.5.6.3.

When the configured aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for candidate beam detection, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if CSI-RS resource occasions for candidate beam detection are fully overlapped with MUSIM gap(s), or the union of MUSIM gap(s) and GAPs, no requirement applies for CSI-RS based candidate beam detection.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MCBD used in table 8.5.6.2-1 and table 8.5.6.2-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set q1 is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that CSI-RS resource configured in the set q1 is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs in at least one DL subband.

The values of PCBD used in table 8.5.6.2-1 and table 8.5.6.2-2 are defined as

-	For each CSI-RS resource in the set q1 configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PCBD = 1.

-	For each CSI-RS resource in the set q1 configured for PSCell in NR-DC

-	PCBD = 2 if UE is configured for candidate beam detection on SCell, 1 otherwise.

-	For each CSI-RS resource in the set q1 configured for a SCell

-	PCBD = Z in EN-DC or NE-DC or SA.

-	PCBD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing candidate beam detection only for SCell

-	PCBD is the number of band(s) on which UE is performing candidate beam detection only for SCell.

Table 8.5.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil((MCBD+L1) P PCBD) max(TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MCBD+L1) P PCBD) TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CBD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. NOTE 3: L1 is only applicable for UE supports supportSBFD and SBFD is configured by the network. |  |  |

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil((MCBD+L1) P N PCBD) max(TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MCBD+L1) P N PCBD) TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set q1. TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CBD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. NOTE 3: L1 is only applicable for UE supports supportSBFD and SBFD is configured by the network. |  |  |

#### 8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI or intra-frequency neighbor cell(s) configured with L1-RSRP measurement or inter-frequency neighbor cell(s) configured with L1-RSRP measurement without gap.

For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has same SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has different SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS based CBD measurement for without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CBD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for CBD measurement without any restriction.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.For both FR1 and FR2, when the CSI-RS for CBD measurement fully or partially overlaps with the OFDM symbol as SSB or CSI-RS from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB or CSI-RS from candidate LTM neighbour cell.

For FR1, when the CSI-RS for CBD measurement fully or partially overlaps with the OFDM symbol as SSB from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC fully or partially overlaps with the OFDM symbol as SSB from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC fully or partially overlaps with the OFDM symbol as CSI-RS from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement, UE is required to measure one of but not both CSI-RS for CBD measurement and CSI-RS for intra-frequency L1-RSRP measurement. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and the other CSI-RS. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

<End of Change 1>