3GPP TSG-RAN WG4 Meeting #117                     	  		 R4-2521886

# Dallas, TX, USA, November 17-21, 2025

Title:	Channel Arrangement Issues for 6G

Source:	Qualcomm Incorporated

Agenda item:	8.2.4

Document for:	Discussion

# 1	Introduction

Issues related to channel arrangement were studied last meeting under the broad agenda of system parameters. Some high level agreements were reached in [1].

In this paper we further discuss issues related to the channel raster and the sync raster design.

# 2	Discussion

## 2.1 Channel Raster for 6GR

Some high level agreements on the directions for study regarding the channel raster were agreed in [1] and are listed below for convenience.

  - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands

    - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster

  - Investigate the interaction between the channel raster and the synchronization raster

  - Investigate the necessity of channel raster or alternative ways for the channel configuration

    - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster

    - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported

  - Study the listed main proposals especially for the migration and co-existence approaches

    - Other options not presented in this meeting are not precluded

  - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

During RAN4#116bis, a fundamental question was raised: is the channel raster still necessary, or has it become obsolete? Every cellular system requires an initial acquisition process, which means the UE must have a predefined set of frequencies to search. Therefore, synchronization signals must be positioned at specific, predetermined locations. As a result, channels need to align with these synchronization signals, creating a finite set of frequency positions where channels can be placed.

Additionally, channels must operate on defined frequency positions to support device compliance testing and facilitate signaling for measurements, handovers, and other procedures. Based on these considerations, maintaining a channel raster definition remains essential and contributes to simpler system operation.

Observation 1: The channel raster is needed to enable compliance testing and operational simplicity/efficiency.

The SCS based raster was introduced in NR as it offers multiple advantages over the “legacy” 100kHz raster. These advantages were heavily debated during the initial NR release (Rel-15) and were briefly presented in [2]. This should be taken as baseline at least in all the NR bands for which it is currently used and in future 6G bands. Further enhancements can be considered depending also on the sync raster design and channel placement options.

Proposal 1: Adopt the SCS based raster as the baseline in all bands where it is currently specified and in all future 6G bands.

Whether continued support for the 100 kHz raster is required will depend primarily on 5G–6G spectrum-sharing methods and whether subcarrier-level alignment is necessary. RAN4 will have to wait for more details on the MRSS design to see if subcarrier/RB level alignment between existing NR deployments and 6GR is needed or not. Channel raster enhancements were introduced for NR to support some specific use cases (nesting of channels with different bandwidths) which led to a larger number of raster entries but without full coverage of the sync raster. RAN4 should further study whether it is possible to further optimize the channel raster for the bands using 100kHz raster to lower the number of entries or to enable migration to SCS based raster when NR will be phased out. Addition of new channel raster entries for future alignment with the SCS based raster can be considered given the future benefits.

Proposal 2: RAN4 should study optimization of the channel raster in bands which use the 100 kHz raster and/or possible addition of new raster points to enable future migration to SCS based raster when coexistence with NR is no longer necessary.

## 2.2 Sync Raster Considerations

Some high level agreements on the directions for study regarding the channel raster were agreed in [1] and are listed below for convenience.

  - Evaluation on sync raster from RAN4 perspective:

    - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4

    - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving

  - Collaboration and planning:

    - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset.

    - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

The sync raster concept was introduced in NR and it was enabled by the flexibility of the SSB placement inside the channel. The sync raster was designed such that any possible channel deployed on the channel raster fits at least one SSB entry. In order to fulfil this design requirement, the sync raster design has to take into account the channel raster design.

Observation 1. The sync raster design has to account for the channel raster design such that any possible channel position is covered by at least a single sync signal entry.

The SCS based raster leads to the minimum number of sync raster entries as the channel positions are fixed in the entire band. The 100kHz raster requires more entries (~3x compared to the SCS based raster) because of the different subcarrier position offsets. Use of the SCS based raster in more bands compared to NR would enable a reduction of the sync raster entries. This would be highly desirable considering the fact that a much longer SSB periodicity is proposed for 6GR. RAN4 should study sync raster optimizations to minimize the number of sync raster entries while maintaining forward compatibility for flexible channel placement.

Proposal 3. RAN4 should study sync raster optimizations to minimize the number of sync raster entries while maintaining forward compatibility for flexible channel placement.

The sync raster design depends on the channel raster, the SSB bandwidth and the minimum channel BW supported in a band. These 3 parameters were the determining factors as shown in [3], [4]. The same design principle should be kept for NR.

Proposal 4. Maintain the same design principles for the 6GR sync raster design as for NR. The raster granularity depends on the channel raster, the SSB bandwidth and the minimum channel BW supported in a band.

In order the make meaningful progress on the sync raster design, more details on the SSB design would have to be known.

# 3 	Conclusion

In this paper we presented further analysis on the scope of the RAN4 6G discussions related to the channel raster and the sync raster. We made the following observations and proposals.

Proposal 1: Adopt the SCS based raster as the baseline in all bands where it is currently specified and in all future 6G bands.

Proposal 2: RAN4 should study optimization of the channel raster in bands which use the 100 kHz raster and/or possible addition of new raster points to enable future migration to SCS based raster when coexistence with NR is no longer necessary.

Observation 1. The sync raster design has to account for the channel raster design such that any possible channel position is covered by at least a single sync signal entry.

Proposal 3. RAN4 should study sync raster optimizations to minimize the number of sync raster entries while maintaining forward compatibility for flexible channel placement.

Proposal 4. Maintain the same design principles for the 6GR sync raster design as for NR. The raster granularity depends on the channel raster, the SSB bandwidth and the minimum channel BW supported in a band.

# 4.	References

[1]	R4-2514650, “WF on 6GR system parameter”, Feature Lead(Huawei, HiSilicon), RAN4#116bis

[2]	R4-2513020, “Qualcomm views on 6G System parameters”, Qualcomm Incorporated

[3]	R4-1706544,“Channel Raster and Synchronization Signal Raster for NR”, Qualcomm Incorporated.

[4]	R4-1712184, “Synchronization Raster for NR”, Qualcomm Incorporated , RAN4#85 Reno