3GPP TSG-RAN WG4 Meeting #117	R4-2521891

Dallas, USA, 17th – 21st November 2025

Agenda Item:	8.9 Spectrum sharing

Source:	ISSDU, NTU

Title:	Discussion on System Parameters for MRSS in NR/6GR

Document for:	Discussion

Introduction

Sub-topic 1-2: Aspects to facilitate MRSS

Issue 1-2-0: General consideration

Agreement:

- To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study

- System parameters

- RF requirements

- RRM requirements

- Interference handling

- Taking following table as starting point to discuss which system parameters are impacted by MRSS.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

At RAN#108, a new Study Item (SID) on 6G Radio was approved [1]. The SID highlights that the evolution towards 6G Radio is essential to support future usage scenarios requiring extreme performance, flexibility, and scalability. In addition, RAN initiated studies on 6G requirements [2][3] to define quantitative targets for key performance indicators (KPIs) such as peak data rate, spectral efficiency, user-experienced data rate, area capacity, user/control plane latency, reliability, mobility, and energy efficiency. During RAN4#116bis, agreements were made to initiate studies on spectrum sharing and system-parameter alignment between 5G NR and 6GR under the MRSS (Multi-RAT Spectrum Sharing) framework. MRSS is one of the important study directions identified for NR/6GR evolution, with the goal of enabling multiple radio access technologies (RATs) to coexist efficiently within shared spectrum resources. At RAN4 #116bis [4], the following aspects were identified as starting points for MRSS studies: System parameters, RF requirements, RRM requirements, Interference handling.

Among these, Numerology, Channel Raster, and Sync Raster were categorized as “Maybe Impacted” by MRSS operation and therefore require further evaluation.

In practical deployment, TN (Terrestrial Network) and NTN (Non-Terrestrial Network) spectrum sharing can serve as one of the representative scenarios to observe interactions among system parameters, as it combines the stable characteristics of terrestrial links with the Doppler and delay variations of non-terrestrial systems.

This contribution focuses on these three parameters, providing observations and proposals that are quantifiable, reproducible, and waveform-independent, to support early RAN4 baseline evaluation under MRSS conditions.

Discussion

2.1 Numerology Considerations

Differences in numerology between NR and 6GR systems (e.g., 15 kHz vs 30 kHz SCS) directly influence symbol duration, sub-carrier spacing, and scheduling alignment in shared-spectrum conditions.

Uncoordinated numerology combinations may result in inter-numerology interference (INI) at boundary regions, increased filtering complexity, and degraded spectral efficiency.

Observation 1: Full numerology alignment simplifies scheduling and minimizes INI but limits flexibility between RATs.

Allowing one numerology ratio step (e.g., 15~30 kHz or 30~60 kHz) maintains integer time-alignment (1:2 or 1:4 symbol duration ratio) and allows simple sub-carrier mapping with minimal guard overhead. Numerology separation beyond two ratio steps (e.g., 15~120 kHz) may require complex filtering and timing compensation.

Proposal 1: RAN4 can evaluate numerology combinations with ratio spacing of 1:1, 1:2, and 1:4 to identify baseline interference and timing behaviors under MRSS conditions.

These cases would provide quantifiable and repeatable references for future coexistence analysis.

2.2 Channel Raster Alignment and Offset

The alignment of channel raster between TN and NTN determines energy coupling across adjacent subcarriers. Full alignment (0 kHz offset) may cause overlap at band edges and degrade Adjacent Channel Leakage Ratio (ACLR). Excessive offset, while reducing coupling, increases UE frequency-tracking and measurement latency.

Observation 2: Introducing a moderate offset (e.g., 10 ~ 30 kHz) may reduce inter-system coupling by approximately 2~4 dB in ACLR while maintaining feasible transceiver tuning. Offsets greater than ≈ 45 kHz further reduce interference but increase UE search delay and complexity.

Proposal 2: RAN4 can define representative channel-raster offset cases of 0 kHz (aligned), 15 kHz (moderate offset), and 30 kHz (larger offset).

Evaluation may also include NTN Doppler conditions (± 3~5 kHz) to analyze impacts on ACLR, blocking, and frequency-tracking behavior. These cases could serve as repeatable baselines for early 6GR frequency-domain studies.

2.3 Sync Raster Alignment

When TN and NTN transmit synchronization signals (e.g. SSB or PRS) with tight alignment, correlation ambiguity may occur within the UE receiver, leading to false peaks and increased acquisition time. NTN’s long propagation delay and Doppler effects amplify this behavior.

Observation 3: A slight sync-raster offset (e.g., 10~30 kHz) can separate correlation peaks, reduce cross-system interference (≈ 3 dB), and improve detection probability. Offsets beyond ≈ 40 kHz further reduce interference but expand the UE search window, causing additional latency.

Proposal 3: RAN4 can establish baseline sync-raster configurations with frequency offsets of 10, 15, and 30 kHz and timing periodicities of 5, 10, and 20 ms.

Evaluations could include UE detection success, false-peak rate, and search delay under various numerology combinations (15/30/60 kHz SCS). Such baselines would support quantifiable and comparable RAN4 evaluations for synchronization parameter studies.

Conclusion

In this contribution, we have the following observations and proposals on system parameters for MRSS in NR/6GR:

Observation 1: Full numerology alignment simplifies scheduling and minimizes INI but limits flexibility between RATs.

Observation 2: Introducing a moderate offset (e.g., 10 – 30 kHz) may reduce inter-system coupling by approximately 2~4 dB in ACLR while maintaining feasible transceiver tuning. Offsets greater than ≈ 45 kHz further reduce interference but increase UE search delay and complexity.

Observation 3: A slight sync-raster offset (e.g., 10~30 kHz) can separate correlation peaks, reduce cross-system interference (≈ 3 dB), and improve detection probability. Offsets beyond ≈ 40 kHz further reduce interference but expand the UE search window, causing additional latency.

Proposal 1: RAN4 can evaluate numerology combinations with ratio spacing of 1:1, 1:2, and 1:4 to identify baseline interference and timing behaviors under MRSS conditions.

Proposal 2: RAN4 can define representative channel-raster offset cases of 0 kHz (aligned), 15 kHz (moderate offset), and 30 kHz (larger offset).

Proposal 3: RAN4 can establish baseline sync-raster configurations with frequency offsets of 10, 15, and 30 kHz and timing periodicities of 5, 10, and 20 ms.

References

[1] RP-251881: New SID: Study on 6G Radio

[2] RP-250810:  Revised SID: Study on 6G Scenarios and requirements

[3] 3GPP TR 38.914: Study on 6G Scenarios and Requirements, v0.1.0.

[4] R4-2514646, “WF for [116bis][108] 6G Spectrum Sharing,” RAN4#116bis, October 2025.