3GPP TSG-RAN4 Meeting #117	R4-2521897

# Dallas, USA, 17th – 21st November 2025

Title:	Views on SBFD-to-SBFD coexistence

Source:	Qualcomm Incorporated

Agenda item:	7.2.4

Document for:	Approval

# Introduction

Following the Rel20 BS RF enhancement WI [1] approved in RAN#109, we have the following objective regarding SBFD-capable BS to SBFD-capable BS in the adjacent channel:

SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

In RAN4#116bis, a WF on the Rel20 BS RF Phase 2 was approved, which entailed the below regarding SBFD-SBFD work:

- For next meeting further consider following options, with the intention to down-select: Option 1: UD (Operator 1) and DU (Operator 2) Option 2: DUD (Operator 1 and 2) Option 3: UD and DUD Provide more description on each specific case. - As baseline use simulation parameters defined in TR 38.858. For 6 GHz, new parameters for BS antenna should be established for SBFD.

In this paper, we provide in this paper our preliminary views on SBFD BS SBFD coexistence aspects envisioned in RAN4 during Release 20.

# General discussion

During Rel-18 study item [2] and Rel-19 work item [3] on SBFD-capable BS requirements, the focus within RAN4 on specifying different Tx and Rx requirements for SBFD-capable BSs given with the assumption that the adjacent channel is a legacy TDD BS. That assumption stems from the fact that in already existing NR bands, deployment of SBFD necessitates ensuring protection and coexistence with the already deployed TDD BSs. For adjacent channel coexistence between TDD and SBFD-capable BS, it was discussed and captured in [2] that the ACIR enhancement is limited by the ACLR of TDD BS. Coexistence was shown to be feasible for 100% grid-shift assumption, which was the baseline assumption during Rel-18 and Rel-19 in RAN4.

Observation 1: Work in Rel-18 and Rel-19 considered TDD to SBFD adjacent channel coexistence, which is in general limited by legacy TDD BS requirements, especially regarding ACIR enhancements.

In Rel-20 work item [2], the above objective is extended to account for cases of green-field spectrum deployments, where SBFD-capable BSs can be deployed with another SBFD-capable BS in the adjacent channel. Through such scope, RAN4 can quantify what is the exact ACIR needed in order to meet the SBFD UL degradation (i.e., case 3 in [2]) target without any restriction on existing TDD deployments.

In terms of simulation work, the WF in [2] considers different SBFD configurations to be studied in the coexistence work. It is important to recall that Rel-18 and Rel-19 simulation work considered the flat ACLR model. In order to have realistic results, it is important to understand how the different configurations (e.g., UD operator 1 and DU operator 2) would impact the results. Since it was agreed in [2] to reuse baseline assumptions in TR 38.858, it is important to note that ACLR model adopted there was flat, meaning that whether DUD or DU SBFD configuration is adopted, the leakage from the aggressor gNB will be flat across the neighboring subbands.

Observation 2: RAN4 considered a flat leakage and selectivity model in TR 38.858, which implies that whether DUD or DU configuration is assumed, the leakage from the interfering gNB is the same.

To analyze the different subband configurations impact on the study, it is important first to understand what leakage and selectivity model (ACIR) that would be used in the simulations. It is a viable option to adopt the step wisemodel given in TR 36.942, which implies that depending on the SBFD configuration, the victim would fall in either ACLR1, ACLR2, or ACLR3 of the aggressor gNB. A visualization of the different configurations and a proposal to account for the leakage from both DL and UL subband is shown in figure 1 below. Thus, to have the results more realistic compared to what was studied in TR 38.858, RAN4 could adopt the model given in TR 36.942

Figure 1 Visualization of leakage from DL and UL subband

Proposal 1: RAN4 to discuss how to account for different ACLR/ACS steps from TR 36.942 in the Rel20 adjacent channel coexistence SBFD-SBFD work.

In order to have efficient discussions in Rel-20 work, it would be viable if RAN4 could agree on which Tx and Rx requirements would be determined via the adjacent channel coexistence work and which ones might not need simulation work. By doing so, it would make a clear direction for RAN4 as well as make it have enough time to agree on controversial simulation parameters.

Proposal 2: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence.

In TR 38.858, RAN4 studied in Section 11 the two following cases when SBFD gNB is a victim:

- Case 3: considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1

- Case 4: considers SBFD as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1.

Case 3 entailed both the case when SBFD DL subband and UL subband were victims. It is expected that RAN4 would consider case 3 as a starting point with the adjustment that the aggressor network would be an SBFD operation. As a starting point, we showcase two scenarios, one where the victim is the SBFD DL subband and interference is an TDD network and the other is where the victim is the SBFD UL subband with the same interference (i.e., TDD network). Our preliminary simulations consider Updated AAS parameters for 7GHz given in TR 38.922. It can be observed that for SBFD DL – SBFD coexistence, impact of interference will be lower compared to SBFD UL- SBFD coexistence. The degradation in UL subband is due to the inter-gNB CLI as well as the additional adjacent channel interference. Compared to the DL case, we can observe that the adjacent channel interference does not lead to additional DL degradation in the performance. Accordingly, RAN4 is expected to carry out adjacent channel coexistence considering gNB ACLR and ACS enhancement to reach a target agreeable metric.

Observation 3: It is expected to start quantifying enhanced ACLR and ACS for SBFD-capable gNB as a first outcome of the Rel-20 SBFD-SBFD coexistence work.

Figure 2 Uma results for SBFD DL as victim (left) and SBFD UL as victim (right) assuming TDD DL as aggressor

One of the important aspects to be discussed is the performance metric. To be consistent, it is viable to adopt the same performance metric given in Section 11 in TR 38.858. 

Proposal 3: RAN4 to adopt the same performance metric in TR 38.858 Section 11 to determine the level of ACIR enhancement needed for the SBFD-capable gNB.

# Conclusion

In this contribution, our observations and proposals for the Rel-20 SBFD-to-SBFD coexistence can be summarized as follows:

Observation 1: Work in Rel-18 and Rel-19 considered TDD to SBFD adjacent channel coexistence, which is in general limited by legacy TDD BS requirements, especially regarding ACIR enhancements.

Observation 2: RAN4 considered a flat leakage and selectivity model in TR 38.858, which implies that whether DUD or DU configuration is assumed, the leakage from the interfering gNB is the same.

Proposal 1: RAN4 to discuss how to account for different ACLR/ACS steps from TR 36.942 in the Rel20 adjacent channel coexistence SBFD-SBFD work.

Proposal 2: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence.

Observation 3: It is expected to start quantifying enhanced ACLR and ACS for SBFD-capable gNB as a first outcome of the Rel-20 SBFD-SBFD coexistence work.

Proposal 3: RAN4 to adopt the same performance metric in TR 38.858 Section 11 to determine the level of ACIR enhancement needed for the SBFD-capable gNB.

# References

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Huawei (Moderator), RAN Meeting #109, Beijing, China.

- R4-2515091, “Way-Forward for NR_BS_RF_Ph2”, Ericsson, RAN 4Meeting #116bis.