3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2521907

Dallas, USA, Nov. 17-21, 2025

Title: 	1Tx PC1.5 for NB-IoT-NTN based on Release 19 PC1 work

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.6.3	IoT-NTN PC1.5 single Tx	[NR_IoT_NTN_req_Ph2-Core]

Document for:	Approval

# 1	Background

In RAN#116bis, as an obvious step, in [2], we proposed MPR/A-MPR and boosting for PC1.5 from the specified R19 PC1 case as it uses the same hardware. A way forward [1] captured several agreements to re-evaluate the PC1.5 MPR. Since this opens up for more evaluation, in this contribution, we re-iterate our proposals but based on our R19 PC1 proposal that is slightly more conservative than the specified R19 PC1 values.

# Discussion

## 2.1 Proposed PC1 MPR for R19 NB-IoT-NTN

In [3], we proposed the Table2 MPR table for PC1 NB-IoT-NTN, based on both PC2 NR PA and GSM mid-band PA measurements. These measurements are still valid for PC1.5 based on the same PAs as the only limitation comes from the “GSM-like” mask that is in absolute power and thus do not depend on the power class.

Table 2: Proposed baseline MPR for PC1 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| SC# 15kHz/3.75kHz | 0 and 11/0-3 and 44-471 | 1 and 10/4-7 and 40-43 | 2-9/8-39 |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 2 | 1 | 0.5 |
| SC# 15kHz | 0-2 and 9-11 |  | 3-5 and 6-8 |
| 3 tone QPSK | 2 |  | 1.5 |
| SC# 15kHz | 0-5 and 6-11 |  |  |  |
| 6 tone QPSK | 2.5 |  |  |  |
| SC# 15kHz | 0-11 |  |  |  |
| 12 tone QPSK | 4 |  |  |  |
| Note 1: except for 0-3 and 44-47 3.75kHz tone positions where MPR is 3dB |  |  |  |  |

## 2.2 PC1.5 NB-IoT-NTN UL requirements

Since it is an obvious option for hardware reuse. especially for handheld devices, the PC1.5 1Tx NB-IoT-NTN UL requirement can also be derived from a PC2 NR PA implementation similar to PC1 in Release 19.

MPR and A-MPR for PC1.5 1Tx NB-IoT-NTN are based on Release 19 PC1 NPR and A-MPR since:

- The same PC2 NR PA with the PC2 NR calibration can implement both power classes

- The MPR limitation is only the NB-IoT mask and the NS_24 and NS_02N that are in absolute power

- In this case, the MPR and A-MPR can be reduced by 2dB as this is the power difference between PC1 and PC1.5

Rather than using the specified PC1 MPR we propose using our slightly more conservative proposal from R19 presented in Chapter 2.1 to derive the PC1.5 MPR and boosting:

- With 2dB MPR reduction, 1.5dB boosting is feasible for the center single-tone allocations and 0.5dB boosting is feasible for the center 3-tone allocation.

- Although boosting was dismissed in the lass meeting, we still believe that any dBm is worthwhile for NTN PC1.5 when the key justification is coverage for handsets. If we recognise that they are challenges in terms of thermal and DC envelope since this is an optional capability, we do not see this as an issue.

- The R19 PC2 boosting framework can be reused for the optional power boosting requirement for PC1.5. However, to minimize the TS impact as the current boosting values are 1dB or 2dB, we suggest only boosting the center single tone allocations by1dB.

Proposal for PC1.5 MPR and A-MPR: The following tables are specified in TS 38.101-5

Table 1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0 dB | 02 dB | ≤ 0 dB | ≤ 0 dB | 02 dB | ≤ 0 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. Note 2: For single-tone positions 2 to 9 for 15kHz SCS and 6 to 41 for 3.75 kHz, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |  |  |

Table 2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 0 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 0.5dB |  | ≤ 0.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. Note 2: For 3-tone positions 3-5 and 6-8, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |

Table 3: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ 8 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |

Proposal for PC1.5 power boosting: Power boosting is specified for PC1.5, by modifying the clause 6.2B.4 in TS 36.102 in adding the yellow text below:

For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for:

The definition of PCMAX_L,c and PCMAX_H,c which would be substituted by:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MAX(MPRc + A-MPRc, P-MPRc) + ΔPNBBoost}

 PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPNBBoost }

, wherein

-	PPowerClass is the maximum UE power specified in Table 6.2B.1-1 without taking into account the tolerance specified in the Table 6.2B.1-1

-	ΔPNBBoost is defined as

2dB for single tone positions 3 to 8 for 15kHz SCS and 8 to 39 for 3.75kHz for PC2

1dB for:

    - Other single tone positions and all multi-tone cases for PC2

    - 2 to 9 for 15kHz SCS and 6 to 41 for 3.75kHz for PC1.5

when all of the following conditions are met:

if the UE indicates support for UE capability [powerBoosting-NBIotNTN-r19]

if IE [powerBoostNBIotNTN-r19] is set to 1

if UE indicates support for power class 2 or power class 1.5.

# Conclusion

In this contribution, we provide our input on PC1.5 NB-IoT-NTN MPR/boosting and A-MPR and make the following proposals.

Proposal for PC1.5 MPR and A-MPR: The following tables are specified in TS 38.101-5

Table 1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0 dB | 02 dB | ≤ 0 dB | ≤ 0 dB | 02 dB | ≤ 0 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. Note 2: For single-tone positions 2 to 9 for 15kHz SCS and 6 to 41 for 3.75 kHz, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |  |  |

Table 2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 0 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 0.5dB |  | ≤ 0.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. Note 2: For 3-tone positions 3-5 and 6-8, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |

Table 3: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ 8 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |

Proposal for PC1.5 power boosting: Power boosting is specified for PC1.5, by modifying the clause 6.2B.4 in TS 36.102 in adding the yellow text below:

For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for:

The definition of PCMAX_L,c and PCMAX_H,c which would be substituted by:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MAX(MPRc + A-MPRc, P-MPRc) + ΔPNBBoost}

 PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPNBBoost }

, wherein

-	PPowerClass is the maximum UE power specified in Table 6.2B.1-1 without taking into account the tolerance specified in the Table 6.2B.1-1

-	ΔPNBBoost is defined as

2dB for single tone positions 3 to 8 for 15kHz SCS and 8 to 39 for 3.75kHz for PC2

1dB for:

    - Other single tone positions and all multi-tone cases for PC2

    - 2 to 9 for 15kHz SCS and 6 to 41 for 3.75kHz for PC1.5

when all of the following conditions are met:

if the UE indicates support for UE capability [powerBoosting-NBIotNTN-r19]

if IE [powerBoostNBIotNTN-r19] is set to 1

if UE indicates support for power class 2 or power class 1.5.

# References

[1] R4-2515157 Way Forward for [116bis][318] NR_IoT_NTN_Ph2, CHTTL, RAN4#116b

[2] R4-2514115 1Tx PC1.5 for NB-IoT-NTN based on Release 19 PC1 work, Skyworks Solutions Inc., RAN4#116b

[3] R4-2509243 MPR input for PC1 NB-IoT-NTN, Skyworks Solutions Inc., RAN4#116