s3GPP TSG RAN WG4 Meeting #117	 													R4-2521918

Dallas, USA, 17 Nov. 2025 – 21 Nov. 2025

Agenda Item:	6.12.2.2

Source: 	Ericsson

Title:	Discussion on CJTC accuracy requirements for MIMO Ph 5

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for the CJT calibration reporting and asymmetric UL TRP requirements.

# Discussion

## CJTC reporting accuracy requirements

RAN4 was discussing how to define the CJTC report accuracy requirements for quite many meetings and during the last meeting, two alternative principles were agreed as potential WF. A decision between these should be made in the next meeting:

- Alternative 1:
Define a single set of tightened accuracy requirements based on SNR ≥ 0 or 3 dB.
→ Further input is encouraged to refine the exact performance criteria.
Note: “Tightened” refers to stricter requirements compared to the -3 dB side condition.

- Alternative 2:
Define two sets of requirements based on UE capability.
→ Input is welcome regarding the specific details of the UE capability.

As discussed over several meetings, there are two sets of results: one set shows better accuracy performance, while the other does not demonstrate similarly improved accuracy. Before analysing which alternative from the WF is preferable, we first review the submitted results from each set to understand the achievable accuracy under various SINR side conditions. We follow the same approach as outlined in the previous meeting:

- Compute the overall phase shift assuming the proposed accuracy is agreed upon.

- Assess the implications of that accuracy level on CJTC performance.

It is important to highlight that the resulting phase shift should remain within 30 to 40 degrees to ensure meaningful benefits from the CJTC report. Since results were submitted by two UE vendors and two NW vendors, we focus the analysis on the submissions from one UE vendor and one NW vendor.

Accuracy of CJTC Frequency Offset:

Phase drift over a slot (across all symbols): 2pi * FO * duration_s *180/pi = 360*FO_Hz * slot_duration_s (deg).

One ppb level in Hz: 1/256 * 200ppb*3.5e9 = 2.734 Hz

Assuming there are no reporting delays, we consider the TRS periodicity of 20ms as the report periodicity. Since phase drift due to frequency offset error adds up linearly over time, we consider 20ms TRS periodicity and 10ms TRS periodicity to compute the phase rotation due to frequency offset error.

Table 1: FO offset and its corresponding phase rotation from UE vendor results

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256. Levels (Hz →degrees) | FO estimation error in AFO = 0.2 ppm and MFO = 256. Levels (Hz →degrees) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #1 | #1 |  |
| 15KHz | -3 | 350 Hz | 65 (178 → 1279) | 65 (178 → 638) |  |
| 15KHz | 0 | 350 Hz | 39 (106 → 768) | 39 (106 → 384) |  |
| 15KHz | 3 | 350 Hz | 26 (71 → 512) | 26 (71 → 256) |  |
| 30KHz | -3 | 350 Hz | 133 (363 → 2618) | 133 (363 → 1309) |  |
| 30KHz | 0 | 350 Hz | 83 (227 → 1634) | 83 (227 → 817) |  |
| 30KHz | 3 | 350 Hz | 52 (142 → 1023) | 52 (142 → 512) |  |

Table 2: FO offset and its corresponding phase rotation from NW vendor results

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256, TRS period of 20ms. Levels (Hz →degrees) | FO estimation error in AFO = 0.2 ppm and MFO = 256, TRS period of 10ms. Levels (Hz →degrees) |
| --- | --- | --- | --- | --- |
|  |  |  | #1 |  |
| 15KHz | -3 | 350 Hz | 14 (38 → 276) | 14 (38 → 138) |
| 15KHz | 0 | 350 Hz | 7 (19 → 138) | 7 (19 → 69) |
| 15KHz | 3 | 350 Hz | 7 (19 → 138) | 7 (19 → 69) |
| 30KHz | -3 | 350 Hz | 26 (71 → 512) | 26 (71 → 256) |
| 30KHz | 0 | 350 Hz | 12 (33 → 236) | 12 (33 → 118) |
| 30KHz | 3 | 350 Hz | 12 (33 → 236) | 12 (33 → 118) |

As shown in the two tables above, neither the UE vendor results, nor the NW vendor results meet the desired accuracy target for the CJTC frequency offset over a 20 ms duration. If we consider a TRS periodicity of 10 ms, we believe the resulting phase rotation could at least be contained within 90 degrees, which may offer some benefit, even though it does not fall within Ericsson’s preferred range of 30 to 40 degrees.

- The FO offset accuracy for CJTC measurement shall be within 7 levels for the SCS of 15 kHz and 12 levels for the SCS of 30 kHz under SNR side condition of 0dB, TRS periodicity of 10ms.

Accuracy of CJTC Delay offset (DO):

As discussed in the last meeting, the phase shift depends on the sub-band size. Based on the contribution presented, a sub-band size of 2 PRBs appears necessary to achieve the desired accuracy level. To further assess this, we analyze the resulting phase shift using one set of results from a UE vendor and one set from a NW vendor, assuming the proposed accuracy is agreed upon.

Phase drift across bandwidth: 2*pi*DO*BW_Hz*180/pi = DO* BW Hz*360

Table 3: DO offset and the sub band wise phase rotation for sub band size of 2 PRB using UE vendor results

| SCS | SNR | Dealy offset b/w two TRPs | DO estimation error in AD = CP and MD = 256 Levels (degrees) for the case of 2RB |
| --- | --- | --- | --- |
|  |  |  | #1 |
| 15KHz | -3 | 4.7µs | 29 (69) |
| 15KHz | 0 | 4.7µs | 17 (40) |
| 15KHz | 3 | 4.7µs | 10 (24) |
| 30KHz | -3 | 2.35µs | 30 (71) |
| 30KHz | 0 | 2.35µs | 16 (38) |
| 30KHz | 3 | 2.35µs | 14 (33) |

Table 4: DO offset and the sub band wise phase rotation for sub band size of 2 PRB using NW vendor results

| SCS | SNR | Dealy offset b/w two TRPs | DO estimation error in AD = CP and MD = 256 Levels (degrees) for the case of 2RB |
| --- | --- | --- | --- |
|  |  |  | #1 |
| 15KHz | -3 | 4.7µs | 4 (10) |
| 15KHz | 0 | 4.7µs | 3 (7) |
| 15KHz | 3 | 4.7µs | 3 (7) |
| 30KHz | -3 | 2.35µs | 5 (12) |
| 30KHz | 0 | 2.35µs | 5 (12) |
| 30KHz | 3 | 2.35µs | 5 (12) |

In the tables, we highlighted the results where the phase rotation across the sub-band is less than 40 degrees. Based on these highlighted results, the submissions from the UE and NW vendors meet the desired accuracy target if the sub-band size is set to 2 PRBs.

- RAN4 to agree on the following for the CJTC DO report.

- Use Alt-1 from the WF to set the accuracy requirement for CJTC-Dd

- Side condition: SNR ≥ 0 dB

- Accuracy level: 17 levels for SCS of 15 kHz and 16 levels for the SCS of 30 kHz

- Sub band size in the test configuration: 2 PRB.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- Use Alt-1 from the WF to set the accuracy requirement for CJTC-Ff. The FO offset accuracy for CJTC measurement shall be within 7 levels for the SCS of 15 kHz and 12 levels for the SCS of 30 kHz under SNR side condition of 0dB, TRS periodicity of 10ms.

- RAN4 to agree on the following for the CJTC DO report.

- Use Alt-1 from the WF to set the accuracy requirement for CJTC-Dd

- Side condition: SNR ≥ 0 dB

- Accuracy level: 17 levels for SCS of 15 kHz and 16 levels for the SCS of 30 kHz

- Sub band size in the test configuration: 2 PRB.

# References

[1] RP-234007, New WID: NR MIMO Phase 5
[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5

[3] R4-2420209 Simulation assumptions for CJT calibration