3GPP TSG-WG4 Meeting #117	R4-2521919

# Dallas, USA, Nov. 17 – Nov. 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CJTC-Dd measurement period and accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core requirements are agreed to be supported for the CJTC delay offset. The TC shoukld be defined to test the feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC on CJTC-Dd measurement period and accuracy is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------- Start of changes ---------------------------

### A.6.7.X	CJTC Delay offset measurement period and accuracy

#### A.6.7. X.1	CJTC Delay offset measurement period and accuracy in FR1

##### A.6.7. X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CJT Calibration Delay Offset (CJTC-Dd) report measurement period and accuracy are within the specified limits outlined in the test requirements section. The configurations used for the test are defined in Table A.6.7.X.1.1-1.

The test consists of two tests, Test 1 and Test 2. Each test further consists of two subtests Test 1, and Test 2.

Test 1: 15 kHz SCS FDD + Delay offset between TRP 4.7µs + MD is 256 levels

Test 2: 30 kHz SCS TDD + Delay offset between TRP 2.35µs + MD is 256 levels

Relevant parameters for each test are provided in the table A.6.7.X.1.2-1. UE needs to pass Test 1 and 2.

Table A.6.7.X.1.1-1: Applicable NR configurations for CJTC Delay offset

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

The test procedure consists of two successive time periods, denoted as T1 and T2, with durations defined as following.

- During T1, the UE is configured to perform CJTC-Dd reporting. Throughout this period, the UE continuously calculates the CJTC delay offset based on the configured TRS periodicity.

- T2 begins when the UE receives a DCI command from the test equipment (TE), requesting the CJTC-Dd report. The DCI command is sent on the next slot of the TRS burst is transmitted.

The test equipment evaluates the CJTC delay by counting the number of slots between the transmission of the DCI command and the reception of the corresponding CJTC-Dd report from the UE.

To verify CJTC delay offset accuracy, the test equipment compares the expected CJTC-Dd index level with the reported delay offset index. A match between the expected and reported values indicates correct CJTC delay offset reporting behavior.

##### A.6.7.X.1.2	Test parameters

In this set of test cases there is one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.X.1.2-1. CJTC Delay offset measurement period, and accuracy is tested by using the parameters in table A.6.7.X.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured with 1 TRS set with the TRS resources in the set are configured in adjacent slot. UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.6.7.X.1.2-1: FR1 CJTC Delay offset test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1~2 |  | freq1 | freq1 |
| Duplex mode |  |  | FDD | TDD |
| TDD Configuration |  |  | N/A | TDDConf.1.1 |
| BWchannel |  | MHz | 10: NPRB,c = 52 | 40: NPRB,c = 106 |
| Sub band size |  |  | 2 PRBS | 2 PRBS |
| PDSCH Reference measurement channel |  |  | SR.1.1 FDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | CR.2.1 TDD |
|  | 2 |  |  |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | CCR.2.1 TDD |
|  | 2 |  |  |  |
| SSB configuration | 1 |  | SSB.3 FR1 | SSB.4 FR1 |
|  | 2 |  |  |  |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration | 1 |  | TRS.1.5 FDD | TRS.1.5 TDD |
|  | 2 |  |  |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1~2 |  | periodic | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 | 2 |
| L1-RSRP reporting period | 1~2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Noc | dBm/15 kHz |  | -98 | -98 |
| Note 2 | dBm/SCS |  | -98 | -98 |
|  | 1~2 | dB | 0 | 0 |
|  | 1~2 | dB | 0 | 0 |
| SSB_RP | 1 |  | -98 | -98 |
| Propagation condition | 1~2 |  | TDL-A 30 ns 10 Hz | TDL-A 30 ns 300 Hz |
| Antenna configuration | 1~2 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

##### A.6.7.X.1.3	Test Requirements

For Test 1 and 2, UE should send the CJTC Delay offset measurement report as per the UL grant sent in the DCI for the 90% of the times over repeated tests.

For Test 1, the reported CJTC Delay offset measurement accuracy index shall be smaller than or equal to 17 for the 90% of the times over repeated tests.

For Test 2, the reported CJTC Delay offset measurement accuracy index shall be smaller than or equal to 16 for the 90% of the times over repeated tests.

--------------------END OF CHANGES--------------------------