3GPP TSG-RAN WG4 Meeting #117	R4-2521946

# Dallas, USA, 17th – 21st November 2025

Agenda item:		10.7

Source: 		Qualcomm Inc.

Title:	 	ETSI requirements for NTN bands

Document for:		Approval

## 1	Introduction

In previous meeting there was a proposal to introduce requirements from ETSI EN 301 681 to both NB-IoT NTN and NR NTN [1, 2]. This contribution discusses the history of ETSI requirements in 3GPP, new developments in ETSI as well as required studies to be done.

## 2	Discussion

Tx requirements from ETSI EN 301 681 are not included in TS 36.102 and TS 38.101-5. This is not an oversight and the approach was discussed over many meetings. In RAN4#107 the following agreement was reached:

Following this, an LS was sent to ETSI in RAN4#108 in [3] where 3GPP asked possibilities for ETSI to reduce misalignment with ETSI and 3GPP requirements. In RAN4#108bis RAN4 received a response [4] stating that

“Current TC-SES ETSI harmonised standards have not yet considered 3GPP NTN access technology and updates may be needed. Therefore, it is proposed to create a new work item (NWI) in ETSI in order to accommodate NTN requirements in harmonised standards. There are already ongoing discussions between ETSI TC SES and ETSI TC MSG groups to find a way forward to capture the 3GPP NTN specifications in ETSI standards.”

Observation 1: ETSI view is that 3GPP NTN access technology has not been considered in TC-SES harmonized standards and new work was started in ETSI to accommodate 3GPP NTN requirements

Since then, ETSI has approved work items to develop two new harmonized standards:

- DEN/SES-00475 (EN 304 122), NR-NTN EN below 7,125 GHz

- DEN/SES-00474 (EN 304 121), IoT-NTN EN below 7,125 GHz

ETSI work programme sets the technical group approval to March 2026

Observation 2: ETSI is working on two new harmonized standards to include requirements applicable to 3GPP NTN

Given the on-going work in ETSI, it may be premature to rush to include the old requirements to 3GPP. Given the expected deployment timelines there also seems to be room to wait and include correct requirements from the beginning.

Introducing the legacy requirements from ETSI right now imposes a risk of doing a large amount of development and testing to meet requirements which will not be required in the real operation.

Proposal 1: Further consider whether it is necessary to rush to introduce the current ETSI requirements in 3GPP standards considering the progress of the new harmonized standards as well as expected deployment timelines.

Proposal 2: In case requirements would be introduced, clear agreements are needed on what happens to the outdated ETSI requirements once the new harmonized standards are complete.

If the requirements would be introduced, one of the most important questions is which is the applicable release. New emissions requirements coupled with new NSs and A-MPR are not appropriate to introduce to closed releases, as there are either devices already out or development done to the point that accommodations are difficult. This avoids any potential backwards compatibility issues and ensures that introduction of local requirements does not prevent existing designs targeted from other regions being deployed.

Proposal 3: New emissions requirements associated with NS-values and A-MPR should be introduced to open release. Earlier release UE is allowed to support those requirements via release independence.

If the requirements would be introduced, detailed A-MPR evaluation needs to be conducted. Given that the original proposal considers introduction of requirements to closed release, this is even more critical as existing designs need to be considered instead of setting targets for upcoming designs. So far no analysis have been provided at least for NB-IoT. We conducted an initial look into the requirements, and it appears PC3 NB-IoT would need A-MPR based on simulation evaluation.

At least one of the problematic emission spikes is IMD3 between a 3.75 kHz or 15 kHz single tone allocation and it’s IQ-image falling on the -20 dBm/3kHz part of the in-band emission mask. In previous 3GPP inputs the ETSI mask and 3GPP SEM have usually been presented as scaled to the same measurement bandwidth is seen in Figure 1. However, when the actual measurement bandwidths are used the difference is much larger. With 3 kHz measurement bandwidth almost all emission energy of 3.75 kHz tone falls within a single measurement bin, and it can clearly violate the ETSI mask as seen in Figure 2.

The same peak would still have margin to the ETSI mask as the power within 30 kHz measurement bandwidth is only slightly larger than within 3 kHz measurement bandwidth.

Given that the result is only from a simulation it is not fully conclusive, further studies would be needed to check existing implementations thoroughly by measurements.

Observation 3: Based on simulation, ETSI EN 301 681 requirements will require A-MPR also for PC3 for NB-IoT at least for some tone allocations.

Proposal 4: Detailed A-MPR studies would be needed before introducing requirements.

Figure 1: NB-IoT SEM mask and ETSI EN 301 681 in-band mask scaled to same measurement bandwidth

Figure 2: NB-IoT SEM mask and ETSI EN 301 681 in-band mask with their actual measurement bandwidths together with IMD3 from single-tone transmission.

For band 255 3GPP specifications also include PC2 and PC1. Introducing emission requirements and A-MPR only for PC3 is not reasonable, as category A CRs will end up making the requirements applicable also for HPUE. This creates a situation where a requirement exist for all power classes but only PC3 has required accommodations (A-MPR and/or guardband)

Proposal 5: All specified power classes would need to be considered and required A-MPR and/or guardband would need to be introduced to all power classes.

Finally, ETSI EN 301 681 includes the nominated bandwidth which has been discussed in RAN4 recently. Nominated bandwidth should be considered as one of the allowed options to meet the emission requirements in case the emission requirements are introduced.

Observation 4: ETSI EN 301 681 includes nominated bandwidth concept as a tool to meet the emission requirements.

Proposal 6: Consider nominated bandwidth as one option to meet the emission requirements, if defined.

## 3	Conclusion

In this contribution ETSI requirements for NTN bands were discussed. Following observations and proposals were made.

Observation 1: ETSI view is that 3GPP NTN access technology has not been considered in TC-SES harmonized standards and new work was started in ETSI to accommodate 3GPP NTN requirements

Observation 2: ETSI is working on two new harmonized standards to include requirements applicable to 3GPP NTN

Proposal 1: Further consider whether it is necessary to rush to introduce the current ETSI requirements in 3GPP standards considering the progress of the new harmonized standards as well as expected deployment timelines.

Proposal 2: In case requirements would be introduced, clear agreements are needed on what happens to the outdated ETSI requirements once the new harmonized standards are complete.

Proposal 3: New emissions requirements associated with NS-values and A-MPR should be introduced to open release. Earlier release UE is allowed to support those requirements via release independence.

Observation 3: Based on simulation, ETSI EN 301 681 requirements will require A-MPR also for PC3 for NB-IoT at least for some tone allocations.

Proposal 4: Detailed A-MPR studies would be needed before introducing requirements.

Proposal 5: All specified power classes would need to be considered and required A-MPR and/or guardband would need to be introduced to all power classes.

Observation 4: ETSI EN 301 681 includes nominated bandwidth concept as a tool to meet the emission requirements.

Proposal 6: Consider nominated bandwidth as one option to meet the emission requirements, if defined.

## 4	References

[1] 	R4-2514407, Views and proposals related to MPR and A-MPR for bands 255 and 256 for IoT-NTN PC1.5 single Tx ViaSat Satellite Holdings Ltd

[2]	R4-2514409, Missing ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255, ViaSat Satellite Holdings Ltd

[3]   R4-2314891, LS on 3GPP NTN requirements and potential impact on specification work in ETSI TC SES, RAN4

[4]	SES(23)000035r1/R4-2317932, LS reply on 3GPP NTN requirements and potential impact on specification work in ETSI TC SES, ETSI TC SES