3GPP TSG-RAN4 Meeting # 117	R4-2521955

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | – | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test case of event triggered reporting with MG cancellation for FR2-1_R19 XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add the test case of SA event triggered reporting with measurement gap cancellation for FR2-1 under DRX for R19 XR. Some test assumptions are approved as below: <Agreement> - Test cases are specified only for per-UE MGs <Agreement> - Test cases are defined only for Type 1 gaps <Agreement> Proposals 1, 2, and 3 are addressed in the same test case - Proposal 1: measurement delay extension with gap cancellation - Proposal 2: Reception/transmission without scheduling restrictions during cancelled gap occasions - Proposal 3: The UE processing timeline of receiving the DCI-based MG skipping indication <Agreement> - Define test case for: - Inter-frequency measurements procedure with gaps - Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1 - FFS whether to define at least one accuracy test case in FR1 and in FR2-1. Some other relevant issues are still suspending: <Way forward> Further discuss on the following proposals - Proposal 1: Further discuss on the UE processing timeline of receiving the DCI-based MG skipping indication <Way forward> Proposals - Proposal 1: For measurement procedure, define test cases without SSB time index detection. - Proposal 2: Use SSB index detection for intra-/inter-frequency measurement TCs, use no SSB index detection with lower priority for intra-/inter-frequency measurement TCs. <Way forward> Proposals for discussion - Option 1: Measurement gap cancelation is modelled as a fixed pattern for the test cases. - Option 2: Measurement gap cancelation is determined randomly by the test equipment. <Way forward> Proposals for discussion - Proposal 1: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. - Proposal 2: For Type 1 MGs test cases: - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset <Way forward> Proposals for discussion - Option 1: The processing time 3ms/5ms shall be verified in the test cases. - Option 1a: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition - Option 2: Define test case for 5ms offset only. <Way forward> Proposals for discussion - Option 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation. - Option 2: Define RAN4 test cases using DCI 1-1 for the canceling indication. <Way forward> Proposals for discussion - Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. - Proposal 2: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows: - Test Option 2 in every test case. - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.. |
| Summary of change: |  | Add the test case of SA event triggered reporting with measurement gap cancellation for FR2-1 under DRX for R19 XR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant test case is vacant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.6.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.7.6.2.x	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used

A.7.6.2.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct measurement and reporting of an event, during the measurement, measurement gap cancellation happens and relevant measurement extension is expected. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4 and 9.3.5. Meanwhile, this test also partly verify the scheduling availability in clause 9.2.5.3.

In this test, there are three cells: NR Cell 1 as PCell in FR2-1 on NR RF channel 1, NR Cell 2 as neighbour cell on the same frequency as the PCell, and NR Cell 3 as neighbour cell in FR2-1 on NR RF channel 2.  The test parameters and configurations are given in tables A.7.6.2.x.1-1, A.7.6.2.x.1-2, and A.7.6.2.x.1-3.

Measurement gap pattern configuration defined in table A.7.6.2.x.1-2 is provided for a UE that does not support per-FR gap, and no gap pattern (Gap Pattern Id and Measurement gap offset) is configured for a UE capable of per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 associated with NR RF channel 2 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and Cell 3. To verify the UE processing timeline for the indication of gap cancellation, UE receives the DCI format 1_1 at the time point of X ms earlier than the ending of T1. X equals to 3 or 5, up to component 4 of FG 64-1. If UE is capable of 3ms for the component 4 of FR64-1, X is set as 3; otherwise X is 5. The measurement gap cancellation is determined randomly by the test equipment except that the 1st measurement gap occasion within T2 should be indicated as cancellation. The starting slot of the 1st measurement gap occasion is configured as the starting slot of T2. The whole cancellation ratio during test is set as 50%. The UE shall be scheduled in the PCell throughout the whole test except the non canceled gap occasions.

Supported test configurations are shown in table A.7.6.2.x.1-1.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.x.1-1: SA event triggered reporting tests without SSB index reading for FR2-1

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: Void. |  |

Table A.7.6.2.x.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2-1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1 | NR Cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1 | NR Cell 2, NR Cell 3 |  | NR Cell 2 is on NR RF channel number 1, NR Cell 3 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1 |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 8 for PC1 or PC 5; 5 for other PC | 82 for PC1 or 5; 52 for other PC |  |

Table A.7.6.2.x.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |  |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  | Rough |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 1 |  | 2 |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  | TDD |  |  |
| BWchannel |  | MHz | Config 1 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  |  | Config 1 | 66 |  | 66 |  | 66 |  |  |
| BWP BW |  | MHz | Config 1 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  | - |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  | - |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  | 120 |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz Note 5 |  | -104.7 |  | -104.7 |  | -104.7 |  |  |
| Note2 |  | dBm/SCS Note 4 | Config 1 | -95.7 |  | -95.7 |  | -95.7 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS Note 5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 | -Infinity |  | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 | -Infinity |  | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 | -Infinity |  | 9 |
| IoNote3 |  | dBm/95.04 MHz Note 5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 | -66.7 |  | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

A.7.6.2.x.2	Test Requirements

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

11520 for UE supporting power class 1 or power class 5, or

7200 for UE supporting other power class.

In test 2  the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

122880 for UE supporting power class 1 or poser class 5, or

76800 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

Neither interruption nor scheduling restriction happens on the canceled measurement gap occasions, as defined in clause 9.2.5.3.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change #1>