3GPP TSG-RAN4 Meeting # 117	R4-2521960

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | – | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test configuration for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test configurations for SBFD have reach the following consensus: Test case scenarios - Agreements - Define test cases for both FR1 and FR2 Test applicability - Agreements - TDD test cases are also applicable to SBFD aware UEs. The new test cases introduced for SBFD aware UEs are additional to the test cases defined for TDD UEs. Test configurations - Agreements - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex - FR2: - Option 1: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode - Option 2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode - TDD UL/DL configuration : DDDSU - Time domain configuration of SBFD slots/symbols: DXXXU - Frequency domain configuration of SBFD subbands for FR1 - DU: 80 MHz – 20 MHz - DUD: 40 MHz – 20 MHz – 40 MHz - FFS: Frequency domain configuration of SBFD subbands for FR2. Change#1: Add the TDD UL/DL configuration for SBFD Change #2,#3: Add the SSB configuration for SBFD Change #4: Add the SBFD configurations |
| Summary of change: |  | - Capture all test configurations for SBFD into TS38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test configurations for SBFD is vacant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.x, A.3.10, A.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.3.1.4	TDD UL/DL configuration

Table A.3.1.4-1: TDD UL/DL configuration for SCS=15 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.1.2 |  |
| referenceSubcarrierSpacing | kHz | 15 | 15 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’10DL:2GP:2UL’ | ‘DSUU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 2 | 2 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘D’ |  |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 0 | 0 |  |
| nrofUplinkSlot |  | 0 | 0 |  |
| nrofUplinkSymbols |  | 0 | 0 |  |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information. |  |  |  |  |

Table A.3.1.4-2: TDD UL/DL configuration for SCS=30 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 |
| nrofDownlinkSlots |  | 3 | 1 | 3 |
| nrofDownlinkSymbols |  | 6 | 11 | 4 |
| nrofUplinkSlot |  | 4 | 2 | 4 |
| nrofUplinkSymbols |  | 4 | 2 | 6 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 | Not configured | 1 |
| nrofDownlinkSlots |  | 2 | Not configured | 2 |
| nrofDownlinkSymbols |  | 0 | Not configured | 0 |
| nrofUplinkSlot |  | 0 | Not configured | 0 |
| nrofUplinkSymbols |  | 0 | Not configured | 0 |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |

Table A.3.1.4-3: TDD UL/DL configuration for SCS=120 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 | TDDConf.3.2 |  |
| referenceSubcarrierSpacing | kHz | 120 | 120 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=’10DL:2GP:2UL’ | ‘DDDSU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 | 0.625 |  |
| nrofDownlinkSlots |  | 3 | 3 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 1 | 1 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured | Not configured |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured | Not configured |  |
| nrofDownlinkSlots |  | Not configured | Not configured |  |
| nrofDownlinkSymbols |  | Not configured | Not configured |  |
| nrofUplinkSlot |  | Not configured | Not configured |  |
| nrofUplinkSymbols |  | Not configured | Not configured |  |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |

Table A.3.1.4-4: TDD UL/DL configuration of SBFD for SCS=30 kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TDDConf.2.4 |
| referenceSubcarrierSpacing | kHz | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S1U’ S=’10DL:2GP:2UL’ |
| dl-UL-TransmissionPeriodicity | ms | 2.5 |
| nrofDownlinkSlots |  | 3 |
| nrofDownlinkSymbols |  | 10 |
| nrofUplinkSlot |  | 1 |
| nrofUplinkSymbols |  | 2 |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |
| dl-UL-TransmissionPeriodicity | ms | Not configured |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |

Table A.3.1.4-5: TDD UL/DL configuration of SBFD for SCS=120 kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.3 |  |  |
| referenceSubcarrierSpacing | kHz | 120 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S1U’ S=’10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 1 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |  |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured |  |  |
| NOTE 1: As specified in TS 38.213 [3] and TS 38.331 [2]. NOTE 2: For information |  |  |  |  |

<End of Change #1>

<Start of Change #2>

A.3.10	SSB Configurations

A.3.10.1	SSB Configurations for FR1

#### A.3.10.1.14	SSB pattern 14 in FR1: SSB allocation for SSB SCS=30 kHz in 100 MHz

Table A.3.10.1.14-1: SSB.4 FR1: SSB Pattern 14 for SSB SCS=30 kHz in 100 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 | 8-11 |
| Slot numbers containing SSB Note 3 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |  |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |  |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

<End of Change #2>

<Start of Change #3>

A.3.10.2	SSB Configurations for FR2

A.3.10.2.28	SSB pattern 28 in FR2: SSB allocation for SSB SCS=120 kHz in 400 MHz

Table A.3.10.2.28-1: SSB.1 FR2: SSB Pattern 28 for SSB SCS = 120 kHz in 400 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 400 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |  |
| PRB numbers containing SSBs within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |  |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

<End of Change #3>

<Start of Change #4>

A.3.X	SBFD configurations

A.3.X.1	SBFD configurations for FR1

This clause provides typical SBFD configurations used for SBFD RRM test cases defined in annex A. Both the time domain and frequency domain configurations for SBFD are provided for FR1.

The applicable TDD DL/UL configurations for SBFD RRM test cases are defined in Clause A.3.1.4. Based on the applicable TDD DL/UL configurations for SBFD, the time domain configuration of SBFD slots/symbols is set as ‘DXXXU’, where ‘X’ denotes the SBFD slot with all ‘D’ and ‘S’ symbols in the slots configured as SBFD symbol.

For the frequency domain configuration of SBFD subbands, DU and DUD cases are allowed. For the case of DU, 80MHz is in D subband, and 20MHz is in U subband. For the case of DUD, 40MHz is in the 1st D subband, 20MHz is in U subband, and 40MHz is in the 2nd D subband.

A.3.X.2	SBFD configurations for FR2

This clause provides typical SBFD configurations used for SBFD RRM test cases defined in annex A. Both the time domain and frequency domain configurations for SBFD are provided for FR2.

The applicable TDD DL/UL configurations for SBFD RRM test cases are defined in Clause A.3.1.4. Based on the applicable TDD DL/UL configurations for SBFD, the time domain configuration of SBFD slots/symbols is set as ‘DXXXU’, where ‘X’ denotes the SBFD slot with all ‘D’ and ‘S’ symbols in the slots configured as SBFD symbol.

Note: For the frequency domain configuration of SBFD subbands, wait for further consensus.

<End of Change #4>