# 3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2521995

# Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			On A-IoT backscatter powr test parameter

Agenda item:		4.20.1.1

Document for:		Approval

# Introduction

In this paper, we present our over views on Rel-19 A-IoT device test parameter for backscatter power.

# Discussion

In WF [1], the test parameter setting relating to filter is to be discussed and below we share our view on it.

In our companion paper[2], we have presented the impact of the ISI on the decoding performance and the modification of the transmission bandwidth accordingly to reduce the SNR degradation.

From testing perspective, the ISI will add additional leakage power to the sampled point in time domain and due to this, it is preferred that the test measure the “clean” backscatter signal without ISI. In this sense, we suggest using the 1/Tc as the filter bandwidth in the positive frequency, this is illustrated in Figure 1 as below.

Figure 1: The baseband filter bandwidth (positive frequency) set as the 1/Tc

The RF frontend may down convert 2SB RF signal to baseband where the baseband 2SB signal is illustrated in Figure 1 and the baseband signal is illustrated with Btx.D2R in Figure 6.1.2.4-1 [TR 38.769].

Figure 6.1.2.4-1: Btx,D2R and Bocc,D2R for 2SB transmission of Manchester line code with option 1, and if no D2R line code is used (i.e., by using a square-wave corresponding to the small frequency-shift)

Figure 2: RF down conversion of backscatter signal for R=1.

Figure 3: RF down converted BB signal for R>1

Assuming the test equipment equips a coherence receiver with a homodyne RF architecture. With the LO frequency set as the same CW frequency, the 2 SB in positive frequency domain can be down converted to baseband. After applying a LPF with filter bandwidth of 2SB, the signal can be sampled in time domain. The LPF bandwidth is 1SB in positive frequency or 2SB in total including both positive and negative frequency. For the R>1 case, the main lobe should be defined as the bandwidth of the 1/Tc (in positive frequency) to avoid the ISI leakage power. This is illustrated in Figure 3.

How the test equipment makes the down conversion and which RF architecture it implements may be up to discussion. If the filter bandwidth is set in RF signal, such discussion can be avoided. With the above analysis, the filter bandwidth can be set in relation to the RF signal in stead and total filter bandwidth is set in relation to the CW signal frequency. This is illustrated in Figure 4. As the filter bandwidth cross the CW frequency, the CW signal should be excluded from the power detection.

Apply a filter bandwidth of 1/Tc in positive frequency or 2/Tc including both positive and negative frequency.

Apply the test parameter in RF domain so to avoid the test equipment implementation discussion.

Figure 4: Filter bandwidth setting in relation to Fcw (R=4)

The proposed text and table is as below:

The test configuration in Table A.1-1 is defined to be used for testing the backscattering loss requirement defined in section 6.1.1, the filter position is illustrated in Figure A.1-1 and Figure A.1-2.

Table A.1-1: Test configuraton

| Test parameter | value |
| --- | --- |
| filter bandwidth (kHz) | 2/Tc* 1.1 |
| CW frequency (MHz) | According to TS 38.194 |
| Filter center frequency | Fcw |

Figure A.1-1: illustration of measurement filter configuration (R>1)

Figure A.1-2: illustration of measurement filter configuration (R=1)

# Conclusions

In this contribution, we present our view on D2R channel bandwidth for R>1 case with below observation and proposals:

Proposal-1:Apply a filter bandwidth of 1/Tc in positive frequency or 2/Tc including both positive and negative frequency.

Proposal-2:Apply the test parameter in RF domain so to avoid the test equipment implementation discussion.

# References

- R4-2515161	Way Forward for [116bis][326] A-IoT_Maintenance, CMCC

- R4-25xyz, On A-IoT BS D2R bandwidth, Ericsson