# 3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2521996

# Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			On A-IoT BS D2R transmission bandwidth

Agenda item:		4.20.1.2

Document for:		Approval

# Introduction

In this paper, we present our over views on Rel-19 A-IoT D2R transmission bandwidth.

# Discussion

In Annex A in 38.194, the BS D2R BW is calculated :

D2R CBW for BS (kHz)

= ceiling ((2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9)

The D2R CBW defined above assumes the BS will set the receiver channel filter with a bandwidth of 2SB transmission BW_without SFO so the main lobe in frequency domain for a specific R>1 received after the down conversion of RF signal to baseband. This is the agreement in WF[3]

- Note : BW and  Small frequency shift  are in kHz;Tc and Tb are in μs

| 2SB Transmission BW (kHz) |  | Small frequency shift (kHz) |  |
| --- | --- | --- | --- |
| Without SFO | With SFO | Without SFO | With SFO |
| Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc) | Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣) | SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc) | SFSwith SFO =SFSwithout SFO×(1±∣SFO∣) |

And conceptually it is illustrated in Figure 1-2 [2].

Figure 1-2 [2]. SFO impact of transmission bandwidth and small frequency shift of FDMA

The PSD of the OOK waveform and BPSK waveform according to TS 38.291 line coding is illustrated in Figure 1 and Figure 2 respectively. It can be observed that PSD of OOK and BPSK are similar. In Figure 1, the transmission bandwidth of waveform of R=1 and waveform of R=4 is the same and for R>1, the transmission bandwidth is centred at R/Tb.

PSD of the Rel-19 A-IoT BPSK and OOK is similar

For waveform with R>1, the transmission bandwidth centred at R/Tb is the same as transmission bandwidth of R=1.

Figure 1: Rel-19 A-IoT D2R Waveform PSD (OOK) for R=1 and R=4

Figure 2:Rel-19 A-IoT D2R Waveform PSD (BPSK) for R=1 and R=4

In WF[2], the LLS for SNR of the REFSENS is simulated using R=1 and it is our understanding that same SNR is used for other R.

The SNR of D2R for BS REFSENS is derived assuming R=1.

The same SNR is assumed for R>1 in current A-IoT BS specification lacking the technical evidence.

The raw BER (Chip) vs Eb/N0 is simulated in Figure 3 with different filter bandwidth for the case of R=4 and Tb= 133.33 us. The coherent receiver and AWGN channel is used in simulation. The result is compared to the theorical BER (BPSK) using the complementary error function 𝑒𝑟𝑓𝑐:

BER=12∙Erfc(EbN0)

Figure 3: BER vs Eb/N0 for different filter bandwidth for case of R=4

It can be observed that receiver Eb/No improves around 3 dB (BER=0.01) when the filter bandwidth is increased from 15kHz to 30kHz (2 times of 15kH). This is due to the ISI from adjacent symbol reduced when filter bandwidth increases. This can be observed from the eye diagram in Figure 4. The left eye opens wider than the right eye and explains the Eb/N0 improvement.

Increasing the receiver filter bandwidth with scaling factor >1 reduce the ISI and improve the Eb/N0.

Figure 4: eye diagram for filter bandwidth of scaling 200% (left) and no filter bandwidth scaling (right)

For R= 16, it does not work if the receiver bandwidth is set as 100% of the transmission bandwidth, it will work if at least the receiver filter bandwidth is set 300% transmission bandwidth (15kHz). This is illustrated in Figure 5.

Figure 5:BER vs Eb/N0 for different filter bandwidth for case of R=16

As the Tc equals Tb/(2R), when R increase, the baseband of chip is 1/Tc and set the receiver filter bandwidth of 2/Tb which is portion of the 1/Tc introduce the ISI as explained above. When receiver filter bandwidth is set with 1/Tc, the eye diagram is open and in line with the BER vs Eb/N0 curve in Figure 6.

Figure 6:BER vs Eb/N0 for different filter bandwidth =1/Tc (16 * 15kHz)

Figure 7:: eye diagram for filter bandwidth of scaling 1600 (16* 15kHz)

From the above analysis, increasing the receiver filter bandwidth improves SNR gain but apparently this needs more spectrum allocation. The above observation applies to R>1 case.  Therefore, we suggest defining a D2R bandwidth scaling factor which can be declared by manufacture. When operator want to increase the D2R coverage which is bottleneck for A-IoT system, wider D2R bandwidth can be allocated to increase the SNR for R>1 case.

BS Manufacture declare the scaling factor for the D2R bandwidth to improve the D2R coverage.

The proposed change is below:

D2R CBW for BS (kHz)

= ceiling ((2SB Transmission BW_without SFO× (1/2) × F +2× Small frequency shift_without SFO)/0.9)

=ceiling ( (2 F +2R)/Tb × (1+∣SFO∣)/0.9)

=ceiling ( (F +R)/ (Tc ×R) × (1+∣SFO∣)/0.9)

Note: F is the scaling factor for R>1 case declared by manufacture.

# Conclusions

In this contribution, we present our view on D2R channel bandwidth for R>1 case with below observation and proposals:

Observation 1 PSD of the Rel-19 A-IoT BPSK and OOK is similar

Observation 2 For waveform with R>1, the transmission bandwidth centred at R/Tb is the same as transmission bandwidth of R=1.

Observation 3 The SNR of D2R for BS REFSENS is derived assuming R=1.

Observation 4 The same SNR is assumed for R>1 in current A-IoT BS specification lacking the technical evidence.

Observation 5 Increasing the receiver filter bandwidth with scaling factor >1 reduce the ISI and improve the Eb/N0.

Proposal-1: BS Manufacture declare the scaling factor for the D2R bandwidth to improve the D2R coverage.

# References

- TS 38.194

- R4-2505097, WF on requirements for A-IoT BS and CW, Huawei

- R4-2508116, WF on A-IOT device requirements, CMCC