3GPP TSG-RAN WG4 Meeting # 117	 R4-2521998

# Dallas, US,  November 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.194 | CR | <CR#> | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.194: Maintenance CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Maintenance CR, correct the D2R bandwidth definition and the Tx modulation signal quality |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding manufacture declaration factor on the D2R bandwidth definition; Correcting some definition of the test metric definition for Tx modulartion siganl quality |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The D2R bandwidth definition is not correct for R> 1 case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* start change */

#### 5.3.2.3	D2R channel bandwidth

The BS D2R channel bandwidth is specified in Table 5.3.2.3-1.

Table 5.3.2.3-1: BS D2R channel bandwidth

|  | BS D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal D2R transmission Bandwidth without SFO for R>1 (kHz) | Nominal D2R transmission Bandwidth without SFO for R=1 (kHz) | Nominal Small frequency shift without SFO(kHz) |  |  |  |  |  |  |  |  |
|  |  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15*X | 15 | 19 | 28 | 46 | 83 | 156 | 303 | 596 | 1183 |  |
| 30*X | 30 |  | 37 | 55 | 92 | 165 | 312 | 605 | 1192 |  |
| 60*X | 60 |  |  | 74 | 110 | 184 | 330 | 624 | 1210 |  |
| 120*X | 120 |  |  |  | 147 | 220 | 367 | 660 | 1247 |  |
| 240*X | 240 |  |  |  |  | 294 | 440 | 734 | 1320 |  |
| 480*X | 480 |  |  |  |  |  | 587 | 880 | 1467 |  |
| 960*X | 960 |  |  |  |  |  |  | 1174 | 1760 |  |
| 2880 |  |  |  |  |  |  |  |  |  | 3520 |
| Note 1 : scaling factor X is declared by manufacturer. |  |  |  |  |  |  |  |  |  |  |

/* Next change */

### 6.4.2	Modulation quality

Based on TS38.291[7], R2D signal includes SIP (Start indicator part), CAP (Clock acquisition part), PRDCH, the R2D postamble and padding if needed.

An is measured high level for the nth chip, in units of V/m or A/m

Bn is measured low level for the mth chip, in units of V/m or A/m

Anavg is the measured RMS average high level for the nth chip during1/2 chip duration, in units of V/m or A/m

Bnavg is the measured RMS average low level for the mth chip during 1/2 chip duration, in units of V/m or A/m

Modulation depth:

Modulation depth is defined with equation below and modulation depth for chip 0/1 shall meet the requirements in Table 6.4.2-1.

Modulation depth =(Anavg-Bnavg)/Anavg

The envelope of electric filed strength for chip 0 RF pulse shall comply the timing mask in Figure 6.4.2-1 and meet the requirements in Table 6.4.2-1. The envelope of electric filed strength for chip 0 shall decrease monotonically from 90% to less than 10 % of initial value Einitial during t1. The envelope of electric filed strength for OOK Bit 0 shall increase monotonically from 10% to less than 90 % of its initial value Einitial during t3.

The initial value Einitial is defined as the field strength difference between Anavg and Bnavg.

RF Envelop Rise Time:

The Tr,10-90 measures the rise of the chip 0 pulse and starts when envelop rises to 10% level of the initial value Einitial and ends when the envelop rises to 90% of the initial value Einitial.

RF Envelop Fall Time:

The Tf,10-90 measures the fall time of the chip 0 pulse and starts when envelop falls to 90% level of the initial value Einitial and ends when the envelop falls to 10% of the initial value Einitial.

Tf,10-90 starts when the envelop drops to the 90% level of the initial value Einitial and ends when envelop rise to 10% level of the initial value Einitial.

Ripple:

Ripple_high (%) = ((An − Anavg) / (Anag-Bnavg)) × 100%

Ripple_low (%) = ((Bn − Bnavg) / (Anavg-Bnavg)) × 100%

In case of an overshoot or undershoot the field shall remain within +/- Ripple_high % of Einitial for OOK chip 1 and +/-Ripple_low % of Einitial for chip 0.

Pulsewidth

The PW measures the time between envelop falling edge at 50% of the initial value Einitial and envelop rinsing edge at 50% of the initial value Einitial.

/* Next change */

Annex A (informative):
D2R channel bandwidth

The following describes the equation to derive BS D2R channel bandwidth.

For BS D2R CBW:

D2R CBW for BS (kHz)

= ceiling ((2SB Transmission BW_without SFO× (1/2) × F +2× Small frequency shift_without SFO)/0.9)

=ceiling ( (2F+2R)/Tb × (1+∣SFO∣)/0.9)

=ceiling ( (F+R)/ (Tc ×R) × (1+∣SFO∣)/0.9)

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band). Tc= Tb/(2R)

Note: F is the scaling factor declared by manufacture.

/* end change */